### CENWP-OP-B                                                                                                                           14 January, 2002

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 02 of 2002, which covers the period 06 through 12 January 2002.

- OPERATION SUMMARY.

  - Daily river flow averaged 122.0 to 171.6kcfs.  Daily project tailwater elevation averaged 12.1’ to 16.6’ msl.  Daily project forebay elevation averaged 74.7’ to 76.0’ msl.  River temperature averaged 45 to 46° F.  Daily turbidity averaged 2.8 to 7’ on the secchi disk.

  - Spill Bays 18 and 1 are closed for spill deflector work.

  - Units 3 and 5 remain out of service for turbine rehab.  Unit 17 remains out of service for VBS slot modification work.  Unit 18 went out of service from 1036 until 1206 on 08 January, for cleaning of the cooling water strainer.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly, if drawdown exceeds 1.0’.  F1 remains out of service for winter maintenance.  As coordinated with NMFS, F2 is supplying all attraction flow at this time. (see Misc. 5.4)

- ACTIVITIES.

  - AWS Closures. Bradford Island AWS valves closed for winter maintenance.

09 January- FV5-9 closed for cleaning from 1513 until 1530

10 January- FV5-9 and FV6-9 closed for cleaning from 1530 until 1610.

  - STS/VBS Inspections.  All STSs have been removed for winter maintenance.

  - Dewatering and Fish Salvage.  No fish salvage or dewatering activities this reporting week.

- RESEARCH.

  - WDFW Fish Counting.  24-hour video count ongoing at the WA Shore ladder.

  - PSMFC Adult PIT.  Installation underway at Bradford Island ladder.

  - WDFW Winter Steelhead tagging.  Trapping continues three or four days a week.

  - NMFS Lamprey.  Tracking continues.  Plans for a trap in FV3-9 AWS channel underway.  More lamprey plates to be installed in the Washington Shore ladder in February.

4.  FISHWAYS.

  - Project biologists inspected 07 through 10 January this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | PH2 electronic velocity meter. | Meter not yet working | Need new cable. | Will be installed in February. |  |
| 1/07-09 | Weir 37 | Reading >1.0’ | Faulty sonic transducer. | See misc. 5.2. |  |
| 1/07 | DSM2 channel | Channel elevation <64.2’ | Unit 15 closed in manual |  |  |
| 1/07-09 | Weir 38, Weir 67 | Reading > or <1.0’ | FV6-9 not controlling properly. | Electricians repaired the problem. See misc. 5.7 |  |
| 1-10 | Gatewell 16C | Closed | Oil sheen in gatewell. | See misc. 5.6 |  |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are not recorded during winter maintenance season.

  - Sensor Calibration.  Calibration was not checked as there were no sensors in service this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - The B-Branch and Cascade Island computers were installed.  Until sensors have been calibrated, entrance gates, diffusers, valves FV4-3, FV4-4, FV5-3, FV5-4 are operated in manual.

  - Weir 37 float control doesn’t adequately control water level over the weir.  A sonic transducer will be installed this winter, which should correct the problem.

  - To aid spawning Chum below Bonneville, a tailwater restriction of 11.4’ to 11.6’ went into effect at 1054 on 28 November.  The restriction will continue until further notice.

  - As coordinated with NMFS, the ITC will not be used for AWS water until after the January FPOM meeting, where the issue of debris and fish collection/delay will be discussed.  Until those issues are resolved, F2 is being operated to maintain a 1.5’ differential.  In an effort to reduce the debris load during daylight hours, the fish unit is put on standby from 1800 until 0500 to float trash.

  - PH2 ITC was opened from 0749 until 0855 on 09 January to remove debris.

  - An oil sheen was reported in gatewell 16C.  The source is believed to be from the contractors spraying off vehicles on the +90 deck.  The orifice was closed from 1140 on 10 January until 1011 on 11 January.  Absorbent pads were used to soak up the oil.  When no sign of a sheen was present, the orifice was opened and placed in auto.

  - Electricians had FV6-9 in manual from 1600 on 09 January until 1032 on 10 January, while repairing the valve.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//