### CENWP-OP-B                                                                                                                           12 February, 2002

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 06 of 2002, which covers the period 03 through 09 February 2002.

- OPERATION SUMMARY.

  - Daily river flow averaged 128.7 to 172.8kcfs.  Daily project tailwater elevation averaged 12.7’ to 15.9’ msl.  Daily project forebay elevation averaged 72.9’ to 74.3’ msl.  River temperature averaged 44° F.  Daily turbidity averaged 6.8 to 7’ on the secchi disk.

  - Spill Bays 18 and 1 are closed for spill deflector work.

  - Unit 5 remains out of service for turbine rehab.  Unit 17 remains out of service for VBS slot modification work.  Unit 3 remains out of service for governor rehab.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly, if drawdown exceeds 1.0’.  F1 remains out of service for winter maintenance.  As coordinated with NMFS, F2 is supplying all attraction flow at this time. (see Misc. 5.4)

  - Gatewell Drawdown was measured on 06 February.  All units were within criteria.

- ACTIVITIES.

  - AWS Closures.

All Cascade Island AWS valves are closed for winter maintenance.

06 February- FV3-7 and FV3-9 were closed for cleaning from 0835 until 1615.

06 February- Bradford Island exit was closed for cleaning from 1625 until 1630.

09 February- FV6-9 closed, Washington Shore fishway to orifice flow.

  - STS/VBS Inspections.  All STSs have been removed for winter maintenance.

  - Dewatering and Fish Salvage.

On 04 February, Project Fisheries and PSMFC removed 41 adult Steelhead from the UMT channel and 4 adult steelhead, 10 juvenile salmonids, 55 resident chubs, peamouth, and whitefish, and 8 adult lamprey from the Cascade Island Ladder.

On 05 February, Project Fisheries removed 7 adult steelhead, 5 juvenile salmonids, 20 resident chubs, peamouth and whitefish, and 1 adult lamprey from the Cascade Island ladder diffuser pools.

All fish were in good condition and released upstream of the new navlock.

On 07 February, Project Fisheries, WDFW and PSMFC dewatered the collection pool of the AFF.  Fish were guided down to the lab entrance ladder bulkhead.  A weir block was installed at the lowest weir and water levels maintained to encourage fish to leave voluntarily.  The exit ladder was left in operation to allow fish to leave voluntarily before the Washington Shore dewatering on 11 February.

- RESEARCH.

  - WDFW Fish Counting.  24-hour video count ongoing at the WA Shore and Bradford Island ladders.

  - PSMFC Adult PIT.  Installation underway at Cascade Island ladder.  Non in-water work ongoing at Bradford Island.

  - WDFW Winter Steelhead tagging.  Trapping came to an end on 05 February to prepare for the AFF dewatering.

  - NMFS Lamprey.  Tracking continues. More lamprey plates to be installed in the Washington Shore ladder in February.

4.  FISHWAYS.

  - Project biologists inspected 05 and 06 February this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | PH2 electronic velocity meter. | Meter not yet working | Need new cable. | Will be installed in February. |  |
| All week | PH1 PCC velocity | Velocity reading <1.5fps on the electronic meter. | Unknown |  |  |
| All week | Daily inspections | Biologists performed <3 inspections this week. | Three days of dewatering fish facilities. | No action taken. |  |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperature probes are out of service for calibration.

  - Sensor Calibration.  Calibration was not checked as there were no sensors in service this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - The B-Branch and Cascade Island computers were installed, though we still wait on brackets for the transducers.  Until brackets are installed, FV4-3, FV4-4, FV5-3, FV5-4 are operated in manual.

  - Weir 37 float control doesn’t adequately control water level over the weir.  A sonic transducer will be installed this winter, which should correct the problem.

  - To aid spawning Chum below Bonneville, a minimum tailwater restriction of 11.4’ went into effect at 1054 on 28 November.  The restriction will continue until further notice.

  - As coordinated with NMFS, the ITC will not be used for AWS water.  F2 is being operated to maintain a 1.5’ differential.  In an effort to reduce the debris load during daylight hours, the fish unit is put on standby from 1800 until 0500 to float trash.   Plant material is a major debris problem.

  - PH1 ITS was closed from 0835 until 1615 on 06 February and from 0635 until 0850 on 07 February for BRZ work and log boom repair at the Bradford Island fish exit.

  - At 1130 on 04 February, Cascade Island fishway went out of service for winter maintenance.

  - At 0454 on 09 February, Powerhouse 1 became the priority powerhouse.

  - At 0816 on 09 February, Washington Shore went to orifice flow.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//