### CENWP-OP-B                                                                                                                           19 February, 2002

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 07 of 2002, which covers the period 10 through 16 February 2002.

- OPERATION SUMMARY.

  - Daily river flow averaged 125.2 to 176.8kcfs.  Daily project tailwater elevation averaged 12.5’ to 15.6’ msl.  Daily project forebay elevation averaged 73.3’ to 74.7’ msl.  River temperature averaged 44° F.  Daily turbidity averaged 4.0 to 7’ on the secchi disk.

  - Spill Bays 18 and 1 are closed for spill deflector work.

  - Unit 3 remains out of service for governor rehab.  Unit 5 remains out of service for turbine rehab.  Units 6, 7, and 8 were out of service from 0754 until 0806 on 14 February due to work on GT5.   Unit 17 remains out of service for VBS slot modification work.

  - Second Powerhouse fish unit 2 went out of service at 0300 on 11 February.   F1 remains out of service for winter maintenance.

  - Gatewell Drawdown was measured on 13 February.  All units were within criteria.

- ACTIVITIES.

  - AWS Closures.

All Cascade Island and Washington Shore AWS valves are closed for winter maintenance.

11 February- FV3-9 was closed for cleaning from 1605 until 1620.

13 February- FV3-9 was closed for cleaning from 1315 until 1330.

  - STS/VBS Inspections.  All STSs have been removed for winter maintenance.

  - Dewatering and Fish Salvage.

11 February Washington Shore ladder- Project Fisheries, Univ. Of Idaho, and WDFW removed 40 adult steelhead, 8 steelhead smolts, 14 adult lamprey and 6 sculpin.

11 February UMT- Project Fisheries and Univ. of Idaho removed 4 adult steelhead, 3 chubs, and 2 steelhead smolts.

11 February AFF- Project Fisheries, Univ. of Idaho, and WDFW removed 68 adult steelhead, 12 steelhead smolts, 6 chubs, 1 sculpin, and 10 sticklebacks from the exit ladder of the lab.  From the entrance ladder 4 live steelhead and 10 (8 clipped and 2 unclipped) dead steelhead were removed.

- RESEARCH.

  - WDFW Fish Counting.  24-hour video count ongoing at the Bradford Island ladder.

  - PSMFC Adult PIT.  Installation underway at Cascade Island ladder.  Non in-water work ongoing at Bradford Island.

  - WDFW Winter Steelhead tagging.  Done until Washington Shore comes back in service.

  - NMFS Lamprey.  Tracking continues. More lamprey plates to be installed in the Washington Shore ladder.

4.  FISHWAYS.

  - Project biologists inspected 11, 13, 14, and 15 February this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 PCC velocity | Velocity <1.5fps on the electronic meter. | Unknown |  |  |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperature will be recorded beginning 01 March.

  - Sensor Calibration.  Calibration was checked on 14 February.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - The B-Branch and Cascade Island computers were installed, though we still wait on brackets for the transducers.  Until brackets are installed, FV4-3, FV4-4, FV5-3, FV5-4 are operated in manual.

  - To aid spawning Chum below Bonneville, a minimum tailwater restriction of 11.4’ went into effect at 1054 on 28 November.  The restriction will continue until further notice.

  - On 13 February a tailwater restriction of 11.4’ to 12’ went into place from 0800 until 1200.  This restriction was to aid Battelle in piezometer maintenance work downstream of the project.

  - BPA requested a 1% violation from 0700 until 1100 on 14 February.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//