### CENWP-OP-B                                                                                                                                  19 March, 2002

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 11 of 2002, which covers the period 10 through 16 March 2002.

- OPERATION SUMMARY.

  - Daily river flow averaged 110.4 to 191.7kcfs.  Daily project tailwater elevation averaged 11.5’ to 16.6’ msl.  Daily project forebay elevation averaged 73.8’ to 76.7’ msl.  River temperature averaged 45 to 46° F.  Daily turbidity averaged 5.0 to  >7’ on the secchi disk.

  - Spill Bays 18 and 1 remain closed for hoist work.  They are scheduled to be back in service by 25 March.  Spill for Spring Creek fish began at 0900 on 12 March and concluded at 0600 on 15 March.  Daily spill averaged 0.0 to 87.2kcfs.

  - Unit 5 remains out of service for turbine rehab.  Unit 17 remains out of service for VBS slot modification work.   Unit 10 remains out of service for its five-year overhaul.

Units 7 and 8 were out of service from 0506 until 0512 and again from 1606 until 1618 on 11 March, from 0500 until 0506 on 13 March, and from 1706 until 1712 on 14 March for switching work.  Unit 6 was out of service from 0500 until 1618 on 11 March for Unit 5 CT replacement and from 0500 on 13 March until 1712 on 14 March for looptesting with Unit 5.  Unit 18 was out of service from 0700 until 0818 on 13 March for transducer replacement.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly, if drawdown exceeds 1.0’.  Fish Unit 1 returned to service at 1248 on 13 March.

  - Gatewell Drawdown was measured on 14 March.  All units were within criteria.

- ACTIVITIES.

  - AWS Closures.  FV3-9 and the Bradford Island exit were closed for cleaning from 1534 until 1546 on 14 March.

  - STS/VBS Inspections.  All STSs have been installed for fish passage season.

  - Dewatering and Fish Salvage.  No dewatering or fish salvage activities for this reporting week.

- RESEARCH.

  - WDFW Fish Counting.  24-hour video count ongoing at the Bradford Island and Washington Shore ladders.

  - PSMFC Adult PIT.  Waiting for the electronics office.

  - PSMFC/NMFS Juvenile PIT.  Monitoring continues.

  - WDFW Winter Steelhead tagging.  Trapping and tagging of steelhead continues.

  - NMFS Lamprey.  Tracking continues.

  - PSMFC JMF.  Smolt monitoring continues.

  - NMFS Saltwater rearing.  Set up underway, start up is expected for April.

4.  FISHWAYS.

  - Project biologists inspected 11-14 March this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| 3/12-14 | PH1 PCC velocity | Velocity <1.5fps on the electronic meter. | Unknown- the rest of the ladder is in criteria. | No action at this time. |  |
| All week | PH2 PCC velocity meter. | Electronic meter not yet installed. | Divers weren’t able to install parts during 19 February dive. | The transducer will be installed during 20 March dive. |  |
| 3/10, 15-16 |  | Biologists did not inspect 7 days per week. | Personnel shortage. | Job advertised. |  |
| 3/12-14 | Weir 37 | Reading > or < 1.0’ | A dampener needs to be installed in the stillwell. | Electricians will fix during 20 March dive. |  |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.   Temperatures are in Celsius.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 3/10 | 5.1 | 5.3 |
| 3/11 | 5.3 | 5.5 |
| 3/12 | 5.5 | 5.7 |
| 3/13 | 5.5 | 5.7 |
| 3/14 | 5.5 | 5.8 |
| 3/15 | 5.5 | 5.8 |
| 3/16 | 5.4 | 5.6 |

  - Sensor Calibration.  Calibration was checked on 14 March this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - To aid spawning Chum below Bonneville, a minimum tailwater restriction of 11.4’ went into effect at 1054 on 28 November.  The restriction will continue until further notice.

  - FV4-3, FV4-4, FV5-3, and FV5-4 sensor brackets are finished and waiting to be installed.  Until they are installed, the valves are operated in manual.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//