### CENWP-OP-B                                                                                                                                  01 April, 2002

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 13 of 2002, which covers the period 24 through 30 March 2002.

- OPERATION SUMMARY.

  - Daily river flow averaged 118.5 to 138.6kcfs.  Daily project tailwater elevation averaged 11.5’ to 13.2’ msl.  Daily project forebay elevation averaged 73.3’ to 76.1’ msl.  River temperature averaged 46 to 47° F.  Daily turbidity averaged 5.0 to 6.5’ on the secchi disk.

  - Spill Bays 1and 18 remain closed for hoist work.  They are scheduled to be back in service sometime the week of 7 April.  Spill averaged 0.0kcfs.

  - Unit 5 remains out of service for turbine rehab.  Unit 17 remains out of service for VBS slot modification work.  Unit 7 remains out of service for a governor rehab and exciter installation.  Unit 4 went out of service from 1324 until 1654 on 25 March for a megawatt meter installation.  Unit 10 went out of service from 1324 until 1506 on 25 March for repairs to the governor supply line.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly, if drawdown exceeds 1.0’.

  - Gatewell drawdown was measured at PH2 on 27 March and at PH1 on 28 March.  All units were within criteria.

- ACTIVITIES.

  - AWS Closures.  FV1-1 was closed for cleaning from 1340 until 1630 on 25 March.  FV3-9 was closed for cleaning from 1505 until 1543 on 29 March.

  - STS/VBS Inspections.  No STS inspections this reporting week.

  - Dewatering and Fish Salvage.  No dewatering or fish salvage activities for this reporting week.

- RESEARCH.

  - NMFS Lamprey.  Tracking continues.

  - NMFS Saltwater Rearing.  Set up underway, start up is expected for April.

  - NMFS VBS & Switchgate Fry Tests. Fry tests were conducted 27-29 March.  No fish were found in the bypass side of the switchgate.  Test fry were not impinged on the VBS.  For complete results, contact Bruce Monk.

  - Normandeau Balloon-tag at Spillway.  Scheduled for 12-31 May and 1-15 August.

  - OCFWRU Transportation Study.  Study of post-transportation migration and survival of juveniles to run 29 April through 14 July.

  - ODFW Pikeminnow Electroshocking.  Scheduled to begin week of 1 April.

  - Pelfrey Sturgeon Hatchery.  Scheduled to begin capturing broodstock in late May.

  - PSMFC Adult PIT.  Waiting for the electronics office.

  - PSMFC JMF.  Smolt monitoring continues.

  - PSMFC/NMFS Juvenile PIT.  Monitoring continues.

  - U of I Transportaion Study.  Evaluation of physiological condition of river and barged snake river juveniles to run May-June.  May add separation–by-code study June-August.

  - Uof I Adult Radiotracking.  Scheduled to begin week of 1 April.

  - USGS Juvenile Radiotracking.  Equipment installation continues.

  - USGS MGR.  Equipment installation continues.

  - WDFW Fish Counting.  24-hour video count ongoing at Bradford Island and Washington Shore ladders.

  - WDFW Winter Steelhead Tagging.  Trapping and tagging of steelhead continues.

  - WES/Battelle/Mevatech Juvenile Hydroacoustics.  Equipment installation and testing continues.

4.  FISHWAYS.

  - Project biologists inspected 24-29 March this reporting week.  NMFS inspected 25 March.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| 3/26 | PH1 PCC velocity | Velocity <1.5fps on the electronic meter. | Unknown- the rest of the ladder is in criteria. | No action at this time. |  |
| All week. | PH2 PCC velocity meter. | Reading <1.5fps on the electronic meter. | U-sonic failure |  |  |
| 3/30 |  | Biologists did not inspect 7 days/ week. | Personnel shortage. | Job advertised. |  |
| 3/25-26, 28-29 | Weir 37 | Reading > or < 1.0’ | A dampener needs to be installed in the stillwell. | Electricians will fix during 20 March dive. |  |
| 3/24, 3/29 | Weir 38 | Reading > or < 1.0’ | Unknown | Operators looking into it. |  |
| 3/24 | A and B branch | Reading <1.0’ | Unknown | No action taken. Will monitor situation. |  |
| 3/25 | DSM2 channel | Elevation <64.2’ | Low forebay | No action taken |  |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.   Temperatures are in Celsius.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 3/24 | 5.8 | 5.8 |
| 3/25 | 6.1 | 6.1 |
| 3/26 | 6.3 | 6.3 |
| 3/27 | 6.3 | 6.4 |
| 3/28 | 6.5 | 6.5 |
| 3/29 | 6.6 | 6.6 |
| 3/30 | 6.7 | 6.8 |

  - Sensor Calibration.  Calibration was checked on 25 March this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - To aid spawning Chum below Bonneville, a minimum tailwater restriction of 11.4’ went into effect at 1054 on 28 November.  The restriction will continue until further notice.

  - FV4-3, FV4-4, FV5-3, and FV5-4 sensor brackets are finished and waiting to be installed.  Until they are installed, the valves are operated in manual.

  - DSM1 inclined screen began to malfunction and was placed in manual at 1505 on 28 March.  Operators  maintained appropriate water levels over the weekend.

  - On Friday 29 March, Spring Creek Hatchery released 4.9 million fall Chinook infected with ICK.  At that time, Bonneville JMF was in bypass to facilitate re-orientation of the wedge wire screen.   As recommended by Larry Basham of the FPC, we will remain in bypass until the facility can be disinfected and the majority of the diseased fish are probably past Bonneville.

  - The PDS wedge-wire screen was re-oriented and began making noise.  Various actions were taken to try to silence the screen but none succeeded.  The original “parallel to flow” wedge-wire will be re-installed on 01 April.

  - Sea lions and harbor seals have been spotted in the tailrace of both powerhouses and the spillway.  No action is being taken by the project to harass, harm, or otherwise annoy these animals.  FFU is monitoring and recording their activity.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//