### CENWP-OP-B                                                                                                                                  07 April, 2002

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 14 of 2002, which covers the period 31 March through 06 April 2002.

- OPERATION SUMMARY.

  - Daily river flow averaged 126.1 to 175.7kcfs.  Daily project tailwater elevation averaged 12.0’ to 15.5’ msl.  Daily project forebay elevation averaged 75.6’ to 76.1’ msl.  River temperature averaged 47 to 51° F.  Daily turbidity averaged 4.0 to 6.0’ on the secchi disk.

  - Spill Bays 1and 18 remain closed for hoist work.  They are scheduled to be back in service sometime the week of 7 April.  Spill averaged 0.0kcfs.

  - Unit 5 remains out of service for turbine rehab.  Unit 17 remains out of service for VBS slot modification work.  Unit 7 remains out of service for a governor rehab and exciter installation.   Unit 3 was out of service at 0354 on 01 April until 1018 on 05 April for governor work.  Units 6 and 8 tripped out of service from 0900 until 0918 on 02 April.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly, if drawdown exceeds 1.0’.

  - Gatewell drawdown was measured at PH2 on 03 April.  All units were within criteria.

- ACTIVITIES.

  - AWS Closures.   The following valves were closed for trashrack raking:

01 April- FV3-9, FV3-7 from 0709 until 0811

03 April- FV4-3, FV4-4 from 1014 until 1551

04 April- FV5-3, FV5-4 from 1013 until 1116

FV1-1 from 1304 until 1325

  - STS/VBS Inspections.  No STS inspections this reporting week.

  - Dewatering and Fish Salvage.  No dewatering or fish salvage activities for this reporting week.

- RESEARCH.

  - CRITFC Age and Length Composition. Collecting data on Chinook and Coho 2 days/week from 1 April to 31 October.

  - NMFS Lamprey.  Tracking continues.   Collection begins in June.

  - NMFS Saltwater Rearing.  Recirculating tanks are up and running.  Fish tanks should be ready by 8 April.

  - Normandeau Balloon-tag at Spillway.  Scheduled for 12-31 May and 1-15 August.

  - OCFWRU Transportation Study.  Study of post-transportation migration and survival of juveniles to run 29 April through 14 July.

  - ODFW Summer Steelhead sampling.  Sampling 2 days/week 8 hours/day.  Will sample 1% run.

  - ODFW Pikeminnow Electroshocking.  Part of northern pikeminnow management program.  Capturing and collecting biological data from pikeminnow, smallmouth bass, and walleye in Bonneville’s BRZ.  Electroshocking  1-5 April and 22-26 April.

  - Pelfrey Sturgeon Hatchery.  Scheduled to begin capturing broodstock in late May.

  - PSMFC Adult PIT.  Waiting for the electronics office to be completed.

  - PSMFC JMF.  Smolt monitoring continues.

  - PSMFC/NMFS Juvenile PIT.  Monitoring continues.

  - U of I Transportation Study.  Evaluation of physiological condition of river and barged Snake River juveniles to run May-June.  May add separation–by-code study June-August.

  - U of I Adult Radio tracking.  Trapping began 1 April and will continue 7 days/week until October.

  - USGS Juvenile Radio tracking.  Equipment installation continues.

  - USGS MGR.  Equipment installation continues.

  - USGS EMG studies.  Tissue samples will be taken from 200 Chinook starting 1 April through June.

  - USGS Lamprey.  Collection begins in May.

  - WDFW Fish Counting.  16-hour visual and 8-hour video counts began on 1 April at Bradford Island and Washington Shore ladders.

  - WDFW Jaw tagging.  Tagging 50-75 fish per week, starting 1 April.

  - WDFW Winter Steelhead Tagging.  Trapping and tagging of steelhead continues.

  - WES Spillway Deflector Testing.  WES will be monitoring water quality equipment to determine effectiveness of new flip lips.  Study to begin when spill begins ~10 April.

  - WES/Battelle/Mevatech Juvenile Hydroacoustics.  Equipment installation and testing continues.

4.  FISHWAYS

  - Project biologists inspected 01-04 April this reporting week.  NMFS inspected 04 April.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| 4/2 | PH1 PCC velocity | Velocity <1.5fps on the electronic meter. | Unknown- the rest of the ladder is in criteria. | No action at this time. |  |
| 3/31, 4/5-6 |  | Biologists did not inspect 7 days/ week. | Personnel shortage. | Job advertised. |  |
| 4/2, 4/4 | Weir 37 | Reading > or < 1.0’ | Unknown |  |  |
| 4/1 | BI and CI | Diffusers not in correct position for TW elev. | Trying to automate the system, still working kinks out. | Diffusers are operated in manual |  |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.   Temperatures are in Celsius.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 3/31 | 6.9 | 7.0 |
| 4/1 | 7.0 | 7.0 |
| 4/2 | 6.9 | 7.0 |
| 4/3 | 7.0 | 7.0 |
| 4/4 | 7.2 | 7.2 |
| 4/5 | 7.2 | 7.2 |
| 4/6 | 7.4 | 7.4 |

  - Sensor Calibration.  Calibration was checked on 03 April this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - FV4-3, FV4-4, FV5-3, and FV5-4 are operated in manual until the computer system in up and running.  Calibration should be completed the week of 7 April.

  - DSM1 inclined screen began to malfunction and was placed in manual at 1505 on 28 March.  It was returned to auto at 1647 on 01 April.

  - The Juvenile Monitoring Facility went to bypass at 0700 on 1 April to facilitate the re-installation of the wedge wire screen.  The system remained in bypass for the emergency release of fish from Spring Creek Hatchery.  The fish were infected with Ichthyopthirius multifilis.  Sampling resumed at 0700 on 04 April, when it was assumed that  most of the Spring Creek fish had passed Bonneville.  PSMFC disinfected the holding tanks and sample troughs prior to returning to sample.

  - NMFS found one Kelt on the JMF separator bars during their 04 April inspection.

  - Sea lions and harbor seals have been observed in the tailrace of both powerhouses and the spillway.  No action is being taken by the project to harass, harm, or otherwise annoy these animals.  FFU is monitoring and recording their activity.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//