### CENWP-OP-B                                                                                                                                  15 April, 2002

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 15 of 2002, which covers the period 07 through 13 April 2002.

- OPERATION SUMMARY.

  - Daily river flow averaged 127.8 to 230.7kcfs.  Daily project tailwater elevation averaged 12.1’ to 18.5’ msl.  Daily project forebay elevation averaged 75.3’ to 76.2’ msl.  River temperature averaged 51 to 53° F.  Daily turbidity measured 3.5 to 5.0’ on the secchi disk.

  - Spill Bays 1and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, are operational and will be included in the new spill pattern.  Spill started on 10 April and averaged from 0.0 to 76.6kcfs for the week.

  - Unit 5 remains out of service for turbine rehab.  Unit 7 remains out of service for a governor rehab and exciter installation.  Unit 1 went out of service at 0600 on 08 April for digital governor installation.  Unit 15 was out of service from 0700 on 08 April until 0924 on 10 April for STS work.  Unit 17 returned to service at 1718 on 08 April.  Unit 11 was out of service from 1630 on 10 April until 1600 on 11 April due to STS malfunction.  Unit 9 tripped off line from 1113 until 1400 on 11 April.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

  - Gatewell drawdowns were measured on 10 April.  All units were within criteria.

- ACTIVITIES.

  - AWS Closures.  The Bradford Island exit was closed for cleaning from 1245 until 1257 on 10 April. 1551

  - STS/VBS Inspections.  No STS inspections this reporting week.

  - Dewatering and Fish Salvage.  Unit 17 tail logs were pulled on 08 April.  One crayfish, 10 sculpin and 1 catfish were recovered and released to the tailrace.

- RESEARCH.

  - CRITFC Age and Length Composition. Collected 40 Spring Chinook.  They will be collecting Chinook and Coho twice a week from 1 April to 31 October.

  - NMFS Lamprey.  Tracking continues.   Collection begins in June.

  - NMFS Saltwater Rearing.  Recirculating tanks are operational.

  - Normandeau Balloon-tag at Spillway.  Scheduled for 12-31 May and 1-15 August.

  - OCFWRU Transportation Study.  Study of post-transportation migration and survival of juveniles to run 29 April through 14 July.

  - ODFW Summer Steelhead sampling.  Sampling 2 days/week 8 hours/day.  Will sample 1% of the run.

  - ODFW Pikeminnow Electroshocking.  Part of northern pikeminnow management program.  Capturing and collecting biological data from pikeminnow, smallmouth bass, and walleye in Bonneville’s BRZ.  Electroshocking  1-5 April and 22-26 April.

  - Pelfrey Sturgeon Hatchery.  Scheduled to begin capturing broodstock in late May.

  - PSMFC Adult PIT.  Detectors are installed in all ladders.  Some of the detectors have developed leaks and will be monitored throughout the season.  Repairs are scheduled for 2002-03 winter maintenance season.

  - PSMFC JMF.  Smolt monitoring continues.  Gas bubble exams began 8 April.

  - PSMFC/NMFS Juvenile PIT.  Monitoring continues.

  - U of I Transportation Study.  Evaluation of physiological condition of river run v. barged Snake River juveniles to occur May-June.  May add separation–by-code study June-August.

  - U of I Adult Radio tracking.  Trapping began 1 April and will continue 7 days/week until October.

  - USGS Juvenile Radio tracking.  Equipment installation continues.

  - USGS MGR.  Equipment installation continues.

  - USGS EMG studies.  Tissue samples will be taken from 200 Chinook starting 1 April through June.

  - USGS Lamprey.  Collection begins in May.

  - WDFW Fish Counting.  16-hour visual and 8-hour video counts continue at Bradford Island and Washington Shore ladders.

  - WDFW Jaw tagging.  Tagging 50-75 fish per week, starting 1 April.

  - WDFW Winter Steelhead Tagging.  Trapping and tagging of steelhead continues.

  - WES Spillway Deflector Testing.  WES will be monitoring water quality equipment to determine effectiveness of new flip lips.  Study began with initiation of spill on 10 April.

  - WES/Battelle/Mevatech Juvenile Hydroacoustics.  Equipment installation and testing continues.

4.  FISHWAYS

  - Project biologists inspected 07-12 April this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 PCC velocity | Velocity <1.5fps on the electronic meter. | Unknown- the rest of the ladder is in criteria. | No action at this time. |  |
| One day |  | Biologists did not inspect 7 days/ week. | Personnel shortage. | Job advertised. |  |
| 4/09 | Weir 37 | Reading > or < 1.0’ | Stillwell is needed. | Added to winter maint. list. |  |
| 4/10, 4/12 | PH2 PCC velocity | Velocity <1.5fps on the electronic meter. | Unknown- the rest of the ladder is in criteria. | No action at this time. |  |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.   The temperature is recorded every 15 seconds, averaged for each hour then averaged for the entire day.  Temperatures are in Fahrenheit.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 4/7 | 45.7 | 45.6 |
| 4/8 | 46.2 | 46.0 |
| 4/9 | 46.8 | 46.8 |
| 4/10 | 47.1 | 47.0 |
| 4/11 | 47.3 | 47.3 |
| 4/12 | 47.8 | 47.6 |
| 4/13 | 48.6 | 48.5 |

  - Sensor Calibration.  Calibration was checked on 08 April this reporting week.  Everything is within criteria.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - FV4-3, FV4-4, FV5-3, and FV5-4 are operated in manual until the computer system in up and running.  Calibration should be completed sometime in April.

  - River temperature is the temperature of the water in the scroll case of Unit 2.  This temperature is recorded every 6 hours (the FPP requires a reading three times a day).

  - Supplying power to the JMF gate keycard detector required opening the spare breaker located in the breaker box at headwall #1.  Due to a mislabeling of the breakers, the upstream switchgate breaker was closed during the process and the switchgate moved to bypass at around 0930.  It was returned to sample by 1030.  The keycard detector is now on its own dedicated breaker.  A warning label informing people to contact fisheries before cutting power to the switchgate has been affixed to the breaker box.

  - Avian wires were installed downstream of the spillway this week.

  - Sea lions and harbor seals have been observed in the tailrace of both powerhouses and the spillway.  No action is being taken by the project to harass, harm, or otherwise annoy these animals.  FFU is monitoring and recording their activity.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//