### CENWP-OP-B                                                                                                                                  16 June, 2002

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 24 of 2002, which covers the period 09 through 15 June 2002.

- OPERATION SUMMARY.

  - Daily river flow averaged 285.2 to 321.5kcfs.  Daily project tailwater elevation averaged 22.0’ to 24.4’ msl.  Daily project forebay elevation averaged 75.3’ to 75.6’ msl.  River temperature averaged 56 to 57° F.  Daily turbidity measured 3.5’ to 5.0’ on the secchi disk.

  - Spill Bays 1and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, are operational and included in the spill pattern.  Spill averaged 96.6 to 153.7kcfs for the week.

  - Unit Operation

    - Unit 1 went out of service at 0730 on 10 June for turbine rehab.

    - Unit 5 remains out of service for turbine rehab.

    - Unit 6 was out of service from 0806 until 0948 on 12 June for instrument removal.

    - Unit 9 remains out of service for digital governor work and a five year overhaul.

    - Unit 10 was out of service from 1012 until 1648 on 11 June for a breaker test.

    - Unit 12 was out of service from 1612 until 1724 for transducer replacement.

    - Unit 15 remains out of service until the VBSs are repaired.

    - Unit 17 was out of service for VBS cleaning from 1154 until 1612 on 10 June and for FGE testing from about 1600 until 2000 11-15 June.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

  - Gatewell drawdowns were measured every day for Unit 17 and on 12 June for the rest of the units.

- ACTIVITIES.

  - AWS Closures.  The following AWS valves were closed this reporting week.

13 June- BI exit from 0650 until 0748 for trashrack repair.

15 June- BI exit from 1850 until 1859 for cleaning.

  - STS/VBS Inspections.  The following STS/VBSs were inspected this reporting week.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | 6/11 | 754 | A slot has a small patch that is coming loose. |
| 12 | 6/11 | 712 |  |
| 13 | 5/9 | 702 | Unable to inspect. WES test in progress. Meter reading taken 6/11 |
| 14 | 6/11 | 715 |  |
| 15 | 6/11 | 100 | Removed flow vane 10 June. |
| 16 | 6/11 | 640 |  |
| 17 | 6/4 | 634 | Mesh ok. Replaced top shaft at South end of A-slot. |
| 18 | 6/13 | 575 |  |

Table 1.  STS Inspection Dates.

  - Dewatering and Fish Salvage.  Biologists removed three 2.5’ sturgeon and approx. 45-50 adult lamprey from the Unit 1 draft tube on 11 June.  All fish were in good condition, several lamprey escaped down the grizzly.

- RESEARCH.

  - CRITFC Age and Length Composition.  Collected 70 Spring Chinook and 10 Sockeye this reporting week.

  - NMFS FGE.  Unit 17 FGE study is ongoing.  Summer tests are expected to be complete by 12 July.

  - NMFS Lamprey.  Collection continues.  Fish are being collected for NMFS, UI, USGS and the Warm Springs tribe.  The lamprey flume will be installed in the Bradford Island AWS channel on 17 June.

  - NMFS Saltwater Rearing.  Collection and rearing continue.

  - Normandeau Balloon-tag at Spillway.  Low flow study commences in August.

  - OCFWRU Transportation Study.  Study of post-transportation migration and survival of juveniles to run 29 April through 14 July.

  - ODFW Summer Steelhead sampling.  Sampling 2 days/week 8 hours/day.  Will sample 1% of the run.

  - Pelfrey Sturgeon Hatchery.  Capturing broodstock May 24 – June 24.

  - PSMFC Adult PIT.  Detectors are installed in all ladders.  Some of the detectors have developed leaks and will be monitored throughout the season.  Repairs are scheduled for 2002-03 winter maintenance season.

  - PSMFC JMF.  One kelt was released from the separator bars, 119 steelhead, 82 salmon and 12 unknown salmonids were counted as fallbacks, and 571 smolts were recorded going over the separator bars.

  - PSMFC/NMFS Juvenile PIT.  200 sub yearling Chinook were examined for signs of GBT.  Total descaling for all fish sampled at PH1 was 6.7%, down from 14.2% last week.  Incidence of fungus decreased for clipped steelhead and bird predation marks decreased for all steelhead.

  - U of I Adult Radio tracking.  Trapping began 1 April and will continue 7 days/week until October.

  - U of I Transportation Study.  Evaluation of physiological condition of river run v. barged Snake River juveniles to occur June-September.

  - USGS EMG studies.  Tissue samples will be taken from 200 Chinook 1 April through June.

  - USGS Juvenile Radio tracking.  Study in progress.

  - USGS Lamprey.  Collection continues.

  - USGS MGR.  Last release occurred 10 June.

  - USGS Sort by Code.  Length and weight are recorded and fish are released back to the river.

  - WDFW Fish Counting.  16-hr visual and 8-hr video counts at Bradford Island and Washington Shore.

  - WES/Battelle/Mevatech Juvenile Hydroacoustics.  Study in progress.

  - WES Spillway Deflector Testing.  WES monitoring water quality equipment to determine effectiveness of new flip lips.  Study began with initiation of spill on 10 April.

4.  FISHWAYS

  - Project biologists inspected 10-14 June this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 PCC velocity | Velocity <1.5fps on the electronic meter. | Unknown- the rest of the ladder is in criteria. | No action at this time. |  |
| 6/9, 6/15 |  | Biologists did not inspect 7 days/ week. | Personnel shortage. | Job advertised. |  |
| 6/10 | PH1 PCC | OG#9 not at correct elevation. | Unknown. | Gate manually adjusted. |  |
| 6/10, 6/12 | CI and B-Branch | Diffusers not in correct position for TW elevation. | Operators manually adjusted diffusers. | See Misc. 5.1 |  |
| 6/14 | Weir 37 | Reading <1.3’ | Unknown | Operators adjusted the control. |  |

  - Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Celsius.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 6/09 | 14.5 | 14.5 |
| 6/10 | 14.6 | 14.6 |
| 6/11 | 14.8 | 14.9 |
| 6/12 | 14.8 | 14.9 |
| 6/13 | 15.0 | 15.0 |
| 6/14 | 15.0 | 15.1 |
| 6/15 | 15.2 | 15.3 |

  - Sensor Calibration.  Calibration was checked on 12 June this reporting week.  Everything is within criteria.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - B-branch and Cascade Island are operated in manual mode until stillwells can be installed for the tailwater sensors.  Installation will wait until spill has concluded.

  - Turbulent flows from spillbays 1 and 18 enter the side entrance gates of Cascade Island and Bradford Island B-branch ladders, causing poor entrance conditions.  District hydraulic engineers are investigating the problem.  On 3 May, the structural crew installed a bulkhead in the side entrance at Cascade Island.  Entrance conditions improved.  CG funds will be used to build a bulkhead for Bradford Island B-branch.

  - Juvenile mortalities at the JMF continue to be an intermittent problem.  PSMFC is digitally photographing some of the fish.  DSM2 Orifices are flushing six times a day.

  - Pacific States reported intermittent descaling at DSM1 again this week.   The problem continues to decrease in severity.  Orifices are flushing six times a day.

  - There has been some difficulty in maintaining an opening for fish at PH1 weir gate 1.  The gate is currently programmed not to close if there is less than an 8’ opening, but it also won’t open automatically unless there is a problem with differential.  Electricians are exploring the possibility of reprogramming the gate such that it will open if there is less than a 6’ opening and not close if less than 8’.  Until then, both WG1 and FV1-1 will be manually adjusted as necessary to keep an opening at the south end of PH1 collection channel.

  - On 8 June excessive debris build-up at unit 17 again caused deflection of the VBSs.  Unit 17 screens have been plugging every other day.   Frequent gatewell dipping and screen cleaning have been taxing project resources.  On 10 June it was decided to remove the turning vane and perforated plate from Unit 15 VBSs  to prevent the same problem at that unit.  Unit 17 remained as is, but gatewell dipping and cleaning of Unit 17 VBSs are now performed during swing shift on FGE test days.

  - PH1 ITS was closed from 1454 until 1512 on 15 June to aid in recovering a drowned person’s body.

  - Seining of the old navigation lock is scheduled for next Thursday, 20 June.  Downstream gates will be closed after the fish are seined out of the lock chamber.

DEBORAH L. CHENOWETH

# Operations Project Manager

//S//