### CENWP-OP-B                                                                                                                                      07 October 2002

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 40 of 2002, which covers the period 29 September through 05 October 2002.

- OPERATION SUMMARY.

  - Daily river flow averaged 83.2 to 129.4 kcfs.  Daily project tailwater elevation averaged 8.0’ to 11.6’ msl.  Daily project forebay elevation averaged 73.1’ to 75.7’ msl.  River temperature averaged 64 to 61° F.  Daily turbidity measured between 6.0’ and 7.0’ on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, are open 6” during daylight hours.  Spill averaged. 1.9 to 2.1 kcfs.

  - Unit Operation

    - Unit 0 was out of service from 0500 until 1430 on 01 October for installation of a new relay.

    - Unit 1 remains out of service for turbine rehab.

    - Unit 5 remains out of service for turbine rehab.

    - Unit 7 was forced out of service at 1630 on 30 September due to the crane rail work.

    - Unit 8 remains forced out of service for crane rail work.

    - Unit 9 remains out of service for an overhaul.

    - Unit 10 was remains forced out of service for crane rail work.

    - Unit 11 was out of service from 1400 until 1612 on 30 September for STS replacement.

    - Unit 13 was out of service from 1048 until 1248 on 30 September for STS inspection/repair.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  F2 was out of service from 0942 until 1118 on 01 October for emergency maintenance on a failed farval line.

  - Gatewell drawdowns were measured on 02 October.

- ACTIVITIES.

  - AWS Closures.  The following AWS closures occurred this reporting week:

03 October- Weir 37 closed for cleaning from 1101 until 1142

  - STS/VBS Inspections.  The following STS/VBSs  inspections this reporting week.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | 9/30 | 494 | B-slot belt not running. Replaced with STS#3 |
| 12 | 10/1 | 433 |  |
| 13 | 10/2 | 432 |  |
| 14 | 10/2 | 447 |  |
| 15 | 10/2 | 594 |  |
| 16 | 10/2 | 626 |  |
| 17 | 10/2 | 698 |  |
| 18 | 10/2 | 673 |  |

Table 1.  STS Inspection Dates.

  - Dewatering and Fish Salvage.  Unit 9 tail logs were pulled on 01 October.  Nineteen sculpin were released back to the river.

- RESEARCH.

  - CRITFC Age and Length Composition.  Collected 42 chinook and 40 coho this reporting week.

  - NMFS Lamprey.  Tracking continues.

  - NMFS Saltwater Rearing.  Rearing continues.  Specific data available from Lyle Gilbreath.

  - ODFW Summer Steelhead sampling.  Sampling 2 days/week 8 hours/day.  Will sample 1% of the run.

  - PSMFC/NMFS Adult PIT.  Detectors have been installed in all ladders, some have developed leaks.  Repairs are scheduled for 2002-03 maintenance season.

  - PSMFC JMF.  One steelhead and nine salmon were recorded as fallback over the separator bars.

  - PSMFC/NMFS Juvenile PIT.  DSM1 flow is running south.

  - U of I Adult Radio tracking.  Trapping began 1 April and will continue 7 days/week into October.

  - U of I lamprey testing.  The AFF lamprey flume ceased operation this week.

  - WDFW Fish Counting.  16-hr visual and 8-hr video counts at Bradford Island and Washington Shore.

4.  FISHWAYS

  - Project biologists inspected 29 September through 05 October this week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | Main dam, PH2 | Avian wires broken or removed | Vandalism at main dam. See misc. 5.5 | Will be repaired this winter. |  |
| 10/1 | UMT | Reading <1.0’ | FV5-9 closed/ unknown | Operators to adjust |  |
| 10/1, 10/3, 10/5 | PH1 PCC velocity | Velocity <1.5 fps on electronic meter. | Unknown | No action taken. |  |
| 9/29-10/1, 10/5 | PH2 PCC velocity | Velocity <1.5fps on the electronic meter. | Low TW causes an inaccurate reading. | Sensor relocation this winter. |  |
| 10/2-4 | Weir 37 | Reading > 1.0’ | In manual | Stillwells to be installed in winter. |  |
| 10/1-5 | DMS1 orifice lights. | Lights burned out for more than 24 hours in Units 8-10. | Operators can’t get to bulbs due to crane rail work. | These units are out of service, no action taken. |  |

  - Fish Ladder Temperatures.  Measurements are taken 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Celsius.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 9/29 | 18.6 | 18.6 |
| 9/30 | 18.2 | 18.2 |
| 10/01 | 17.9 | 17.9 |
| 10/02 | 17.7 | 17.7 |
| 10/03 | 17.5 | 17.5 |
| 10/04 | 17.2 | 17.2 |
| 10/05 | 17.0 | 16.9 |

  - Sensor Calibration.  Checked calibration on 03 October this week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - B-branch and Cascade Island are in manual mode until tailwater stillwells can be installed this winter.

  - During low tailwater, the PH2 monoliths are unable to maintain greater than 13’ openings.

  - During low tailwater, the PH1 weir gates are unable to maintain greater than 8’ openings.

  - Repairing the PDS screen cleaner requires the JMF go to bypass.  The screen will be cleaned manually until the end of the season (31 October).

  - As of 30 September, Units 7,  8 and 10 are out of service due to the inability to deploy headgates during the crane rail construction.

  - The outfall hydrocannons were turned off on 30 September.

  - Due to probe problems, FV5-9 was in manual from 1600 on 01 October until 1040 on 02 October and FV6-9 went to manual at 1600 on 01 October.

JAMES R. MAHAR

# Operations Manager

//S//