### CENWP-OP-B                                                                                                                                  04 November 2002

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 44 of 2002, which covers the period 27 October through 02 November 2002.

- OPERATION SUMMARY.

  - Daily river flow averaged 96.3 to 140.9 kcfs.  Daily project tailwater elevation averaged 9.0’ to 12.0’ msl.  Daily project forebay elevation averaged 74.9’ to 75.5’ msl.  River temperature averaged 51 to 56° F.  Daily turbidity measured 4.0’ to 7.0’ on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, are open 6” during daylight hours.  Spill averaged. 1.9 to 2.2 kcfs.

  - Unit Operation

    - Unit 1 remains out of service for turbine rehab.

    - Unit 5 remains out of service for turbine rehab.

    - Unit 7 remains forced out of service for crane rail work.

    - Unit 8 remains forced out of service for crane rail work.

    - Unit 9 remains out of service for an overhaul.

    - Unit 10 remains forced out of service for crane rail work.

    - Unit 16 remains out of service for an overhaul.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  Fish Unit 1 went out of service at 0300 on 01 November for overhaul and stator rewind.

  - Gatewell drawdowns were measured on 30 October.

- ACTIVITIES.

  - AWS Closures.  The following AWS closures occurred this reporting week.

28 October- FV3-9 closed for cleaning from 0750 until 0800.

FV4-3 and FV4-4 closed for cleaning from 1027 until 1150.

FV5-3 and FV5-4 closed for cleaning from 1512 until 1650.

29 October- FV5-9 closed for cleaning from 0829 until 0850.

30 October- FV3-7 closed for cleaning from 0710 until 0835.

FV1-1 closed for cleaning from 1530 until 1600

01 November- FV1-1 closed for cleaning from 1115 until 1342.

  - STS/VBS Inspections.  No STS/VBSs were inspected this reporting week.

  - Dewatering and Fish Salvage.

31 October- The JMF was switched to bypass at 0700.  No fish were stranded.

01 November- Biologists removed one live sculpin and five dead juvenile shad from F1 draft tube.

- RESEARCH.

  - NMFS Lamprey.  Tracking continues.

  - NMFS Saltwater Rearing.  Rearing continues.  Specific data available from Lyle Gilbreath.

  - PSMFC/NMFS Adult PIT.  Some detectors have developed leaks, however no repairs are scheduled for 2002/03 maintenance season.

  - PSMFC JMF.  Two steelhead and one salmon were recorded as fallback over the separator bars.  Monitoring concluded at 0700 on 31 October.

  - WDFW Fish Counting.  24-hr video counts at Bradford Island and Washington Shore started on 01 November.

4.  FISHWAYS

  - Project biologists inspected 27, 29-31 October and 01-02 November this week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | Main dam, PH2 | Avian wires broken or removed | Vandalism and BCC work. | Will be repaired this winter. |  |
| All week | DSM 1 | Orifice lights out for more than 24 hours. | Crane rail contractors on site. | Operators to correct after hours. |  |
| 10/29-11/02 | PH1 PCC velocity | Velocity <1.5 fps on electronic meter. | Unknown | No action taken. |  |
| 10/27-11/01 | PH2 PCC velocity | Velocity <1.5fps on the electronic meter. | Low TW causes an inaccurate reading. | Sensor relocation this winter. |  |
| 10/30, 11/01 | Weir 37, 38, and 67 | Reading > or < 1.0’ | FV6-9 not controlling. | Maintenance working on valve. |  |
| 10/28 |  | Biologists failed to inspect 7 days/week | Short staffed |  |  |

  - Fish Ladder Temperatures.  Measurements are taken 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Celsius.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 10/27 | 14.2 | 14.2 |
| 10/28 | 14.0 | 14.0 |
| 10/29 | 13.7 | 13.7 |
| 10/30 | 13.0 | 13.0 |
| 10/31 | 12.2 | 12.2 |
| 11/01 | 11.7 | 11.7 |
| 11/02 | 11.4 | 11.4 |

  - Sensor Calibration.  Checked calibration on 27 October this week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - B-branch and Cascade Island are in manual mode until tailwater stillwells can be installed this winter.

  - During low tailwater, the PH2 monoliths are unable to maintain greater than 13’ openings.

  - During low tailwater, the PH1 weir gates are unable to maintain greater than 8’ openings.

  - FV6-9 went to manual at 0815 on 26 October due to excessive alarming.  The valve failed to control properly.  The valve went back to auto at 0800 on 29 October.  At 2140 on 29 October, FV6-9 alarmed again.  On 30 October, a new transducer was installed; the valve appears to be working properly now.

  - On 30 October, the Project has a hard constraint tailwater elevation of no greater than 16’ to facilitate B2 Corner Collector construction.

  - The juvenile monitoring facility went to bypass at 0700 on 31 October.

JAMES R. MAHAR

# Operations Manager

//S//