### CENWP-OP-B                                                                                                                                  24 November 2002

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 47 of 2002, which covers the period 17 through 23 November 2002.

- OPERATION SUMMARY.

  - Daily river flow averaged 118.9 to 127.3 kcfs.  Daily project tailwater elevation averaged 11.5’ to 11.8’ msl.  Daily project forebay elevation averaged 72.8’ to 75.0’ msl.  River temperature averaged 48° F.  Daily turbidity measured 6.2’ to 7.0’ on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, are open 6” during daylight hours.  Spill averaged 1.8 to 2.0 kcfs.

  - Unit Operation

    - Unit 1 remains out of service for turbine rehab.

    - Unit 7 remains forced out of service for crane rail work.

    - Unit 8 remains forced out of service for crane rail work.

    - Unit 9 remains out of service for an overhaul.

    - Unit 10 remains forced out of service for crane rail work.

    - Unit 16 remains out of service for an overhaul.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  Fish Unit 1 remains out of service for overhaul and stator rewind.

  - Gatewell drawdowns were measured on 20 November.

- ACTIVITIES.

  - AWS Closures.  No AWS closures occurred this reporting week.

  - STS/VBS Inspections.  No STS/VBSs were inspected this reporting week.

  - Dewatering and Fish Salvage.   No dewatering or fish salvage activities this reporting week.

- RESEARCH.

  - NMFS Lamprey.  Tracking continues.

  - NMFS Saltwater Rearing.  Rearing continues.  Specific data available from Lyle Gilbreath.

  - WDFW Fish Counting.  24-hr video counts at Bradford Island and Washington Shore.

  - WDFW Chum tagging.  Collection and tagging of adult chum salmon concluded on 23 November due to the AFF coming out of service on 25 November.

4.  FISHWAYS

  - Project biologists inspected 17-23 November this week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | Main dam, PH2 | Avian wires broken or removed | Vandalism and BCC work. | Will be repaired this winter. |  |
| All week | PH1 PCC velocity | Velocity <1.5 fps on electronic meter. | Unknown | No action taken. |  |
| 11/17, 11/20-23 | PH2 PCC velocity | Velocity <1.5fps on the electronic meter. | Low TW causes an inaccurate reading. | Sensor relocation this winter. |  |
| 11/17, 11/19-23 | Weir 37/38 | Reading > or < 1.0’ | Weir is in manual. | Stillwell to be installed this winter |  |
| 11/17 | North monolith | NDE differential <1.0’ | F1 OOS | Operators to adjust gates. |  |

  - Fish Ladder Temperatures.  Measurements are taken 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Celsius.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 11/17 | 10.0 | 10.1 |
| 11/18 | 10.1 | 10.1 |
| 11/19 | 10.2 | 10.2 |
| 11/20 | 10.3 | 10.3 |
| 11/21 | 10.4 | 10.4 |
| 11/22 | 10.4 | 10.4 |
| 11/23 | 10.4 | 10.4 |

  - Sensor Calibration.  Calibration checked on 17 November this week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - B-branch and Cascade Island are in manual mode until tailwater stillwells can be installed this winter.

  - During low tailwater, the PH2 monoliths are unable to maintain greater than 13’ openings.

  - During low tailwater, the PH1 weir gates are unable to maintain greater than 8’ openings.

  - As of 13 November, and until further notice, the Project has a hard constraint tailwater elevation no less than 11.3’ msl during all hours and a soft constraint of 11.3’-11.7’msl between 0700-1900 hours.  This is for the protection of the chum redds around Ives Island.

  - On 13 November, Unit 8A ESBS cleaner failed to function properly.  Until the screen can be pulled, it is out of service.  8B and 8C are still in operation.

  - Orifices 14A-S and 12A-N are open in local control due to a bad proximity switch and a bad air valve, respectively.

JAMES R. MAHAR

# Operations Manager

//S//