### CENWP-OP-B                                                                                                                                  31 December 2002

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 52 of 2002, which covers the period 22 through 28 December 2002.

- OPERATION SUMMARY.

  - Daily river flow averaged 110.7 to 120.6 kcfs.  Daily project tailwater elevation averaged 11.5’ to 11.9’ msl.  Daily project forebay elevation averaged 73.5’ to 75.0’ msl.  River temperature averaged 42 to 43° F.  Daily turbidity measured 6.2’ to 7.0’ on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, are now closed 24 hours.

  - Unit Operation

    - Unit 1 remains out of service for turbine rehab.

    - Unit 5 was out of service 0642-0830 on 22 December for STS removal.

    - Unit 10 was out of service 1012-1312 on 22 December for STS removal.

    - Unit 14 was out of service 1400-1530 on 22 December for STS removal.

    - Unit 16 remains out of service for an overhaul.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  During fish passage season, the units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  Fish Unit 1 remains out of service for overhaul and stator rewind.  F2 went out of service for winter maintenance at 0700 on 16 December.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES.

  - AWS Closures.  The following AWS valves were closed for cleaning.

    - FV1-1: On 26 December 1030-1323.

    - FV3-9: On 23, 24 (times not available), and 26 December 0957-1021.

  - STS/VBS Inspections.  No STS/VBSs were inspected this reporting week.

  - Dewatering and Fish Salvage.  There were no dewaterings this week.

- RESEARCH.

  - NMFS Lamprey.  Tracking continues.

  - NMFS Saltwater Rearing.  Rearing concluded in mid December.

  - WDFW Fish Counting.  24-hr video counts at Bradford Island.

4.  FISHWAYS

  - Project biologists inspected 18-20 December this week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | Main dam, PH1, PH2 | Avian wires broken or removed | Vandalism and contractor work. | Will be repaired this winter. |  |
| All week | PH1 PCC velocity | Velocity <1.5 fps on electronic meter. | Unknown | No action taken. |  |
|  | All | Calibration not checked. | Staff shortage because of holiday. | No action taken. |  |

  - Fish Ladder Temperatures.  Temperatures are not recorded in the winter.

  - Sensor Calibration.  Calibration was not checked this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - B-branch and Cascade Island are in manual mode until tailwater stillwells can be installed this winter.

  - During low tailwater, the PH1 weir gates are unable to maintain greater than 8’ openings.

  - As of 13 November, and until further notice, the Project has a hard constraint tailwater elevation no less than 11.3’ msl during all hours and a soft constraint of 11.3’-11.7’msl between 0700-1900 hours.  This is for the protection of the chum redds around Ives Island.

  - To facilitate the B2 corner collector work, a teletype was issued on 18 December detailing operations.  Those operations include PH1 priority, PH1 may operate outside of the 1% peak efficiency, no more than 4 units (preferably northern units) operating at PH2, any additional generation at PH2 or any spill requires a two hour notice.  These operations will be in effect until 15 January and are the result of damage to the construction site when six units were online at PH2.

JAMES R. MAHAR

# Operations Manager

//S//