### CENWP-OP-B                                                                                                                                  27 January 2003

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 04 of 2003, which covers the period 19 through 25 January 2003.

- OPERATION SUMMARY.

  - Daily river flow averaged 113.0 to 122.6 kcfs.  Daily project tailwater elevation averaged 11.3’ to 11.6’ msl.  Daily project forebay elevation averaged 73.5’ to 74.8’ msl.  River temperature averaged 40 to 41° F.  Daily turbidity measured 6.0’ to 7.0’ on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, are now closed 24 hours.

  - Unit Operation

    - Unit 0 was out of service from 0454 until 1736 on 21 January for brush work.

    - Unit 1 remains out of service for turbine rehab.

    - Unit 2 was out of service from 0712 until 1048 on 23 January for trashrack raking.

    - Unit 16 remains out of service for an overhaul.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  During fish passage season, the units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  Fish Unit 1 remains out of service for overhaul and stator rewind.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES.

  - AWS Closures.  The following AWS valves were closed for cleaning.

    - FV1-1: 2230 on 18 January until 0115 on 19 January to float debris, 1510 until 1540 on 21 January.

    - FV3-7: 0750 until 0820 on 22 January.

    - FV3-9: 1448 until 1510 on 21 January.

  - STS/VBS Inspections.  No STS/VBSs were inspected this reporting week.

  - Dewatering and Fish Salvage.  There were no dewaterings this week.

- RESEARCH.

  - NMFS Lamprey.  Tracking continues.

  - WDFW Fish Counting.  24-hr video counts at Bradford Island and Washington Shore.

4.  FISHWAYS

  - Project biologists inspected 21-24 January this week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | Main dam, PH1, PH2 | Avian wires broken or removed | Vandalism and contractor work. | Will be repaired this winter. |  |
| All week | PH1 PCC velocity | Velocity <1.5 fps on electronic meter. | Unknown | No action taken. |  |

  - Fish Ladder Temperatures.  Temperatures are not recorded in the winter.

  - Sensor Calibration.  Calibration was checked on 23 January this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - B-branch and Cascade Island are in manual mode until tailwater stillwells can be installed this winter.

  - During low tailwater, the PH1 weir gates are unable to maintain greater than 8’ openings.

  - On 24 January, the Project tailwater hard constraint decreased to 11.0’ msl for all hours.  The soft constraint went to  11.2’ msl between 0700-1900 hours.  This is for the protection of the chum redds around Ives Island.

  - Cascades Island went to tailwater on 24 January.  The water up date is still undecided.  We are waiting for direction from FPOM.

JAMES R. MAHAR

# Operations Manager

//S//