### CENWP-OP-B                                                                                                                                  10 February 2003

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 06 of 2003, which covers the period 02 through 08 February 2003.

- OPERATION SUMMARY.

  - Daily river flow averaged 139.8 to 219.2 kcfs.  Daily project tailwater elevation averaged 13.6’ to 20.8’ msl.  Daily project forebay elevation averaged 74.3’ to 75.5’ msl.  River temperature averaged 40 to 41° F.  Daily turbidity measured 1.4’ to 7.0’ on the secchi disk.

  - Spill Bay 18, which supplies attraction flow for Bradford Island ladder, is closed 24 hours.   Spill Bay 1, which supplies attraction flow for Cascade Island ladder, is open .5’ during daylight hours.  See misc. 5.5.

  - Unit Operation

    - Unit 1 remains out of service for turbine rehab.

    - Unit 5 was out of service from 1012 until 1106 on 06 February for exciter testing.

    - Unit 16 returned to service at 1900 on 06 February.  Unit 16 was out of service from 0900 until 1200 on 07 February to drain the turbine bearing oil.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  During fish passage season, the units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  Fish Unit 1 remains out of service for overhaul and stator rewind.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES.

  - AWS Closures.  FV4-3 and FV4-4 were closed at 1145 on 03 February to facilitate BI ladder dewatering activities.

  - STS/VBS Inspections.  No STS/VBSs were inspected this reporting week.

  - Dewatering and Fish Salvage.  Bradford Island fishway was dewatered on 03 February.  With help from PSMFC, WDFW and UI, biologists removed the following fish:

Mainstem- 33 lamprey, 20 steelhead adults, 11 sculpin, 18 juvenile steelhead, six juvenile chinook, 20 resident fish, and one stickleback.

Junction Pool- Four unknown adult steelhead, 16 clipped adult steelhead, 17 unclipped adult steelhead, 20 unknown juvenile steelhead, four juvenile chinook, one chinook fry, 27 chubs, 12 whitefish, one stickleback.

A-Branch- Two unknown adult steelhead, three crayfish, three sculpin, two chubs

B-Branch- One unknown adult steelhead, 30 chub, three sculpin.

The B-Branch entrance and diffuser pools were dewatered on 04 February.  Biologists removed six juvenile chinook, one clipped juvenile steelhead, and one lamprey from the entrance.  One clipped adult steelhead was removed from the diffuser pools.

- RESEARCH.

  - NMFS Lamprey.  Tracking continues.

  - WDFW Fish Counting.  24-hr video counts at Washington Shore.

  - WDFW steelhead.  Trapping should begin on 10 February.

4.  FISHWAYS

  - Project biologists inspected 04-07 February this week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | PH1, PH2 | Wires removed. | Contractor work. | Will replace in winter/spring. |  |
| All week | Main dam | Two wires broken | Vandalism | Will repair this winter/spring. |  |
| 2/5 | Weir 37 | Reading > or < 1.0’ | Unknown |  |  |
| All week | PH2 PCC velocity | Velocity <1.5 fps. | Meter is missing | see misc. 5.1 |  |

  - Fish Ladder Temperatures.  Temperatures are not recorded in the winter.

  - Sensor Calibration.  Calibration could not be checked as there were no calibrated sensors in operation.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - Divers discovered the PH2 velocity meter was missing from the collection channel.  This explains why the velocity has been out of criteria.  The project has a spare meter to be installed during the mid season dives this summer.

  - Cascade Island controls are in manual mode awaiting tailwater sensor calibration.  Electricians are awaiting parts.  B-branch is also in manual mode, pending installation of a tailwater stillwell.

  - On 24 January, the Project tailwater hard constraint decreased to 11.0’ msl for all hours.  The soft constraint went to 11.2’ msl between 0700-1900 hours.  This is for the protection of the chum redds around Ives Island.

  - DSM1 and DSM2 orifice lights timers have been disconnected.  Orifice lights will remain on 24 hours a day as per FPOM.

  - On 06 February, and continuing until further notice, Project spill increased to approximately 1.7 Kcfs from spill bay 1 during daylight hours.  This action was deemed necessary to attract wild winter steelhead to Cascades Island entrance while the south monolith and Bradford Island entrances are out of service.

  - The AFF was watered up on 04 February.

  - DSM2 returned to service at 1400 on 06 February.

JAMES R. MAHAR

# Operations Manager

//S//