### CENWP-OP-B                                                 Happy Easter                                                                   20 April 2003

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 16 of 2003, which covers the period 13 through 19 April 2003.

- OPERATION SUMMARY.

  - Daily river flow averaged 178.9 to 253.0 kcfs.  Daily project tailwater elevation averaged 16.3’ to 20.0’ msl.  Daily project forebay elevation averaged 74.4’ to 75.9’ msl.  River temperature averaged 48 to 49° F.  Daily turbidity measured 3.5’ to 5.0’ on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascade Island and Bradford Island fishways respectively, are included in the spill pattern.  Spill started at 75kcfs at 0420 on 14 April.  Spill was bumped up to 120kcfs at 1400 on 16 April.  Spill averaged 2.1 to 149.6 kcfs.

  - Unit Operation

    - Unit 1 remains out of service for turbine rehab.

    - Unit 15 was out of service from 0900 on 14 April until 1736 on 15 April for brake repairs.  Unit 15 went out of service at 1700 on 17 April due to exciter trouble.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  During fish passage season, the units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  Fish Unit 1 returned to service at 1506 on 15 April.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES.

  - AWS Closures.  The following AWS valves were closed for cleaning this reporting week.

FV1-1- 0714 until 0800 on 14 April.

  - STS/VBS Inspections.  No STS/VBS inspections were conducted this reporting week.

  - Dewatering and Fish Salvage.  No dewatering or fish salvage activities this reporting week.

- RESEARCH.

  - CRITFC.  Sampling of chinook, sockeye, and coho continues until 31 October.

  - NMFS Lamprey.  Tracking continues once a month.

  - ODFW Pikeminnow Tagging.  Electrofishing in Bonneville’s forebay and tailrace.

  - ODFW Steelhead.  Trapping 1% of the summer steelhead continues two days a week.

  - PSMFC at DSM1.  Continuous PIT tag interrogation continues.  GBT and monitoring began 07 April.

  - PSMFC at the SMF.  Six kelts (five alive and one dead) were stuck on the separator bars.  Monitoring personnel observed 161 steelhead, 60 salmon, eight unknown salmonids and 40 smolt slide over the separator bars.

  - Spring Creek NFH.  Spring Creek National Fish Hatchery released 4.1 million Tule fall chinook on 10 April.

  - University of Idaho.  Adult tracking tagging and tracking continue.

  - University of Idaho/WDFW jaw tagging.  Steelhead and spring chinook are tagged for studies involving the tooth tangle net fishery on the Columbia.

  - USGS Juvenile Radiotracking.  USGS still installing equipment around the project.

  - WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island.

  - WES/Battelle/BAE systems Juvenile hydroacoustic monitoring.  Installing equipment.

4.  FISHWAYS

  - Project biologists inspected 13-19 April this week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 PCC | Velocity <1.5fps | Meter in slowest part of channel | No response at this time. |  |
| All week | PH2 PCC | No velocity measure. | Meter is missing | See misc. 5.1 |  |
| 4/13 | B-Branch | Staff gauge reading <1.0’ | Unknown | No action taken. |  |
| 4/13 | North Monolith | Entrance differential <1.0’ | Unknown | Operators increased F2 MW |  |
| 4/14-16 | Weir 37 | Staff gauge reading >1.0’ | Unknown | Operators to adjust |  |

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Kelvin.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 4/13 | 282.8 | 282.8 |
| 4/14 | 283 | 282.9 |
| 4/15 | 283.2 | 283.2 |
| 4/16 | 283.3 | 283.3 |
| 4/17 | 283.3 | 283.2 |
| 4/18 | 283.4 | 283.3 |
| 4/19 | 283.5 | 283.4 |

  - Sensor Calibration.  Calibration was checked on 16 April this week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - Divers discovered the PH2 velocity meter was missing from the collection channel.  A new meter will be installed during the mid season dives this summer.

  - On 24 January, the Project tailwater hard constraint decreased to 11.0’ msl for all hours.  The soft constraint went to 11.2’ msl between 0700-1900 hours.  This is for the protection of the chum redds around Ives Island.  On 17 February, BPA agreed to a hard tailwater constraint of 11.3’ msl and a soft constraint of 11.5’ msl when inflows to Bonneville are >120Kcfs.

  - A request for a contract for a tug to remove debris from PH2 forebay has been made.

  - Sea lions and harbor seals have been observed in the tailrace of both powerhouses and the spillway.  No action is being taken by the project to harass, harm, or otherwise annoy these animals.  FFU is monitoring and recording their activity.

JAMES R. MAHAR

# Operations Manager

//S//