### CENWP-OP-B                                                                                                                                           08 June 2003

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 23 of 2003, which covers the period 01 through 07 June 2003.

- OPERATION SUMMARY.

  - Daily river flow averaged 280.9 to 352.9 kcfs.  Daily project tailwater elevation averaged 21.2’ to 24.8’ msl.  Daily project forebay elevation averaged 73.8’ to 75.1’ msl.  River temperature averaged 56 to 59° F.  Daily turbidity measured 3.2’ to 6.5’ on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascade Island and Bradford Island fishways respectively, are included in the spill pattern.  Spill averaged 90.1 to 144.7 kcfs.

  - Unit Operation

    - Unit 1 remains out of service for turbine rehab.

    - Unit 4 was out of service from 0636 until 1654 on 04 June for cooler replacement and bearing level checks.

    - Unit 14 returned to service at 1518 on 02 June.

    - Unit 15 was out of service from 0736 until 1148 on 04 June for VBS cleaning.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

    - F1 was out of service from 0900 until 1024 on 02 June for trashrack raking.  F1 was out of service from 1506 until 1530 on 06 June due to an emergency shutdown.

    - F2 was out of service from 1030 until 1400 on 02 June for trashrack raking.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES.

  - AWS Closures.  The following AWS valves were closed for cleaning this reporting week.

    - FV1-1 from 1457 until 1523 on 04 June.

  - STS/VBS Inspections.  The following STS/VBS inspections were conducted this reporting week.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | 6/3 | 850 |  |
| 12 | 6/3 | 798 |  |
| 13 | 6/3 | 777 |  |
| 14 | 6/3 | 448 |  |
| 15 | 5/20 | 134 | Cleaned VBS on 04 June |
| 16 | 6/3 | 766 |  |
| 17 | 5/20 | 671 | Inspected during gap loss study |
| 18 | 6/3 | 855 |  |

Table 1.  STS Inspection Dates.

  - Dewatering and Fish Salvage.  No dewatering or fish salvage activities this reporting week.

- RESEARCH.

  - CRITFC.  Sampling of chinook, sockeye, and coho continues until 31 October.

  - NMFS Lamprey.  Trapping, tagging and tracking continue.

  - OCFWRU.  Fish collection continues.

  - ODFW Steelhead.  Trapping continues.

  - PSMFC at DSM1.  Continuous PIT tag interrogation continues.  GBT and monitoring continue.

  - PSMFC at the SMF.  Monitoring personnel removed one dead kelt from the separator bars.  206 steelhead, 15 salmon, three unknown salmonids and 759 smolts slid over the separator bars.

  - University of Idaho.  Adult tracking tagging and tracking continue.

  - U of I Transportation Study.  Evaluation of river run vs. barged Snake River juveniles to occur May-June.

  - University of Washington.  Northern pikeminnow tagging and tracking from May through July.

  - USGS Lamprey.  Collected 111 lamprey from NMFS on 06 June.

  - WDFW jaw tagging.  Tagging concluded.

  - WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island.

  - WES/Battelle/BAE systems Juvenile hydroacoustic monitoring.  Gap loss monitoring ended 1 June for spring fish.  Gap loss monitoring for summer fish to begin 12 June.

4.  FISHWAYS

  - Project biologists inspected 01 through 07 June this week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 PCC | Velocity <1.5fps | Meter in slowest part of channel | No response at this time. |  |
| All week | PH2 PCC | No velocity measure. | Meter is missing | See misc. 5.1 |  |
| 6/1 | Cascades Is. | Entrance head <1.0’ | FV5-3 not controlling. | See misc. 5.4 |  |
| 6/2 | B-Branch | Water over weir <1.3’ +/- .1’ | Unknown | No action taken |  |
| 6/4-5 | Weir 37 | Reading >1.3’ +/- .1’ | Shad on grate | Valve backflushed |  |

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Kelvin.  http://students.washington.edu/kyle/temptable.jpg

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 6/1 | 287.8 | 287.7 |
| 6/2 | 287.8 | 287.8 |
| 6/3 | 288.0 | 287.9 |
| 6/4 | 288.1 | 288.1 |
| 6/5 | 288.6 | 288.6 |
| 6/6 | 289.1 | 289.1 |
| 6/7 | 289.2 | 289.4 |

  - Sensor Calibration.  Calibration was checked on 06 June this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - Divers discovered the PH2 velocity meter was missing from the collection channel.  A new meter will be installed during the mid season dives this summer.

  - At 1500 on 30 May, the Project is restricted to a maximum tailwater of 27’.  This is to prevent overtopping the bulkhead in the B2 Corner Collector.  The bulkhead went in about a month ago and is located at Station 24 +56 (near the start of the monolith).

  - The SMF PDS screen cleaner failed at 0020 on 02 June.  Electricians had it back in service at 1018 on 04 June.

  - FV5-3 was out of service from 1500 on 01 June until 0825 on 03 June due to a bad motor.

  - DSM2 screen cleaner #2 was out of service from 0702 until 1724 on 06 June due to a broken brush arm chain and from 2015 on 06 June until 1200 on 07 June for a broken brush arm chain.

JAMES R. MAHAR, P.E.

Operations Manager

//S//