### CENWP-OP-B                                                                                                                                           15 June 2003

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 24 of 2003, which covers the period 08 through 14 June 2003.

- OPERATION SUMMARY.

  - Daily river flow averaged 277.7 to 313.7 kcfs.  Daily project tailwater elevation averaged 20.6’ to 22.5’ msl.  Daily project forebay elevation averaged 73.5’ to 75.7’ msl.  River temperature averaged 59 to 60° F.  Daily turbidity measured 3.1’ to 5.0’ on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascade Island and Bradford Island fishways respectively, are included in the spill pattern.  Spill averaged 88.6 to 144.3 kcfs.

  - Unit Operation

    - Unit 1 remains out of service for turbine rehab.

    - Unit 8 was out of service from 1006 on 10 June until 1054 on 11 June for 8C ESBS repairs.

    - Unit 11 was out of the 1% criteria due to trash raking operations from 0842 until 1152 on 12 June.

    - Unit 15 went out of service at 0936 on 13 June due to a broken restoring cable.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

    - F2 had reduced load from 1254 until 1600 on 11 June to facilitate trashrack raking.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES.

  - AWS Closures.  The following AWS valves were closed for cleaning this reporting week.

    - FV3-7 from 0736 until 0830 on 09 June.

  - STS/VBS Inspections.  No STS/VBS inspections were conducted this reporting week.

  - Dewatering and Fish Salvage.  No dewatering or fish salvage activities this reporting week.

- RESEARCH.

  - CRITFC.  Sampling of chinook, sockeye, and coho continues until 31 October.

  - NMFS Lamprey.  Trapping, tagging and tracking continue.

  - OCFWRU.  Fish collection continues.

  - ODFW Steelhead.  Trapping continues.

  - PSMFC at DSM1.  Continuous PIT tag interrogation continues.  GBT and monitoring continue.

  - PSMFC at the SMF.  SMF personnel removed two live kelts from the separator bars.  200 steelhead, 24 salmon and 1337 smolts slid over the separator bars.

  - University of Idaho.  Adult tracking tagging and tracking continue.

  - U of I Transportation Study.  Evaluation of river run vs. barged Snake River juveniles to occur May-June.

  - University of Washington.  Northern pikeminnow tagging and tracking from May through July.

  - USGS Lamprey.  Collected 36 lamprey from NMFS on 13 June.

  - Warm Springs Tribes.  Collected 43 lamprey from NMFS on 13 June

  - WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island.

  - WES/Battelle/BAE systems Juvenile hydroacoustic monitoring.  Gap loss monitoring to resume mid June.

4.  FISHWAYS

  - Project biologists inspected 08 through 14 June this week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 PCC | Velocity <1.5fps | Meter in slowest part of channel | No response at this time. |  |
| All week | PH2 PCC | No velocity measure. | Meter is missing | See misc. 5.1 |  |
| 6/8, 6/11, 6/13 | Weir 37 | Reading >or <1.3’ +/- .1’ | Believed to be shad related. | Weir 37 cleaned. |  |

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Kelvin.  http://students.washington.edu/kyle/temptable.jpg

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 6/8 | 289.2 | 289.2 |
| 6/9 | 289.2 | 289.0 |
| 6/10 | 289.1 | 289.0 |
| 6/11 | 289.2 | 289.2 |
| 6/12 | 289.4 | 289.4 |
| 6/13 | 289.7 | 289.8 |
| 6/14 | 289.7 | 289.7 |

  - Sensor Calibration.  Calibration was checked on 14 June this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - Divers discovered the PH2 velocity meter was missing from the collection channel.  A new meter will be installed during the mid season dives this summer.

  - As of 1500 on 30 May, the Project is restricted to a maximum tailwater of 27’.  This is to prevent overtopping the bulkhead in the B2 Corner Collector.  The bulkhead went in about a month ago and is located at Station 24 +56 (near the start of the monolith).

  - From 1256 until 1350 on 09 June, Spillbays 10 and 12 were adjusted to compensate for Bay 11 being on seal while the gantry limit switches were set.

JAMES R. MAHAR, P.E.

Operations Manager

//S//