### CENWP-OP-B                                                                                                                                          13 July 2003

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 28 of 2003, which covers the period 06 through 12 July 2003.

- OPERATION SUMMARY.

  - Daily river flow averaged 141.5 to 199.0 kcfs.  Daily project tailwater elevation averaged 11.1’ to 15.6’ msl.  Daily project forebay elevation averaged 74.4’ to 75.5’ msl.  River temperature averaged 63 to 65° F.  Daily turbidity measured 4.7’ to 7.0’ on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascade Island and Bradford Island fishways respectively, are included in the spill pattern.  Spill averaged 84.6 to 98.0 kcfs.

  - Unit Operation

    - Unit 1 remains out of service for turbine rehab.

    - Unit 9 went out of service at 0430 on 10 July to facilitate headgate delivery and testing. See misc. 5.5

    - Unit 11 was out of service from 0448 until 1824 on 08 July for annual maintenance.

    - Unit 12 was out of service from 0354 until 1724 on 09 July for annual maintenance.

    - Unit 13 was out of service from 0430 until 1712 on 10 July for annual maintenance.

    - Unit 15 was out of service from 0642 until 1654 on 07 July and from 0706 until 0906 on 08 July for gap loss testing.

    - Unit 17 was out of service from 0830 until 1624 on 07 July for gap lost testing.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.   Fish Unit 1 was out of service from 2100 on 11 July until 0006 on 12 July for vibration testing.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES.

  - AWS Closures.  No AWS valves were closed this reporting week.

  - STS/VBS Inspections.  No STS/VBS inspections were conducted this reporting week.

  - Dewatering and Fish Salvage.  No dewatering or fish salvage activities this reporting week.

- RESEARCH.

  - CRITFC.  Sampling of chinook, sockeye, and coho continues until 31 October.

  - NMFS Lamprey.  Trapping, tagging and tracking continue.

  - OCFWRU.  Fish collection concluded this week.

  - ODFW Steelhead.  Trapping continues.

  - PSMFC at DSM1.  Continuous PIT tag interrogation continues.   See misc. 5.6.

  - PSMFC at the SMF.  Two salmon and one smolt slid over the separator bars.  GBT exams continue.

  - University of Idaho.  Adult tracking tagging and tracking continue.

  - U of I Transportation Study.  Evaluation of Snake River fall chinook will continue until 15 August.

  - University of Washington.  Northern pikeminnow tagging and tracking from May through July.

  - WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island.

  - WES/Battelle/BAE systems Juvenile hydroacoustic monitoring.  Gap loss monitoring continues until 21 July.

4.  FISHWAYS

  - Project biologists inspected 06 through 12 July this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| 7/6-7/11 | PH1 PCC | Velocity <1.5fps | Meter in slowest part of channel | No response at this time. |  |
| All week | PH2 PCC | No velocity measure. | Meter is missing | See misc. 5.1 |  |
| 7/12 | Weir 37 and 38 | Reading >or <1.3’ +/- .1’ | Unknown |  |  |
| 7/10-7/12 | PH1 ITS | Chain gates 7A and 10C are closed. | Headgate delivery. | 4B & 6B opened. See misc. 5.5 |  |

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Kelvin.  http://students.washington.edu/kyle/temptable.jpg   (295.37K = 22.22C= 72F)

| Date | Wash. Shore | Bradford Is. | ##### AFF exit |
| --- | --- | --- | --- |
| 7/6 | 291.8 | 291.8 | 291.8 |
| 7/7 | 291.9 | 291.9 | 291.9 |
| 7/8 | 292.0 | 292.0 | 292.0 |
| 7/9 | 292.2 | 292.2 | 292.1 |
| 7/10 | 292.5 | 292.5 | 292.6 |
| 7/11 | 292.7 | 292.7 | 292.7 |
| 7/12 | 293.1 | 293.0 | 293.1 |

  - Sensor Calibration.  Calibration was checked on 09 July this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - Divers discovered the PH2 velocity meter was missing from the collection channel.  A new meter will be installed during the mid season dives scheduled for 21 and 22 July.

  - As of 28 June, the Project will have a daytime spill of 75 kcfs.

  - Orifice 18A-N remains in local and open due to a bad proximity switch.

  - AFF picket lead #2 was back in service on 08 July.

  - Due to headgate delivery, chain gates 7A and 10C were closed on 10 July.  To compensate, 4B and 6B are opened to 72’msl and 70’ msl respectively.

  - As of 0430 on 12 July, DSM1 is in manual operation.  The PLC appears to be out of calibration.  An electrician will look at the system early next week.  Operators report a temporary cut in power to the flat plate detector when they tried to reboot the PLC.

JAMES R. MAHAR, P.E.

Operations Manager

//S//