### CENWP-OP-B                                                                                                                        03 August 2003

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 31 of 2003, which covers the period 27 July through 02 August 2003.

- OPERATION SUMMARY.

  - Daily river flow averaged 144.9 to 180.9 kcfs.  Daily project tailwater elevation averaged 11.5’ to 14.1’ msl.  Daily project forebay elevation averaged 74.7’ to 76.0’ msl.  River temperature averaged 68 to 70° F.  Daily turbidity measured 5.0’ to 7.0’ on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascade Island and Bradford Island fishways respectively, are included in the spill pattern.  Spill averaged 85.9 to 96.4 kcfs.

  - Unit Operation

    - Unit 1 remains out of service for turbine rehab.

    - Unit 7 went out of service at 0812 on 30 July for its five year overhaul.

    - Unit 12 was out of service from 0748 until 1618 on 29 July for exciter power pack replacement.

    - Unit 14 went out of service at 1006 on 28 July for its five year overhaul.

    - Unit 15 was out of service from 0642 until 1636 on 29 July for removal of modified STSs and VBSs.

    - Unit 17 was out of service from 0818 until 1612 on 28 July for the selysen motor replacement.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES.

  - AWS Closures.  No AWS valves were closed this reporting week.

  - STS/VBS Inspections.  No STS/VBS inspections were conducted this reporting week.

  - Dewatering and Fish Salvage.  The following dewatering/ fish salvage activities occurred this week.  All fish were released at the Hamilton boat launch.

    - 29 July- Unit 14 draft tube.  Fisheries removed 14 live sturgeon, nine channel catfish, one rainbow trout, two dead shad and two dead sturgeon.

    - 29 July- Unit 14 scroll case.  Fisheries removed approximately 40 dead shad.

    - 31 July- Unit 7.  Fisheries removed five live sturgeon, three lamprey, one channel catfish and two dead sturgeon (victims of the grizzly) from the draft tube.

- RESEARCH.

  - CRITFC.  Sampling has concluded until water levels drop below 70º F.

  - NMFS Lamprey.  Trapping, tagging and tracking continue.  See misc. 5.5

  - ODFW Steelhead.  Trapping continues.  See misc. 5.5

  - PSMFC at DSM1.  All PIT tag interrogation and monitoring concluded at 0800 on 31 July.

  - PSMFC at the SMF.  GBT exams continue.

  - University of Idaho.  Adult tracking tagging and tracking continue.  See misc. 5.5

  - U of I Transportation Study.  Evaluation of Snake River fall chinook will continue until 15 August.

  - WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island.

4.  FISHWAYS

  - Project biologists inspected 27 July through 02 August this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 PCC | Velocity <1.5fps | Meter in slowest part of channel | No response at this time. |  |
| All week | PH2 PCC | No velocity measure. | Cable not installed. | See misc. 5.1 |  |
| 7/27 | PH1 PCC | WG2 open < 8’ | Breaker tripped | See misc. 5.4 |  |
| 8/1-2 | PH1 ITS | Chain gate 7A and ITS end gate closed. | Crane rail work | 8B opened. See misc. 5.6 |  |

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Kelvin.  http://students.washington.edu/kyle/temptable.jpg   (295.37K = 22.22C= 72F)

| Date | WA Shore | Bradford Is. | ##### AFF exit |
| --- | --- | --- | --- |
| 7/27 | 294.3 | 294.3 | 294.4 |
| 7/28 | 294.8 | 294.8 | 294.9 |
| 7/29 | 295.2 | 295.1 | 295.2 |
| 7/30 | 295.5 | 295.5 | 295.6 |
| 7/31 | 295.6 | 295.6 | 295.6 |
| 8/1 | 295.3 | 295.3 | 295.3 |
| 8/2 | 295.2 | 295.0 | 295.2 |

  - Sensor Calibration.  Calibration was checked on 02 August this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- MISCELLANEOUS

  - A new velocity meter was installed for PH2 PCC during the mid season dive on 21 July.  We are now waiting for the electricians to finish installation.

  - As of 28 June, the Project will have a daytime spill of 75 kcfs.

  - In a 10 July teletype the Project was given a hard constraint maximum tailwater of 16’ until 01 September.  This is for the B2CC work at PH2.  In a 15 July teletype, the maximum tailwater constraint overrules spill to the 120% gas cap if spill raises tailwater over 16’.

  - WG2 stopped controlling properly on 23 July.  The gate wouldn’t open to greater than about a four foot opening.  With WG2 raised, FV1-1 closed down.  Differential was maintained in the collection channel but velocities were well below 1.5fps.  Operators reset WG2 on 27 July.

  - Water temperatures first reached 72ºF on 30 July.  Trapping concluded for the rest of the week at noon on 30 July.  Researchers will be able to return on 03 August for one day a week if temperatures are still at or above 72ºF or four days a week if temperatures are between 70ºF and 72ºF

  - Crane rail replacement work started at the south end of PH1 on 01 August.  Chain gate 7A was closed on 31 July so contractors could install their scaffolding, 8A has been opened to compensate.  As a safety precaution, the end gate will be closed during the hours the contractors are working over the ITS.  The end gate will be opened each evening to flush fish.  Coordination with NOAA Fisheries occurred on 01 August.

  - PH1 ITS was closed from 0655 until 1754 on 01 August and from 0700 until 1620 on 02 August for crane rail work.

JAMES R. MAHAR, P.E.

Operations Manager

//S//