### CENWP-OP-B                                                                                                                        15 September 2003

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 37 of 2003, which covers the period 07 through 13 September 2003.

- OPERATION SUMMARY.

  - Daily river flow averaged 73.6 to 93.7 kcfs.  Daily project tailwater elevation averaged 7.7’ to 9.1’ msl.  Daily project forebay elevation averaged 75.4’ to 76.9’ msl.  River temperature averaged 66 to 69° F.  Daily turbidity measured 4.0’ to 7.0’ on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascade Island and Bradford Island fishways respectively, are open .5’ from 0530 until 1930 each day.  Spill averaged 2.4 to 2.6 kcfs.

  - Unit Operation

    - Unit 1 remains out of service for turbine rehab.

    - Unit 2 remains out of service due to crane rail replacement.

    - Unit 3 remains out of service due to crane rail replacement.

    - Unit 4 remains out of service due to crane rail replacement.

    - Unit 5 remains out of service due to crane rail replacement.

    - Unit 6 remains out of service due to crane rail replacement.

    - Unit 7 remains out of service for its five year overhaul.

    - Unit 8 remains out of service due to crane rail replacement.

    - Unit 11 was out of service from 0644 until 1126 on 09 September for STS replacement.

    - Unit 14 remains out of service for its four year overhaul.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES.

  - AWS Closures.  The following AWS valves were closed this reporting week.

    - BI Exit closed for cleaning from 0715 until 0730 on 08 September

    - B-Branch and Cascades Island ladder computers in manual at 1600 on 08 September.  Having problems with the low TW indication.

    - FG3-3 went out of service at 0700 on 13 September.  The gate can still be hand cranked open or closed but the motor will not run in auto.

  - STS/VBS Inspections.  No STS/VBS inspections were conducted this reporting week.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | 9/8 | 670 | Rip in A slot STS, replaced with spare STS. VBS seam repaired. |
| 12 | 9/8 | 277 |  |
| 13 | 9/8 | 283 |  |
| 14 | 8/11 | 314 | Pulled for unit overhaul |
| 15 | 9/8 | 286 |  |
| 16 | 9/8 | 501 |  |
| 17 | 9/8 | 501 |  |
| 18 | 9/9 | 669 |  |

Table 1.  STS Inspection Dates.

  - Dewatering and Fish Salvage.  No dewatering or fish salvage activities occurred this week.

- RESEARCH.

  - CRITFC.   Sampling of chinook, sockeye, and coho continues until 31 October.  Reported one dead coho.  See misc 6.2

  - NMFS Lamprey.  Trapping, tagging and tracking continue.

  - ODFW Steelhead.  Trapping continues one to two days a week.

  - PSMFC at the SMF.  GBT exams conclude at the end of August.  One salmonid slid over the separator bars.

  - University of Idaho.  Adult tracking tagging and tracking continues.

  - WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island.

  - WDFW Tule counter.  Visual counts continue at Washington Shore and Bradford Island.

4.  FISHWAYS

  - Project biologists inspected 07 through 12 September this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR/WAR |
| --- | --- | --- | --- | --- | --- |
| 9/7-9/8, 9/10-9/12 | PH1 PCC | Velocity <1.5fps | Meter in slowest part of channel | No response at this time. |  |
| All week | PH2 PCC | No velocity measure. | Logic board was short circuited | Sending in to mfr for repair |  |
| All week | PH1 ITS | Chain gate 7A and ITS end gate closed. | Crane rail work | 8B opened. See const. 5.2.1 |  |
| 9/10-9/12 | Weir 37 | Reading >1.0’ | Fish clogging up the grate | Operators to blow off screen. |  |
| 9/13 |  | Project Bios did not inspect 7 days/week | Personnel shortage |  |  |

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Celsius this week.  http://students.washington.edu/kyle/temptable.jpg   (295.37K = 22.22C= 72F)

| Date | WA Shore | Bradford Is. | ##### AFF exit |
| --- | --- | --- | --- |
| 9/7 | 21.55 | 21.55 | 21.55 |
| 9/8 | 21.15 | 21.25 | 21.15 |
| 9/9 | 20.85 | 20.85 | 20.85 |
| 9/10 | 20.65 | 20.65 | 20.65 |
| 9/11 | 20.45 | 20.3 | 20.45 |
| 9/12 | 20.25 | 20.25 | 20.25 |
| 9/13 | 19.95 | 19.85 | 19.85 |

  - Sensor Calibration.  Calibration was checked on 12 September this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - PH 2 Corner Collector

    - In a 10 July teletype the Project was given a hard constraint maximum tailwater of 16’ until 01 September.  This is for the B2CC work at PH2.  In a 15 July teletype, the maximum tailwater constraint overrules spill to the 120% gas cap if spill raises tailwater over 16’.

  - PH 1 Crane Rail Replacement- started on 01 August and will continue until 16 December.

    - As a safety precaution, chain gate 7A has been closed (8A to compensate) and the end gate will be closed during the hours of 0530 until 1900 for the contractors working over the ITS.  The end gate will be opened each evening to flush fish.

    - PH1 priority is as follows until 16 September; Unit 9 before Unit 10 without screens.  After 16 September; Unit 10 before Unit 9 with screens.  The ITS will be opened during unit operation and allowed to run for not less than four hours after the units have been shutdown (to flush fish that may have been attracted into PH1 forebay).

    - PH1 ITS, Unit 9 and Unit 10 operations summary

- September- ITS closed from 0530 until 1430

- September- ITS closed from 0600 until 1705

- September- ITS closed from 0520 until 1750

- September- ITS closed from 0530 until 1706

- September- ITS closed from 0630 until 1645

- September- ITS closed from 0541 until 0940, Units 9 and 10 in operation at 0942.

- MISCELLANEOUS

  - FV6-9 remains in manual.

  - Due to high numbers of fish coming through the Washington Shore fishway, researchers are raising one or two picket leads during trapping operations.

  - CRITFC reported one dead coho on 09 September.  Please call John Whiteaker for more information.

  - At 0936 on 12 September, RCC issued a teletype directing Bonneville to run PH1 units as the priority.  This action was taken in an attempt to more evenly distribute the high numbers of salmon going through the Washington Shore fishway and relatively few going through Bradford Island.

  - At 1630 on 13 September, DSM2 screen cleaner #2 failed.   It was returned to auto at 0715 on 14 September.

JAMES R. MAHAR, P.E.

Operations Manager

//S//