### CENWP-OP-B                                                                                                                        10 November 2003

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 45 of 2003, which covers the period 02 through 08 November 2003.

- OPERATION SUMMARY.

  - Daily river flow averaged 120.3 to 140.1 kcfs.  Daily project tailwater elevation averaged 10.4 to 12.1’ msl.  Daily project forebay elevation averaged 74.7’ to 75.6’ msl.  River temperature averaged 51 to 54° F.  Daily turbidity measured 5.0’ to 7.0’ on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascade Island and Bradford Island fishways respectively, are open .5’ from 0530 until 1930 each day.  Spill averaged 2.3 to 2.4 kcfs.

  - Unit Operation

    - Unit 1 remains out of service for turbine rehab.

    - Units 2 through 4 remain out of service due to crane rail replacement.

    - Unit 11 remains out of service due to an emergency shutdown on 17 October.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES.

  - AWS Closures.  The following AWS valves were closed for cleaning this reporting week.

FV1-1- closed for cleaning from 1515 until 1530 on 03 November, from 1509 until 1541 on 06 November, and 1022 until 1055 on 08 November.

FV3-7- closed for cleaning from 1105 until 1145 on 03 November, from 1317 until 1327 on 05 November, and from 1300 until 1320 on 07 November.

FV3-9- closed for cleaning from 0700 until 0712 on 03 November, from 1317 until 1327 on 05 November, from 1300 until 1320 on 07 November, and from 1115 until 1200 on 08 November.

FV6-9- closed for cleaning from 1400 until 1420 on 07 November.

BI Exit- closed for cleaning from 1300 until 1320 on 07 November and from 1115 until 1200 on 08 November.

  - STS/VBS Inspections.  No STS/VBS inspections were conducted this reporting week.

  - Dewatering and Fish Salvage.  No dewatering or fish salvage activities occurred this week.

- RESEARCH.

  - WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island concluded on 01 November.  16 hr video counts for both ladders began on 01 November.

  - WDFW Chum.  Chum trapping and tagging began on 04 November.

4.  FISHWAYS

  - Project biologists inspected 02 through 08 November this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 PCC | Velocity <1.5fps | Meter in slowest part of channel | No response at this time. |  |
| All week | PH2 PCC | No velocity measure. | Logic board was short circuited | Sending in to mfr for repair |  |
| All week | FG3-3, FG3-4 | FG3-3 OOS | Mechanical binding. | FG3-4 to compensate. |  |
| 11/7 | PH2 monoliths | Head differential <1.0’ –2.0” | Fish unit trashracks being raked. | Ladder back in criteria after F1 & F2 trashrack raking. |  |

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Kelvin this week.  http://students.washington.edu/kyle/temptable.jpg   (295.37K = 22.22C= 72F)

| Date | WA Shore | Bradford Is. |
| --- | --- | --- |
| 11/02 | 286.5 | 286.3 |
| 11/03 | 286.0 | 285.9 |
| 11/04 | 285.9 | 285.8 |
| 11/05 | 285.5 | 285.4 |
| 11/06 | 285.1 | 285.0 |
| 11/07 | 284.9 | 284.8 |
| 11/08 | 284.6 | 284.5 |

  - Sensor Calibration.  Calibration was checked on 08 November this week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - PH 2 Corner Collector- Contract coming to a close.

  - PH 1 Crane Rail Replacement- started on 01 August and will continue until 16 December.

    - As a safety precaution, chain gate 7A (8B to compensate) is closed during construction.

- MISCELLANEOUS

  - Per a teletype sent on 31 October, Project tailwater will have a soft constraint of 11.2’ to 11.5’ msl between the hours of 0700 until 1900.  This constraint is for chum and the spawning habitat located below the dam.

  - PH1 ITS was closed from 1255 until 1405 on 06 November.  This was to facilitate the removal of logs from chain gate 10C.

  - Per a teletype dated 07 November, Project elevation will be held within a one-foot range to facilitate hydraulic measurements on 10 November from 1000 until 1600.

JAMES R. MAHAR, P.E.

Operations Manager

//S//