### CENWP-OP-B                                                                                                                        17 November 2003

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 46 of 2003, which covers the period 09 through 15 November 2003.

- OPERATION SUMMARY.

  - Daily river flow averaged 119.8 to 155.2 kcfs.  Daily project tailwater elevation averaged 11.4 to 11.5’ msl.  Daily project forebay elevation averaged 73.4’ to 75.3’ msl.  River temperature averaged 49 to 50° F.  Daily turbidity measured 5.0’ to 7.0’ on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascade Island and Bradford Island fishways respectively, are open .5’ from 0530 until 1930 each day.  Spill averaged 2.3 to 2.4 kcfs.

  - Unit Operation

    - Unit 1 remains out of service for turbine rehab.

    - Units 2 through 4 remain out of service due to crane rail replacement.

    - Unit 8 was out of service from 1130 on 09 November until 1356 on 13 November due to a broken ESBS.

    - Unit 11 remains out of service due to an emergency shutdown on 17 October.

    - Unit 17 had a 1% violation from 0730 until 0815 on 13 November due to trashrack raking activities.

    - Unit 18 had a 1% violation from 1028 until 1608 on 12 November and from 0705 until 0720 on 13 November to facilitate trashrack raking.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES.

  - AWS Closures.  The following AWS valves were closed for cleaning this reporting week.

FV1-1- closed for cleaning from 0813 until 0849 on 10 November and from 0715 until 0743 on 15 November.

FV3-7- closed for cleaning from 0827 until 0900 on 13 November.

FV3-9- closed for cleaning from 0714 until 0741 on 10 November, from 0652 until 0708 on 12 November, from 0748 until 0800 on 13 November, and from 0750 until 0819 on 15 November.

FV5-9- closed for cleaning from 0819 until 0825 on 15 November.

FV6-9- closed for cleaning from 0825 until 0900 on 15 November.

  - STS/VBS Inspections.  The following STS/VBS inspections were conducted this reporting week.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | 11/13 | 258 |  |
| 12 | 11/13 | 817 |  |
| 13 | 11/13 | 825 |  |
| 14 | 11/13 | 266 |  |
| 15 | 11/13 | 797 |  |
| 16 | 11/17 | 968 |  |
| 17 | 11/17 | 1009 | Repaired rip in “A” slot |
| 18 | 11/17 | 1008 | Repaired several rips in “A” slot |

Table 1.  STS Inspection Dates.

  - Dewatering and Fish Salvage.  The following dewatering or fish salvage activities occurred this week.

2.3.1 DSM1 turned south at 1400 on 10 November.  No fish were salvaged.

- RESEARCH.

  - WDFW Fish Counting.  16-hr video counts at Washington Shore and Bradford Island.

  - WDFW Chum.  Chum trapping and tagging continues.

4.  FISHWAYS

  - Project biologists inspected 09 through 15 November this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 PCC | Velocity <1.5fps | Meter in slowest part of channel | No response at this time. |  |
| All week | PH2 PCC | No velocity measure. | Logic board was short circuited | Meter replaced on 13 November. |  |
| All week | FG3-3, FG3-4 | FG3-3 OOS | Mechanical binding. | FG3-4 to compensate. |  |
| 11/14 | Weir 37 | Reading >1.0’ over the weir. | Unknown | Weir cleaned. |  |

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Kelvin this week.  http://students.washington.edu/kyle/temptable.jpg   (295.37K = 22.22C= 72F)

| Date | WA Shore | Bradford Is. |
| --- | --- | --- |
| 11/09 | 284.5 | 284.5 |
| 11/10 | 284.6 | 284.6 |
| 11/11 | 284.2 | 284.2 |
| 11/12 | 283.9 | 283.9 |
| 11/13 | 283.6 | 283.6 |
| 11/14 | 283.5 | 283.5 |
| 11/15 | 283.3 | 283.3 |

  - Sensor Calibration.  Calibration was checked on 15 November this week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - PH 2 Corner Collector- Avian anchors to be installed.

  - PH 1 Crane Rail Replacement- started on 01 August and will continue until 16 December.

    - As a safety precaution, chain gate 7A (8B to compensate) is closed during construction.

- MISCELLANEOUS

  - Per a teletype sent on 13 November, Project tailwater will have a soft constraint of 11.3’ to 11.6’ msl between the hours of 0700 until 1900.  This constraint is for chum spawning habitat located below the dam.

JAMES R. MAHAR, P.E.

Operations Manager

//S//