### CENWP-OP-B                                                                                                                        01 December 2003

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 48 of 2003, which covers the period 23 through 29 November 2003.

- OPERATION SUMMARY.

  - Daily river flow averaged 114.5 to 144.3 kcfs.  Daily project tailwater elevation averaged 11.5’ to 13.5’ msl.  Daily project forebay elevation averaged 73.2’ to 75.8’ msl.  River temperature averaged 45 to 46° F.  Daily turbidity measured 6.0 to 7.0 on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island fishways respectively, are open .5’ from 0530 until 1930 each day.  Spill averaged 2.4 to 2.5 kcfs.

  - Unit operation

    - Unit 1 remains out of service for turbine rehab.

    - Units 2 through 4 remain out of service due to crane rail replacement.

    - Unit 10 went out of service at 0130 on 27 November due to exciter trouble.

    - Unit 11 remains out of service due to an emergency shutdown on 17 October.

    - Unit 12 was out of service from 1052 until 1326 on 25 November for STS removal.

    - Unit 13 was out of service from 0841 until 1053 on 25 November for STS removal.

    - Unit 14 was out of service from 0714 until 0839 on 25 November for STS removal.

    - Unit 15 was out of service from 1316 until 1528 on 24 November for STS removal.

    - Unit 16 was out of service from 1047 until 1312 on 24 November for STS removal.

    - Unit 17 was out of service from 0833 until 1039 on 24 November for STS removal.

    - Unit 18 was out of service from 0644 until 0827 on 24 November for STS removal.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.   F1 returned to service at 0200 on 23 November.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES.

  - AWS Closures.  The following AWS valves were closed for cleaning this reporting week.

FV1-1- from 1525 until 1548 on 25 November.

FV3-7- from 1325 until 1630 on 24 November.

FV3-9- from 1325 until 1630 on 24 November and from 1555 until 1615 on 25 November.

  - STS/VBS Inspections.  The following STS/VBS were inspected this reporting week.

| 11 | 11/24 | 0 | Removed for DSM2 dewater |
| --- | --- | --- | --- |
| 12 | 11/25 | 331 | Removed for DSM2 dewater |
| 13 | 11/25 | 331 | Removed for DSM2 dewater |
| 14 | 11/25 | 300 | Removed for DSM2 dewater |
| 15 | 11/24 | 331 | Removed for DSM2 dewater |
| 16 | 11/24 | 159 | Removed for DSM2 dewater |
| 17 | 11/24 | 161 | Removed for DSM2 dewater |
| 18 | 11/24 | 159 | Removed for DSM2 dewater |

Table 1.  STS Inspection Dates.

  - Dewatering and Fish Salvage.  Upcoming activities include-

    - DSM2/SMF dewatering and two mile pipe inspection on 01/02 December.

    - Cascades Island dewater on 03 December.

- RESEARCH.

  - University of Idaho.  Removed antenna cables from the spillway on 26 November.

  - WDFW Fish Counting.  16-hr video counts at Washington Shore and Bradford Island.

  - WDFW Chum.  Chum trapping and tagging continues.

4.  FISHWAYS

  - Project biologists inspected 23 through 29 November this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 PCC | Velocity <1.5fps | Meter in slowest part of channel | No response at this time. |  |
| 11/23-26, 11/28 | PH2 PCC | Velocity <1.5fps |  | No response |  |
| All week | FG3-3, FG3-4 | FG3-3 OOS | Mechanical binding. | FG3-4 to compensate. |  |
| 11/24 | BI weirs | Reading <1.0’ over the weir. | AWS trashracks need cleaned. | Trash raking underway |  |

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Kelvin this week.  http://students.washington.edu/kyle/temptable.jpg   (295.37K = 22.22C= 72F)

| Date | WA Shore | Bradford Is. |
| --- | --- | --- |
| 11/23 | 281.9 | 281.9 |
| 11/24 | 281.8 | 281.8 |
| 11/25 | 281.7 | 281.8 |
| 11/26 | 281.7 | 281.7 |
| 11/27 | 281.6 | 281.7 |
| 11/28 | 281.5 | 281.6 |
| 11/29 | 281.8 | 281.7 |

  - Sensor Calibration.  Calibration was checked on 29 November this week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - PH 2 Corner Collector- Repairs to the ogee underway.  Another gate test will be scheduled once the concrete has cured, tentatively expected for late December.

  - PH 2 Corner Collector Hi-Q PIT tag detector-  Prototype still planned for installation in 2004.

  - PH 1 Crane Rail Replacement- started on 01 August and will continue until 16 December.

    - As a safety precaution, chain gate 7A (8B to compensate) is closed during construction.

- MISCELLANEOUS

  - Per a teletype sent on 17 November, Project tailwater will have a soft constraint of 11.4’ to 11.7’ msl between the hours of 0700 until 1900.  This constraint is for chum spawning habitat located below the dam.

  - Project Fisheries is working with Bonneville Rangers in submitting a grant proposal for the Trout Unlimited Embrace A Stream grant.  We are looking at weed removal, re-vegetation, and in stream restoration at Bass Lake.

  - A new teletype was issued on 25 November detailing flow restrictions on 02 December to facilitate the installation of the boat exclusion device at the B2CC.

JAMES R. MAHAR, P.E.

Operations Manager

//S//