### CENWP-OP-B                                                                                                                        29 December 2003

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 52 of 2003, which covers the period 21 though 27 December 2003.

- OPERATION SUMMARY.

  - Daily river flow averaged 115.4 to 142.1 kcfs.  Daily project tailwater elevation averaged 12.2’ to 13.8’ msl.  Daily project forebay elevation averaged 73.0’ to 73.7’ msl.  River temperature averaged 41 to 42° F.  Daily turbidity measured 5.0 to 7.0 on the secchi disk.

  - Spill Bay 18, which supplies attraction flow for the Bradford Island fishway, is open .5’ from 0630 until 1600 each day.  Spill averaged .7 to .8 kcfs.  Spillbay 1 will remain closed until Cascades Island returns to service.

  - Unit operation

    - Unit 1 remains out of service for turbine rehab.

    - Units 2 and 3 had a 1% violation due to curve testing on 23 December.

    - Unit 18 had a 1% violation due to trash rack raking from 11400 until 1621 on 23 December.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

    - F1 remains out of service for testing and winter maintenance.

    - F2 remains out of service for F1 testing and winter maintenance.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES.

  - AWS Closures.  The following AWS valves were closed for cleaning this reporting week.

FV1-1- from; 1215 until 1230 on 21 December, 1007 until 1045 on 22 December, 1044 until 1128 on 23 December, 0828 until 0900 on 24 December, and 0845 until 0900 on 26 December.

FV3-7- from; 0736 until 0918 on 22 December, 0749 until 0820 on 24 December, and 0750 until 0830 on 26 December.

FV3-9- from; 1140 until 1215 on 21 December, 0736 until 0918 on 22 December, 0958 until 1012 on 23 December, 0749 until 0820 on 24 December, and 0750 until 0830 on 26 December.

  - STS/VBS Inspections.  All STSs are removed until fish passage season.

  - Dewatering and Fish Salvage.  No dewatering or salvage activities this reporting week.

    - Upcoming events

05 Jan- Washington Shore and Cascades Island return to service.

12 Jan- two mile pipe survey

20 Jan- Bradford Island fishway dewatering

22 Jan- PH1 Collection Channel dive inspection

- RESEARCH.

  - NOAA Fisheries.  Demonstration of measuring and weighing board at the AFF on/around 12 January.

  - WDFW Fish Counting.  16-hr video counts at Bradford Island.

  - WES/Battelle/BAE Systems.  Collecting preliminary hydroacoustic data on MU 10.

4.  FISHWAYS

  - Project biologists inspected 22-24 December this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 PCC | Velocity <1.5fps | Meter in slowest part of channel | No response at this time. |  |
| All week | FG3-3, FG3-4 | FG3-3 OOS | Mechanical binding. | FG3-4 to compensate. |  |
| 12/23 | A-Branch | Water over weir >1.0’ | Unknown. | No response. |  |

  - Fish Ladder Temperatures.  Temperatures are not recorded during winter maintenance season.

  - Sensor Calibration.  Calibration was checked on 22 December this week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - PH 2 Corner Collector- Repairs to the ogee underway.  Another gate test is scheduled on 8-9 January.

  - PH 2 Corner Collector Hi-Q PIT tag detector- Prototype still planned for installation in 2004.

  - PH2 Corner Collector Avian Wires- Frame work completed for avian wire installation scheduled in February over the plunge pool.

  - PH 1 Crane Rail Replacement- Started on 01 August and will continue until 31 December.

  - PH1 PSC Removal.  Scheduled for 05 January 04.

- MISCELLANEOUS

  - Project Fisheries is working with Bonneville Rangers in submitting a grant proposal for the Trout Unlimited Embrace A Stream grant.  We are looking at weed removal, re-vegetation, and in stream restoration at Bass Lake.

  - Per a teletype sent on 18 December, Project tailwater will have a hard constraint of 11.8’ and a soft constraint of 11.8’ to 12.1’ between the hours of 0700 until 1700.  This constraint is for chum spawning habitat located below the dam.

JAMES R. MAHAR, P.E.

Operations Manager

//S//