### CENWP-OP-B                                                                                                                                     09 March 2004

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 10 of 2004, which covers the period 29 February through 06 March 2004.

- OPERATION SUMMARY.

  - Daily river flow averaged 126.5 to 171.9 kcfs.  Daily project tailwater elevation averaged 12.1’ to 14.3’ msl.  Daily project forebay elevation averaged 74.6’ to 75.9’ msl.  River temperature averaged 39 to 40° F.  Daily turbidity measured 5.0 to 6.0 on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are open .5’.  Spill averaged 0 to 50.3 kcfs.  50 kcfs was spill for Spring Creek fish from 02 until 05 March.

  - Unit operation.  PH2 became the priority powerhouse on 24 February.

    - Unit 1 remains out of service for turbine rehab.

    - Unit 9 was out of service from 0713 until 0806 on 02 March for transducer installation.

    - Unit 10 remains out of service.

    - Unit 11 remains out of service.

    - Unit 17 remains out of service for VBS modifications.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

    - F1 returned to service at 0812 on 01 March.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES.

  - AWS Closures. The following AWS valves were closed this reporting week.

FV1-1—closed for cleaning from 1037 until 1112 on 01 March.

  - STS/VBS Inspections.  No STSs were installed this reporting week.

  - Dewatering and Fish Salvage.  The following dewatering or salvage activities this reporting week.

Navlock- The upstream miter gate was dewatered on 06 March.  No fish were found.

- RESEARCH.

  - Battelle FGE testing.  Hydroacoustic installation complete.

  - CRITFC.  Scheduled to start in mid-March.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - NOAA FISH lamprey.  Lamprey ladder installation in Spring.  Meeting on 10 March to discuss the ladder design and placement at Bonneville.

  - NOAA FISH NaCl.  Maintaining the saltwater facility though no research is scheduled this season.

  - PSMFC SMF.  Sampling and PIT tag Interrogation began at 0700 on 02 March.

  - PSMFC PIT tag.  Testing an antenna for the serpentine sections of the fishways.

  - Spring Creek Hatchery.  First release scheduled for 01 March.  Spill began at 2010 on 02 March to evaluate passage via spill of 3.7 million Tule Fall chinook.  Second release scheduled for 10 March.

  - USGS FPE testing.  Radio telemetry equipment to be installed on the Project throughout March.

  - WDFW Fish Counting.  16-hr video counts at Washington Shore and Bradford Island.

4.  FISHWAYS

  - Project biologists inspected 01-06 March this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 PCC | Velocity <1.5 fps | Meter in slowest part of the channel. | No action taken. This is a normal condition. |  |
| 3/5-6 | FG3-4 | Not in correct position for TW | FG3-4 has a bent shaft and is closed. | Diffuser may not be repaired until winter 04/05. |  |

  - Fish Ladder Temperatures.  .  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Fahrenheit.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 3/1 | 41.4 | 42.4 |
| 3/2 | 41.6 | 42.7 |
| 3/3 | 41.7 | 42.6 |
| 3/4 | 41.8 | 42.7 |
| 3/5 | 41.8 | 42.9 |
| 3/6 | 41.9 | 43.2 |

  - Sensor Calibration.  Calibration was checked on 02 March this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Fishway Status.

    - DSM2 airburst is run in manual every six hours.  Debris loading will be monitored.

    - DSM1 remains out of service.

    - PH1 chaingates were adjusted on 01 March.  2C, 4C, and 6C are set at 71.5’.  All other gates are closed.

    - Antennas in the B2CC gate slot filler.  The corner collector will be opened for the second Spring Creek release.  Per a teletype sent out on 08 March, the corner collector will be opened from 1500 on 11 March until 1500 on 15 March.

    - Navlock dewatering started at 0703 on 06 March.

- CONSTRUCTION

  - PH 2 Corner Collector- Antennas being installed in the gate slot filler panel.

  - PH 2 Corner Collector Hi-Q PIT tag detector- Prototype still planned for installation in 2004.

  - Unit 17 VBS mods.  From the hours between 1200-1800, after 1 March 2004 until completion, no work such as concrete saw cutting, bush and vibro-hammering may be done inside the gatewells of unit 17 at PH2.  Between 0000-1200 and 1800-2400 hours, the contractor may complete work inside the gatewells with the associated equipment and construction techniques dealing with concrete saw cutting, bush and vibro-hammering.

  - B2 Follow-on.  The contractors finished the airburst installation.  Due to problems with the I/O hardware, the system will be ran in manual every six hours (or more often if necessary) until repairs can be made.

- MISCELLANEOUS

  - Per a teletype sent on 18 December, Project tailwater will have a hard constraint of 11.8’ and a soft constraint of 11.8’ to 12.1’ between the hours of 0700 until 1700.  This constraint is for chum spawning habitat located below the dam.

  - Sea lions and harbor seals have been observed in the tailrace of both powerhouses and the spillway.  No action is being taken by the project to harass, harm, or otherwise annoy these animals.

  - On the morning of 27 February, an oil sheen was found in gatewell slot 11C.  Operators shut down all Unit 11 gatewell orifices.  All orifices were back in normal operation by 1645 on 01 March.

  - The hydro cannons at the SMF are not operational at this time.  There is a break in the water lines.  The Project is working on repairs.  Please see memo sent out by Project Fisheries on 02 March.

  - The PH1 ITS was closed from 0920 until 1305 and again from 1340 until 1620 on 01 March to facilitate work on the +90’ deck.

JAMES R. MAHAR, P.E.

Operations Manager

//S//