### CENWP-OP-B                                                                                                                                     17 March 2004

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 11 of 2004, which covers the period 07 through 13 March 2004.

- OPERATION SUMMARY.

  - Daily river flow averaged 121.3 to 144.8 kcfs.  Daily project tailwater elevation averaged 12.1’ to 13.1’ msl.  Daily project forebay elevation averaged 74.4’ to 76.2’ msl.  River temperature averaged 40 to 42° F.  Daily turbidity measured 6.0 to 7.0 on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are open .5’.  Spill averaged 2.1 to 2.2 kcfs.

  - Unit operation.  PH2 became the priority powerhouse on 24 February.

    - Unit 1 remains out of service for turbine rehab.

    - Unit 2 was out of service from 0430 on 08 March until 0808 on 11 March for pre-rehab testing.

    - Unit 10 remains out of service.

    - Unit 11 remains out of service.

    - Unit 12 was out of service from 0916 until 1152 on 08 March for transducer installation.

    - Unit 15 was out of service from 0728 until 0912 on 08 March for transducer installation.

    - Unit 17 remains out of service for VBS modifications.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES.

  - AWS Closures. No AWS valves were closed this reporting week.

  - STS/VBS Inspections.  No STSs were installed this reporting week.

  - Dewatering and Fish Salvage.  The following dewatering or salvage activities this reporting week.

Navlock- the navlock floor was dewatered on 07-08 March.  With the help of Rangers and Field Unit, biologists were able to remove approximately 2000 subyearling chinook, 100 yearling chinook, three adult steelhead, one tadpole, one crappie, one smallmouth, 24 sculpin, 60 crawdads and about a dozen sticklebacks.  All fish were released downstream.  The lock still holds a couple hundred juveniles, but due to the size of the lock and the hidey holes in the sumps, these fish will be kept in a safety pool of water until the lock returns to service.

- RESEARCH.

  - Battelle FGE testing.  Hydroacoustic installation complete.

  - CRITFC.  Scheduled to start in mid-March.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - NOAA FISH lamprey.  Lamprey ladder installation in April.  Met on 10 March to discuss the ladder design and placement at Bradford Island.

  - NOAA FISH NaCl.  Maintaining the saltwater facility though no research is scheduled this season.

  - PSMFC SMF.  One adult steelhead was observed to slide over the separator bars this week.

  - PSMFC PIT tag.  Testing an antenna for the serpentine sections of the fishways.

  - Spring Creek Hatchery.  Released approximately 3.7 million fall chinook on 10 March.

  - USGS FPE testing.  Radio telemetry equipment to be installed on the Project throughout March.

  - WDFW Fish Counting.  16-hr video counts at Washington Shore and Bradford Island.

4.  FISHWAYS

  - Project biologists inspected 07-13 March this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 PCC | Velocity <1.5 fps | Meter in slowest part of the channel. | No action taken. This is a normal condition. |  |

  - Fish Ladder Temperatures.  .  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Fahrenheit.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 3/7 | 42.1 | 43.3 |
| 3/8 | 42.6 | 43.7 |
| 3/9 | 43.1 | 44.2 |
| 3/10 | 43.3 | 44.4 |
| 3/11 | 43.7 | 44.9 |
| 3/12 | 43.9 | 44.9 |
| 3/13 | 44.0 | 45.0 |

  - Sensor Calibration.  Calibration was checked on 12 March this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Fishway Status.

    - DSM2 airburst is run in manual every six hours.  Debris loading will be monitored.

    - DSM1 remains out of service.

    - PH1 ITS chaingates adjusted to Spring settings.

    - The B2CC was opened at 1513 on 11 March for the second Spring Creek release.  Per a teletype sent out on 08 March, the corner collector will be opened from 1500 on 11 March until 1500 on 15 March.

    - Navlock dewatered.

- CONSTRUCTION

  - PH 2 Corner Collector- complete.

  - PH 2 Corner Collector Hi-Q PIT tag detector- Prototype still planned for installation in 2004.

  - Unit 17 VBS mods.  From the hours between 1200-1800, after 1 March 2004 until completion, no work such as concrete saw cutting, bush and vibro-hammering may be done inside the gatewells of unit 17 at PH2.  Between 0000-1200 and 1800-2400 hours, the contractor may complete work inside the gatewells with the associated equipment and construction techniques dealing with concrete saw cutting, bush and vibro-hammering.

  - B2 Follow-on.  The contractors finished the airburst installation.  Due to problems with the I/O hardware, the system will be ran in manual every six hours (or more often if necessary) until repairs can be made.

- MISCELLANEOUS

  - Per a teletype sent on 31 December, Project tailwater has a hard constraint of 11.8’ and soft constraint of 12.1’ during all hours.  This constraint is for chum spawning habitat located below the dam.

  - Sea lions and harbor seals have been observed in the tailrace of both powerhouses and the spillway.  No action is being taken by the project to harass, harm, or otherwise annoy these animals.

  - The hydro cannons at the SMF are not operational at this time.  There is a break in the water lines.  The Project is working on repairs.  Please see memo sent out by Project Fisheries on 02 March.

  - The 30 day comment period for the Avian Predation Deterrent Program EA is open until 05 April.  You may find the EA at www.nww.usace.army.mil/reports/avian/default.htm

  - Per a teletype on 09 March, Bonneville forebay will have a hard constraint of 71.5’ – 72.5’ and a soft constraint of 71.5’ – 72.0’ between 0600-1700 on 22 March and from 0600 on 23 March until 1700 on 24 March.  This is to facilitate the installation of a dock at Drano Lake and for the cleaning of latch pockets for spill gate dogs at Bonneville Lock and Dam.

  - The Bonneville and The Dalles Pools closed for gillnet fishing at 1200 on 10 March.  Platform and hook and line will remain open for subsistence use only.

JAMES R. MAHAR, P.E.

Operations Manager

//S//