### CENWP-OP-B                                                                                                                                     20 March 2004

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 12 of 2004, which covers the period 14 through 20 March 2004.

- OPERATION SUMMARY.

  - Daily river flow averaged 129.7 to 170.8 kcfs.  Daily project tailwater elevation averaged 12.3’ to 15.1’ msl.  Daily project forebay elevation averaged 73.4’ to 75.7’ msl.  River temperature averaged 42 to 43° F.  Daily turbidity measured 4.0 to 6.4 on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are open .5’ during daylight hours.  Spill averaged 2.2 to 2.3 kcfs.

  - Unit operation.

    - Unit 1 remains out of service for turbine rehab.

    - Unit 10 remains out of service.

    - Unit 11 remains out of service.

    - Unit 17 remains out of service for VBS modifications.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES.

  - AWS Closures. No AWS valves were closed this reporting week.

  - STS/VBS Inspections.  No STSs were installed this reporting week.

  - Dewatering and Fish Salvage.  No dewatering or salvage activities this reporting week.

- RESEARCH.

  - Battelle FGE testing.  Hydroacoustic installation complete.

  - CRITFC.  Scheduled to start in mid-March.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - NOAA FISH lamprey.  Lamprey ladder installation in April.

  - PSMFC SMF.  One unknown salmonid was observed to slide over the separator bars this week.

  - PSMFC PIT tag.  Testing an antenna for the serpentine sections of the fishways.

  - USGS FPE testing.  Radio telemetry equipment to be installed on the Project throughout March.

  - University of Idaho.  Scheduled to start 29 March.

  - WDFW Fish Counting.  16-hr video counts at Washington Shore and Bradford Island.

4.  FISHWAYS

  - Project biologists inspected 14-20 March this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 PCC | Velocity <1.5 fps | Meter in slowest part of the channel. | This is a normal condition. |  |
| 3/16 | FG3-4 | Incorrect position for TW | Bent shaft | No action taken. |  |

  - Fish Ladder Temperatures.  .  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Fahrenheit.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 3/14 | 43.9 | 45.0 |
| 3/15 | 44.3 | 45.4 |
| 3/16 | 44.9 | 45.9 |
| 3/17 | 45.1 | 45.6 |
| 3/18 | 45.5 | 45.7 |
| 3/19 | 45.1 | 45.3 |
| 3/20 | 45.1 | 45.3 |

  - Sensor Calibration.  Calibration was checked on 18 March this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Fishway Status.

    - DSM2 airburst system switched to auto on 20 March.

    - DSM1 remains out of service.

    - PH1 ITS chaingates adjusted to Spring settings.

    - The B2CC was closed at 1513 on 15 March for the second Spring Creek release.  Per a teletype sent on 08 March, the corner collector will be opened from 1500 on 11 March until 1500 on 15 March.

    - Navlock watered back up on 1423 on 19 March.

- CONSTRUCTION

  - PH 2 Corner Collector Hi-Q PIT tag detector- The prototype antenna will be installed and tested in the pit above the channel this June and July.  If feasible the prototype antenna will then be constructed into the existing channel during the in-water work period in FY05.

  - Unit 17 VBS mods.  From the hours between 1200-1800, after 1 March 2004 until completion, no work such as concrete saw cutting, bush and vibro-hammering may be done inside the gatewells of unit 17 at PH2.  Between 0000-1200 and 1800-2400 hours, the contractor may complete work inside the gatewells with the associated equipment and construction techniques dealing with concrete saw cutting, bush and vibro-hammering.

  - B2 Follow-on.  The airburst system was repaired this week.  It can now be operated in auto.

  - Adult PIT tag exit detectors.  A meeting to introduce the Bonneville Exit Ladder Pit Tag Detection Design as been scheduled for 0900 on 23 March.

  - Head Gate Delivery.  Off loading of headgates at PH1 is scheduled for 14-23 June 2004.

- MISCELLANEOUS

  - Per a teletype sent on 31 December, Project tailwater has a hard constraint of 11.8’ and soft constraint of 12.1’ during all hours.  This constraint is for chum spawning habitat located below the dam.

  - Sea lions and harbor seals have been observed in the tailrace of both powerhouses and the spillway.  No action is being taken by the project to harass, harm, or otherwise annoy these animals.

  - The hydro cannons at the SMF are not operational at this time.  The Project is working on repairs.

  - The 30 day comment period for the Avian Predation Deterrent Program EA is open until 05 April.  You may find the EA at www.nww.usace.army.mil/reports/avian/default.htm

  - Per a teletype on 09 March, Bonneville forebay will have a hard constraint of 71.5’ – 72.5’ and a soft constraint of 71.5’ – 72.0’ between 0600-1700 on 22 March and from 0600 on 23 March until 1700 on 24 March.  This is to facilitate the installation of a dock at Drano Lake and for the cleaning of latch pockets for spill gate dogs at Bonneville Lock and Dam.

JAMES R. MAHAR, P.E.

Operations Manager

//S//