### CENWP-OP-B                                                                                                                                     29 March 2004

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 13 of 2004, which covers the period 21 through 27 March 2004.

- OPERATION SUMMARY.

  - Daily river flow averaged 136.4 to 173.9 kcfs.  Daily project tailwater elevation averaged 12.6’ to 15.4’ msl.  Daily project forebay elevation averaged 72.0’ to 75.2’ msl.  River temperature averaged 43 to 45° F.  Daily turbidity measured 4.5 to 6.0 on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B Branch respectively, are open .5’ during daylight hours.  Spill averaged 1.8 to 2.2 kcfs.  Spill bay 18 was on seal from 0630 until 1000 and bay 1 went on seal at 1430 on 23 March to facilitate latch pocket cleaning.

  - Unit operation.

    - Unit 1 remains out of service for turbine rehab.

    - Unit 7 was out of service from 0500 until 1722 on 22 March for semi-annual maintenance and from 1130 until 1405 on 25 March due to a breaker problem.

    - Unit 8 was out of service from 0500 until 1741 on 23 March for semi-annual maintenance.

    - Unit 9 was out of service from 0441 until 1652 on 24 March for semi-annual maintenance.

    - Unit 10 remains out of service.

    - Unit 11 remains out of service.

    - Unit 14 went out of service at 1000 on 22 March for wicket gate repairs.

    - Unit 17 remains out of service for VBS modifications.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES.

  - AWS Closures.  The following AWS valves were closed this reporting week.

FV6-9—closed from 0912 until 0930 on 24 March.

  - STS/VBS Inspections.  No STSs were inspected this reporting week.

  - Dewatering and Fish Salvage.  No dewatering or salvage activities this reporting week.

- RESEARCH.

  - Battelle FGE testing.  Hydroacoustic installation complete.

  - CRITFC.  Scheduled to start in mid-March.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - NOAA FISH lamprey.  Lamprey ladder installation in April.

  - PSMFC SMF.  Seven steelhead and one salmon slid over the separator bars this week.

  - PSMFC PIT tag.  Testing an antenna for the serpentine sections of the fishways.

  - USGS FPE testing.  Radio telemetry equipment to be installed on the Project throughout March.

  - University of Idaho.  Scheduled to start 29 March.

  - WDFW Fish Counting.  16-hr video counts at Washington Shore and Bradford Island.

4.  FISHWAYS

  - Project biologists inspected 21-27 March this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 PCC | Velocity <1.5 fps | Meter in slowest part of the channel. | This is a normal condition. |  |
| 3/22-24, 3/26-27 | FG3-4 | Incorrect position for TW | Bent shaft | No action taken. |  |
| 3/21-23 | DSM2 | Channel elevation < 64.2’ | Low forebay | Opened F2 manual orifices. |  |
| 3/21-23 | Unit 11 | Gatewell orifices in 11C were closed. | Oil sheen in gatewell. | Pads dropped in the gatewell. |  |

  - Fish Ladder Temperatures.  .  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Fahrenheit.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 3/21 | 45.4 | 45.7 |
| 3/22 | 45.8 | 46.0 |
| 3/23 | 46.5 | 46.8 |
| 3/24 | 47.0 | 47.4 |
| 3/25 | 47.1 | 47.3 |
| 3/26 | 47.0 | 47.2 |
| 3/27 | 46.9 | 47.3 |

  - Sensor Calibration.  Calibration was checked on 26 March this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Fishway Status.

    - DSM2 airburst system back in manual.  The computer has yet to be programmed.

    - DSM1 remains out of service.

    - PH1 ITS chaingates adjusted to Spring settings.

- CONSTRUCTION

  - PH 2 Corner Collector Hi-Q PIT tag detector- Prototype antenna construction started on Cascades Island.  If feasible the prototype antenna will then be constructed into the existing channel during the in-water work period in FY05.

  - Unit 17 VBS mods.  From the hours between 1200-1800, after 1 March 2004 until completion, no work such as concrete saw cutting, bush and vibro-hammering may be done inside the gatewells of unit 17 at PH2.  Between 0000-1200 and 1800-2400 hours, the contractor may complete work inside the gatewells with the associated equipment and construction techniques dealing with concrete saw cutting, bush and vibro-hammering.  VBS delivery currently scheduled for 09 April.

  - B2 Follow-on.  The airburst system remains in manual until the computer can be programmed in Auto.

  - Adult PIT tag exit detectors.  Current schedule is set for installation in both ladders for Winter 04/05.

  - Head Gate Delivery.  Off loading of headgates at PH1 is scheduled for 07 June 2004.

  - PH 1 Gantry Crane Delivery and Erection.  Scheduled for 9 August through 4 December.

- MISCELLANEOUS

  - Sea lions and harbor seals have been observed in the tailrace of both powerhouses and the spillway.  No action is being taken by the project to harass, harm, or otherwise annoy these animals.

  - The hydro cannons at the SMF are not operational at this time.  The Project is working on repairs.

  - The 30 day comment period for the Avian Predation Deterrent Program EA is open until 05 April.  You may find the EA at www.nww.usace.army.mil/reports/avian/default.htm

  - Per a teletype on 09 March, Bonneville forebay will have a hard constraint of 71.5’ – 72.5’ and a soft constraint of 71.5’ – 72.0’ between 0600-1700 on 22 March and from 0600 on 23 March until 1700 on 24 March.  This is to facilitate the installation of a dock at Drano Lake and for the cleaning of latch pockets for spill gate dogs at Bonneville Lock and Dam.

JAMES R. MAHAR, P.E.

Operations Manager

//S//