### CENWP-OP-B                                                                                                                                     14 April 2004

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 15 of 2004, which covers the period 04 through 10 April 2004.

- OPERATION SUMMARY.

  - Daily river flow averaged 137.7 to 158.5 kcfs.  Daily project tailwater elevation averaged 12.9’ to 14.5’ msl.  Daily project forebay elevation averaged 73.8’ to 75.8’ msl.  River temperature averaged 47 to 49° F.  Daily turbidity measured 3.9 to 7.0 on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B Branch respectively, are open .5’ during daylight hours.  Spill averaged 2.4 to 2.6 kcfs.

  - Unit operation.

    - Unit 1 remains out of service for turbine rehab.

    - Unit 2 was out of service from 0701 until 1002 on 05 April for transducer repairs.

    - Units 3-5 were out of service from 0813 until 1210 on 07 April to facilitate the placement of the USGS barge in the tailrace of PH1 for testing at Unit 4.

    - Unit 10 returned to service at 1000 on 08 April.  Unit 10 was out of service from 1119 until 1445 on 09 April due to a high water alarm.

    - Unit 11 was out of service from 0910 until 1215 on 09 April to facilitate Unit 17 VBS delivery.

    - Unit 12 was out of service from 0910 until 1215 on 09 April to facilitate Unit 17 VBS delivery.

    - Unit 13 was out of service from 0910 until 0939 on 09 April to facilitate Unit 17 VBS delivery.

    - Unit 14 was out of service from 0910 until 0939 on 09 April to facilitate Unit 17 VBS delivery.

    - Unit 15 went to manual 3D cam adjustment at 0933 on 05 April.  The cam was automatically adjusting and draining power which led to a governor lock up and complete loss of control.  New modules are on order.  Repairs should be made by the end of the month.

    - Unit 17 remains out of service for VBS modifications.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES.

  - AWS Closures.  No AWS valves were closed this reporting week.

  - STS/VBS Inspections.  No STSs were inspected this reporting week.

  - Dewatering and Fish Salvage.  The following dewatering or salvage activities occurred this reporting week.

Unit 10 Tail logs.  Biologists removed seven sculpin, three crayfish, seven juvenile salmonids and one salmon fry on 07 April.

Unit 17 Tail logs.  Biologists removed 11 sculpin, one juvenile salmon and five crawfish on 07 April.

- RESEARCH.

  - Battelle/BAE Systems FGE testing.  Hydroacoustic installation complete.  Data collection scheduled to begin 15 April.

  - Battelle/BAE Systems Gap Loss Testing.  Scheduled to begin 06 May.

  - CRITFC.  Sampling twice a week.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - ODFW Pikeminnow Tagging.  Electrofishing in Bonneville’s forebay and tailrace.

  - NOAA FISH lamprey.  Lamprey ladder installation in April.

  - PSMFC SMF.  One live kelt, one dead kelt and eight fallback steelhead slid over the separator bars this week.   Unusually high fish mortalities reported.  Please see misc. 6.2

  - PSMFC PIT tag.  Testing an antenna for the serpentine sections of the fishways.

  - Spring Creek.  A release of 3.9 million fall chinook scheduled for 08 April.

  - USGS FPE testing.  Installation of Radio telemetry equipment complete.  First tagged fish release scheduled for 29 April.

  - University of Idaho.  Sampling daily in the AFF starting 05 April.

  - Wildlife Services/Avian Abatement.  The 30 day comment period for the Avian Predation Deterrent Program EA is open until 11 April.  www.nww.usace.army.mil/reports/avian/default.htm

  - WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island.

4.  FISHWAYS

  - Project biologists inspected 05 through 10 April this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 PCC | Velocity <1.5 fps | Meter in slowest part of the channel. | This is a normal condition. |  |
| 4/5-6 4/8-9 | FG3-4 | Incorrect position for TW | Bent shaft | No action taken. |  |
| 4/4-7 | B-Branch | TW-Entrance differential <1.0’ | Computer not controlling. | Operators to adjust. |  |
| 4/5, 4/7, 4/9 | Weirs 37/38 | Weirs reading < > 1.0’ | Valve 2 in manual. | Mechanics making repairs to Valve 2 |  |
| 4/4 | Bonneville Fishways | No inspection by Fisheries | Illness |  |  |

  - Fish Ladder Temperatures.  .  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Fahrenheit.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 4/4 | 49.2 | 49.5 |
| 4/5 | 49.5 | 49.8 |
| 4/6 | 49.8 | 50.1 |
| 4/7 | 50.2 | 50.4 |
| 4/8 | 50.3 | 50.6 |
| 4/9 | 50.7 | 50.9 |
| 4/10 | 51.0 | 51.1 |

  - Sensor Calibration.  Calibration was checked on 09 April this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Fishway Status.

    - DSM1 remains out of service.

- CONSTRUCTION

  - PH 2 Corner Collector Hi-Q PIT tag detector- Prototype antenna construction started on Cascades Island.  If feasible, the prototype antenna will then be constructed into the existing channel during the in-water work period in FY05.

  - Unit 17 VBS mods.  VBS delivery scheduled for 09 April.

  - B2 Follow-on.  The airburst system has been programmed to run in auto.

  - SMF hydrocannons.  The hydro cannons at the SMF are not operational at this time.  The Project is working on repairs.

  - Adult PIT tag exit detectors.  Current schedule is set for installation in both ladders for Winter 04/05.

  - Head Gate Delivery.  Off loading of headgates at PH1 is scheduled for 07 June 2004.

  - PH 1 Gantry Crane Delivery and Erection.  Scheduled for 9 August through 4 December.

- MISCELLANEOUS AND OTHER NOTEWORTHY ITEMS

  - Sea lions and harbor seals have been observed in the tailrace of both powerhouses and the spillway.  No action is being taken by the project to harass, harm, or otherwise annoy these animals.

  - On 09 April PSMFC reported elevated numbers of juvenile morts in their samples.  Starting on 09 April and throughout the weekend, Project Biologists thoroughly inspected the DSM 2 gatewells, orifices and channel for blockages or signs of dead fish.  Nothing unusual was noted, no debris or dead fish could be found.  In recent years we have experienced a sudden increase in mortalities, usually later in the spring, but have yet to figure out where they are coming from.  The following numbers were reported by PSMFC.

09 April- four unclipped yearling chinook, nine clipped yearling chinook, and one unclipped steelhead morts were reported.

10 April- four unclipped yearling chinook, four clipped yearling chinook, one chinook fry, one unclipped coho and one clipped coho morts were reported.

JAMES R. MAHAR, P.E.

Operations Manager

//S//