### CENWP-OP-B                                                                                                                                     29 June 2004

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 26 of 2004, which covers the period 20 June through 26 June 2004.

- OPERATION SUMMARY.

  - Daily river flow averaged 192.4 to 245.5 kcfs.  Daily project tailwater elevation averaged 15.1’ to 18.0’ msl.  Daily project forebay elevation averaged 74.6’ to 76.2’ msl.  Daily river temperature averaged 60 to 63° F.  Daily turbidity measured 4.5 to 5.2 on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B Branch respectively, are part of the FPP spill pattern.  Spill averaged 50.3 to 99.2 kcfs.      BiOp versus 50kcfs testing started on 17 June.

  - Unit operation.

    - Unit 1 remains out of service for turbine rehab.

    - Unit 3 was out of service from 0500 on 21 June until 1633 on 22 June for annual maintenance.

    - Unit 4 was out of service from 0500 on 23 June until 0655 on 25 June for annual maintenance.

    - Unit 8 was out of service from 1012 until 1530 on 21 June and from 0636 until 1354 on 22 June to facilitate trashrack replacement.  Unit 8 returned to normal priority on 23 June.

    - Unit 11 was out of service from 0708 until 0953 on 25 June for B2CC PIT tag antenna delivery.

    - Unit 12 was out of service from 0716 until 0951 on 25 June for B2CC PIT tag antenna delivery.

    - Unit 13 was out of service from 0719 until 0946 on 25 June for B2CC PIT tag antenna delivery.

    - Unit 14 was out of service from 0724 until 0950 on 25 June for B2CC PIT tag antenna delivery.

    - Unit 15 remains on manual 3D cam adjustment.  New modules are on order.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

  - Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

- ACTIVITIES.

  - AWS Closures.  No AWS valves were closed for cleaning.

  - STS/VBS Inspections.  The following STSs were inspected this reporting week.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 4 | 6/24 |  | VBSs inspected and cleaned. |

Table 1.  STS Inspections.

  - Dewatering and Fish Salvage.  The following dewatering or salvage activities occurred this reporting week.

B2CC was dewatered on 25 June for PIT tag antenna delivery.  No fish were salvaged.  A report of the condition of the corner collector can be requested from Randy Lee in hydraulic design.

- RESEARCH.

  - Battelle/BAE Systems FGE testing.  Data collection continues.

  - CRITFC.  Sampling twice a week.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - ODFW Pikeminnow Tagging.  Electrofishing in Bonneville’s forebay and tailrace will continue on 28 June through 2 July.

  - NOAA FISH lamprey.  Lamprey ladder installation complete.

  - Pelfrey’s Sturgeon Hatchery.  Capturing breeding size sturgeon in Bonneville’s tailrace daily from 5/28 – 6/28.

  - PSMFC SMF.  Four steelhead were observed sliding over the separator bars this week.

  - USFWS.  Lower Columbia River Fish Health Center collecting shad from the AFF.

  - USGS FPE testing.  Releasing of radio tagged fish continues.   Last fish release scheduled for 31 July.  PSMFC collected 1,055 subyearling chinook for USGS RT testing.

  - University of Idaho.  Sampling daily in the AFF.

  - Wildlife Services/Avian Abatement.  Birds were hazed this reporting week with rockets, cracker shells, and 15mm pyrotechnics.  None were lethally taken.

  - WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island.

4.  FISHWAYS

  - Project biologists inspected 20 June through 26 June this reporting week.  FPC inspected on 24 June.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 PCC | Velocity <1.5 fps | Meter in slowest part of the channel | This is a normal condition. |  |
| All week | FG3-4 | Incorrect position for TW | Bent shaft | No action taken. |  |
| All week | PH2 PCC | Velocity < 1.5 fps | No acoustic sensor | Will repair during winter 04/05 |  |
| 6/24 | Weir 37 | Reading <1.3’> | Blocked drain. | Operators backflushed. |  |

  - Fish Ladder Temperatures.   Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Fahrenheit.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 6/20 | 63.1 | 63.4 |
| 6/21 | 63.7 | 63.8 |
| 6/22 | 64.2 | 64.4 |
| 6/23 | 64.4 | 64.6 |
| 6/24 | 65.0 | 65.1 |
| 6/25 | 65.3 | 65.4 |
| 6/26 | 65.4 | 65.6 |

  - Sensor Calibration.  Calibration was checked on 24 June this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Fishway Status.

    - Cascade Island briefly returned to auto but due to computer program troubles, is now in manual.

    - The Corner Collector was closed from 0813 until 1050 on 25 June to facilitate the PIT tag detector antenna delivery.

- CONSTRUCTION

  - PH 2 Corner Collector Hi-Q PIT tag detector- PIT antenna delivered at PH 2 forebay on 25 June.  If feasible, the prototype antenna will then be constructed into the existing channel during the in-water work period during FY05.

  - SMF hydrocannons.  The hydro cannons at the SMF are not operational at this time.  Repairs to the avian cannon are scheduled for completion by the end of June.  Wildlife services will conduct full time hazing activities until 17 July or until the hydro cannon is operational.

  - Adult PIT tag exit detectors.  Current schedule is set for installation in WA ladder for Winter 04/05.

  - PH 1 Gantry Crane Delivery and Erection.  Scheduled for 9 August through 4 December.

- MISCELLANEOUS AND OTHER NOTEWORTHY ITEMS

  - On 17 June, a teletype was sent changing Unit 8 to a last on, first off unit from 17-23 June.  This change is needed to facilitate the trashrack replacement scheduled for the week of 21 June.

  - On 17 June, a new spill teletype was sent, detailing the dates for BiOp (75kcfs) and 50kcfs spill.  The pre-test started on 17 June.  The testing period is from 20 June through 07 August.

JAMES R. MAHAR, P.E.

Operations Manager

//S/