### CENWP-OP-B                                                                                                                                     29 July 2004

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 30 of 2004, which covers the period 18 July through 24 July 2004.

- OPERATION SUMMARY.

  - Daily river flow averaged 136.4 to 151.5 kcfs.  Daily project tailwater elevation averaged 11.3’ to 12.3’ msl.  Daily project forebay elevation averaged 75.2’ to 76.1’ msl.  Daily river temperature averaged 68 to 69°F.  Daily turbidity measured 5.0 to 7.0 on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B Branch respectively, are part of the FPP spill pattern.  Spill averaged 50.2 to 80.3 kcfs.   BiOp versus 50kcfs testing started on 17 June.

  - Unit operation.

    - Unit 1 remains out of service for turbine rehab.

    - Unit 2 went out of service from 1052 on 19 July through 0929 on 23 July to store Unit 1 tail logs.

    - Unit 15 remains on manual 3D cam adjustment.  New modules are on order.

    - Unit 18 went out of service from 1353 on 19 July for 4-year overhaul.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

  - Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

- ACTIVITIES.

  - AWS Closures.  No AWS valves were closed this reporting week:

  - STS/VBS Inspections.    No inspections were conducted this week.

  - Dewatering and Fish Salvage.  The following dewatering or salvage activities this reporting week.  Unit 1 tail logs were removed on 19-22 July.  Nine sturgeon, one small mouth bass, 100 sculpin, three lamprey ammocoete, two catfish, and 107 crayfish were removed and released alive downstream.  Unit 18 draft tube was dewatered on 20 July.  20 sturgeon, 25 subyearling chinook, six lamprey, two catfish, and one northern pikeminnow were removed and released alive downstream.

- RESEARCH.

  - CRITFC.  Sampling once a week.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - NOAA FISH lamprey.  Lamprey ladder installation complete.

  - PSMFC SMF.  One salmon was observed sliding over the separator bars this week.

  - USGS FPE testing.  Releasing of radio tagged fish continues.   Last fish release scheduled for 31 July.  PSMFC collected 1,740 subyearling chinook for USGS RT testing.

  - University of Idaho.  Sampled four days last week in the AFF.  Began sampling one day per week on 27 July due to river temperatures exceeding 72.0° F.

4.  FISHWAYS

  - Project biologists inspected 18 July through 24 July this reporting week.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 PCC | Velocity <1.5 fps | Meter in slowest part of the channel | This is a normal condition. |  |
| All week | PH2 PCC | Velocity < 1.5 fps | No acoustic sensor | Will repair during winter 04/05 |  |

  - Fish Ladder Temperatures.   Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Fahrenheit.

| Date | Wash. Shore | Bradford Is. | ##### AFF exit |
| --- | --- | --- | --- |
| 7/18 | 70.7 | 70.9 | 70.7 |
| 7/19 | 70.6 | 70.9 | 70.6 |
| 7/20 | 70.5 | 70.7 | 70.7 |
| 7/21 | 70.9 | 71.0 | 70.8 |
| 7/22 | 71.1 | 71.2 | 71.1 |
| 7/23 | 71.4 | 71.4 | 71.4 |
| 7/24 | 71.8 | 72.0 | 71.9 |

  - Sensor Calibration.  Calibration was checked on 22 July this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - PH 2 Corner Collector Hi-Q PIT tag detector.  If feasible, the prototype antenna will be constructed into the existing channel during the in-water work period during FY05.

  - Adult PIT tag exit detectors.  Current schedule is set for installation in WA ladder for Winter 04/05.

  - PH 1 Gantry Crane Delivery and Erection.  Scheduled for 9 August through 4 December.

- MISCELLANEOUS AND OTHER NOTEWORTHY ITEMS

  - On 17 June, a new spill Teletype was sent, detailing the dates for BiOp (75kcfs) and 50kcfs spill.  The pre-test started on 17 June.  The testing period is from 20 June through 07 August.

JAMES R. MAHAR, P.E.

Operations Manager

//S/