### CENWP-OP-B                                                                                                                                  22 November 2004

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 47 of 2004, which covers the period from 14 November through 20 November 2004.

- OPERATION SUMMARY

  - Daily average river flows ranged from 117.7 to 134.5 kcfs.  Daily average project tailwater elevation ranged from 11.5’ to 12.4’ msl.  Daily average project forebay elevation ranged from 74.3’ to 76.1’ msl.  Daily average river temperature ranged from 52 to 53F.  Daily turbidity measured 7.0 on the secchi disk.

  - Spill Bays 1 and 18, which supply adult attraction flow for Cascades Island and B Branch respectively, are open during daylight hours (0500 until 1930).  Daily average spill ranged from 2.2 to 2.3 kcfs.

  - Unit operation.

    - Unit 0 remains out of service for a 5-year overhaul.

    - Unit 2 remains out of service for turbine rehab.

    - Unit 10 remains out of service for gantry crane three construction.

    - Unit 11 went out of service at 0359 on 15 November for head gate inspection and for unit overhaul.

    - Unit 16 was out of service from 1112 until 1645 on 15 November and 0850 until 1613 on 16 November for index testing.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

  - Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

- ACTIVITIES

  - AWS Closures.  The following AWS valves were closed for cleaning this reporting week.

    - FV 1-1- from 1456 until 1516 on 16 November and 1601 until 1614 on 18 November.

  - STS/VBS Inspections.  The following  STSs/VBSs were inspected this reporting week.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | 11/15 | 669 |  |

Table 1.  STS/VBS Inspections.

  - Dewatering and Fish Salvage.  No dewatering or salvage activities occurred this reporting week.

- RESEARCH

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - WDFW Chum.  Chum trapping and tagging starts 04 November through 26 November.

  - WDFW Fish Counting.  Video counting continues at Washington Shore and Bradford Island.

- FISHWAYS

  - Project biologists inspected 14-15, 17-20 November.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 PCC | Velocity <1.5 fps | Eddies. | Normal condition. |  |
| All week | PH2 PCC | Velocity < 1.5 fps | No acoustic sensor | Replace this winter |  |
| 11/14-17 | PH1 ITS | Chain gate 6C closed | crane barge delivery | Riggers opened. |  |
| 11/15, 11/18 | Weir 37 & 38 | > 1.0 | Unknown |  |  |
| 11/16 | Bonneville | No inspection by fish bio | Scheduling conflicts |  |  |
| 11/17 | FG3-4 | Closed, should be open. | Broken diffuser. | Repairs in 05/06 |  |
| 11/18 | NUE, NDE | Head diff >FPP criteria | Unknown |  |  |

Table 2.  FPP Out of Criteria items.

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Fahrenheit.

| ##### Date | Wash. Shore daily avg. | Bradford Is. daily avg. |
| --- | --- | --- |
| 11/14 | 52.8 | 52.9 |
| 11/15 | 52.6 | 52.8 |
| 11/16 | 52.8 | 52.9 |
| 11/17 | 52.6 | 52.8 |
| 11/18 | 52.2 | 52.5 |
| 11/19 | 51.9 | 52.0 |
| 11/20 | 51.5 | 51.6 |

Table 3.  Fishway Temperatures.

  - Sensor Calibration.  Calibration was checked on 20 November this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - Adult PIT tag exit detectors.  Washington Shore construction to start 29 November.

  - PH 1 Gantry Crane Delivery and Erection.  Construction continues.

  - North Monolith repairs.  Plans and specs not yet completed.

  - B2 Follow on.  Mods to the perf plate and the bypass flume to be completed in February 2005.

- FISHWAY STATUS

  - Adult Fishways.

    - Cascades Island remains in manual and maintaining criteria.

  - Juvenile Fishways.

    - DSM1 continues to run south with STSs and VBSs in Units 1 and 3.

    - The SMF is in bypass mode.

- HAZMAT, SPILLS AND CLEANUP

Nothing to report this week.

- MISCELLANEOUS AND OTHER NOTEWORTHY ITEMS

  - A teletype issued on 12 November requests the following tailwater elevations to facility a chum spawning study on 17-19 November.  The study requires ramping flows up and then down over a 10 hour period to determine how flow velocity affects chum behavior.

    - 17 November-

0500 hrs increase TW from 11.5’ to 12.7’

0700 hrs increase TW from 12.7’ to 13.9’

0900 hrs increase TW from 13.9’ to 15.1’

1100 hrs decrease TW from 15.1’ to 13.9’

1300 hrs decrease TW from 13.9’ to 12.7’

1500 hrs decrease TW from 12.7’ to 11.5’

    - 18 November through 19 November-

1600 hrs. increase TW from 11.5’ to 12.7’

1800 hrs. increase TW from 12.7’ to 13.9’

2000 hrs. increase TW from 13.9’ to 15.1’

2200 hrs. decrease TW from 15.1’ to 13.9’

2400 hrs. decrease TW from 13.9’ to 12.7’

0200 hrs. decrease TW from 12.7’ to 11.5’

  - A teletype issued on 19 November requests a repeat of the above tailwater elevations for 24 November and 26-27 November.  This will be the second of five day/night tests.

JAMES R. MAHAR, P.E.

Operations Manager

//S/