### CENWP-OP-B                                                                                                                                  10 January 2005

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 02 of 2005, which covers the period from 02 through 08 January 2005.

- OPERATION SUMMARY

  - Daily average river flows ranged from 132.1 to 166.7 kcfs.  Daily average project tailwater elevation ranged from 12.1’ to 14.7’ msl.  Daily average powerhouse forebay elevation ranged from 72.2’ to 74.7’ msl.  Daily average river temperature remained 44 to 42F.  Daily turbidity measured 7.0 on the secchi disk.

  - Spill Bay 1, which supplies adult attraction flow for Cascades Island, is open during daylight hours.   Daily average spill ranged from 1.0 to 1.1 kcfs.

  - Unit operation.

    - Unit 0 remains out of service for a 5-year overhaul.

    - Unit 2 remains out of service for turbine rehab.

    - Unit 10 remains out of service for gantry crane three construction.

    - Unit 11 remains out of service for unit overhaul.

    - Unit 18 was last on first off due to governor trouble until 07 January.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

    - Fish Unit 1 remains out of service for winter maintenance at the Washington Shore fishway.

    - Fish Unit 2 remains out of service for winter maintenance at the Washington Shore fishway.

  - Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

- ACTIVITIES

  - AWS Closures.  The following AWS valves activity occurred this reporting week.

    - FV1-1- Closed to float trash from 1823 on 02 January until 0445 on 03 January.

    - FV3-7- Closed to float trash from 1823 on 02 January until 0445 on 03 January.

    - FV3-9- Closed to float trash from 1823 on 02 January until 0445 on 03 January.  Closed for cleaning from 1600 until 1615 on 08 January.

    - FV 6-9- remains closed.

  - STS/VBS Inspections.  STSs/VBSs have been removed until February.

  - Dewatering and Fish Salvage.  No dewatering or salvage activities occurred this reporting week.

- RESEARCH

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - WDFW Fish Counting.  Video counting continues at Bradford Island and Cascades Island.

- FISHWAYS

  - Project biologists inspected 03-07 January.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 N PCC | Velocity <1.5 fps | Slow water | Normal condition. |  |
| 1/04, 1/07 | PH1 WG2 | Opening < 8’ | Unknown | Adjusted WG 2 | 05-10052 |
| 1/04 - 1/06 | FG3-4 | Incorrect position for TW | Bent shaft | No action taken. |  |
| 1/05 | FV 3-9 trashrack | Head diff >FPP criteria | Aquatic vegetation in the river. | Riggers raked trashracks. |  |
| 1/05 | A and B branch | Staff gauge < 1.0 | Aquatic vegetation in the river. | Riggers raked trashracks. |  |

Table 2.  FPP Out of Criteria items.

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are not recorded during the winter maintenance period.

  - Sensor Calibration.  Calibration was checked on 06 January this reporting week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - Adult PIT tag exit detectors.  Washington Shore construction continues.

  - PH 1 Gantry Crane Delivery and Erection.  Construction continues.

  - North Monolith repairs.  Plans and specs not yet completed.

  - B2 Follow on.  Mods to the perf plate and the bypass flume to be completed in February 2005.

- FISHWAY STATUS

  - Adult Fishways.

    - Bradford Island and Cascades Island fishway PLC remains in manual and maintaining criteria.   The Cascades Island exit is open.

    - Washington Shore fishway, AWS, and AFF remain dewatered.

  - Juvenile Fishways.

    - DSM 1 remains dewatered.

    - DSM2 remains dewatered.

- HAZMAT, SPILLS AND CLEANUP- Nothing to report this week.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype issued on 22 December switches PH2 unit priority such that Unit 18 is last on first off due to governor problems.

  - A teletype issued on 29 December requests forebay elevations be maintained between 71.5 – 72.5’ msl as a hard constraint from 0800 until 1700 on 04- 06 January.  This is to facilitate Stevenson boat ramp repairs.

  - A teletype issued on 07 January switches PH2 Unit 18 back to normal operating priority, first on and last off.

JAMES R. MAHAR, P.E.

Operations Manager

//S/