### CENWP-OP-B                                                                                                                                  20 March 2005

MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 12 of 2005, which covers the period from 13 March through 19 March 2005.

- OPERATION SUMMARY

  - Daily average river flows ranged from 122.7 to 139.4 kcfs.  Daily average project tailwater elevation ranged from 11.6’ to 12.7’ msl.  Daily average powerhouse forebay elevation ranged from 72.6’ to 75.5’ msl.  Daily average river temperature averaged 44 to 45F.  Daily turbidity measured 6.0 to 7.0 on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are open 0.5’ during daylight hours.   Daily average spill ranged from 1.2 to 1.5 kcfs.

  - Unit operation.

    - Unit 0 remains out of service for a 5-year overhaul.

    - Unit 1 was out of service from 1040 until 1245 on 16 March for transducer installation.

    - Unit 2 remains out of service for turbine rehab.

    - Unit 3 was out of service from 1245 until 1626 on 16 March for transducer installation.

    - Unit 4 was out of service from 0910 until 1110 on 17 March for transducer installation.

    - Unit 5 was out of service from 1110 until 1330 on 17 March for transducer installation.

    - Unit 6 was out of service from 1330 until 1612 on 17 March for transducer installation.

    - Unit 10 remains out of service for gantry crane three construction.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

  - Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

- ACTIVITIES

  - AWS Closures.  The following AWS valve activity occurred this reporting week.

    - FV1-1 - Closed for cleaning from 0735 until 0755 on 14 March.

    - FV3-9 - Closed for cleaning from 0720 until 0730 on 14 March.

  - STS/VBS Inspections.  No STSs/VBSs were inspected this reporting week.

  - Dewatering and Fish Salvage.  No dewatering or salvage activities occurred this reporting week.

- RESEARCH

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - Univ of Idaho.  All antennas installed/repaired.

  - NOAA Fish.  Prototype lamprey flume installed.  Installation of above water equipment will occur in April.

  - PSMFC.  Eight adult steelhead were observed crossing the separator bars this week.

  - USGS FPE and Survival.  Installing radio telemetry equipment.

  - Battelle FPE and FGE.  Installing hydroacoustic transducers.

  - WDFW Fish Counting.  Video counting continues at Bradford Island and Washington Shore.

- FISHWAYS

  - Project biologists inspected 13 through 19 March.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 N PCC | Velocity <1.5 fps | Slow water | Normal condition. |  |
| All week | PH2 SCCH | Velocity <1.5 fps | Improper installation | No action taken until S. monolith dewatered. |  |
| 3/13 | Bradford Is. weir | Too low | Forebay fluctuation | No response |  |
| 3/15 | FG6-14, 6-15 | Closed | Unknown | Operators adjusted |  |

Table 2.  FPP Out of Criteria items.

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island as well as at the junction of the UMT and the main ladder at Washington shore.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 3/13 | 44.5 | 44.8 |
| 3/14 | 44.5 | 44.8 |
| 3/15 | 44.8 | 45.0 |
| 3/16 | 44.9 | 45.1 |
| 3/17 | 45.1 | 45.2 |
| 3/18 | 45.2 | 45.4 |
| 3/19 | 45.2 | 45.4 |

  - Sensor Calibration.  Calibration was checked on 18 March.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - PH 1 Gantry Crane Delivery and Erection.  Construction continues.

  - B2 VBS modifications.  Gatewell modifications are scheduled to begin in Winter 05/06.

  - PH1 STS/VBS removal and TIES removal.  Six TIES and 23 PH1 STSs/VBSs will be removed in August.

- FISHWAY STATUS

  - Adult Fishways.

    - Bradford Island and Cascades Island fishway PLC are back in manual.  Electricians have determined the problems and are working on corrections.

    - A-branch diffusers have been reprogrammed to compensate for FG3-4 being out of service.  FG3-4 will remain out of service until the winter maintenance period 2005/06.

    - The AFF remains dewatered.

  - Juvenile Fishways.

    - DSM 1 remains dewatered.  Chain gates 1C, 3C, and 6C are at 71.5’ msl.

    - SMF booster pump is on order.

- HAZMAT, SPILLS AND CLEANUP – Nothing to report this week.

- TELETYPES AND OTHER NOTEWORTHY ITEMS.

  - A teletype issued on 11 March requests a hard forebay constraint between 71.5’ – 73.0’ msl, and a soft forebay constraint from 72.5’ – 73.0’ msl, from 0800 until 1700 each day 14-17 March.   This is to facilitate bumper log replacement at the upper navlock entrance.

  - Marine mammals have been observed in the tailraces of PH1, PH2, the Main Dam, the forebay, the Washington shore and Bradford Is. fishways.  No action is being taken by the project to harass, harm, or otherwise annoy these animals.

JAMES R. MAHAR, P.E.

Operations Manager

//S/