### CENWP-OP-B                                                                                                                                  17 April 2005

MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 16 of 2005, which covers the period from 10 through 16 April 2005.

- OPERATION SUMMARY

  - Daily average river flows ranged from 119.0 to 155.0 kcfs.  Daily average project tailwater elevation ranged from 11.9’ to 13.8’ msl.  Daily average powerhouse forebay elevation ranged from 74.8’ to 75.5’ msl.  Daily average river temperature ranged from 47 to 49F.  Daily turbidity measured 4.0 to 6.0 on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are open 0.5’ during daylight hours.  50 kcfs spill started at 1800 on 15 April.  See 8.2.   Daily average spill ranged from 1.5 to 50.2 kcfs.

  - Unit operation.

    - Unit 0 remains out of service for a 5-year overhaul.

    - Unit 2 remains out of service for turbine rehab.

    - Unit 10 remains out of service for gantry crane three construction.

    - Unit 15 was out of service from 0435 on 11 April until 1540 on 14 April for distribution valve work.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

  - Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

- ACTIVITIES

  - AWS Closures.  No AWS valves were closed this reporting week.

  - STS/VBS Inspections.   No STS/VBSs were inspected this reporting week.

  - Dewatering and Fish Salvage.  The following dewatering or salvage activities occurred this reporting week.

    - The upper section of Cascades Island was partially dewatered to facilitate the removal of fish trapped above the UMT picket leads.  Project Bios herded 8-12 adult steelhead and four juvenile steelhead from the upper ladder to below the picket leads on 13 April.  All fish moved volitionally or were encouraged to move when herded with a net.  Only two adults were found above the count window.

- RESEARCH

  - CRITFC.  Started sampling on 14 April.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - Fisheries Field Unit.  Monitoring and hazing of marine mammals entering the fishways.  See. 8.1

  - Univ of Idaho.  UI researchers are assisting with the marine mammals hazing.

  - ODFW Pikeminnow Tagging.  Electrofishing in Bonneville’s forebay and tailrace.

  - PSMFC.  Three steelhead kelts were removed from the separator bars (two released alive, one arrived dead).  15 adult steelhead and two unidentified salmonids were observed crossing the separator bars.

  - USGS FPE and Survival.  Installing radio telemetry equipment.

  - Battelle FPE and FGE.  Installing hydroacoustic transducers.

  - WDFW Fish Counting.  16-hr visual counting at Washington Shore and Bradford Island.

  - USDA Wildlife Services.   Hazing activities for gulls continues.

- FISHWAYS

  - Project biologists inspected 10 through 16 April.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 N PCC | Velocity <1.5 fps | Slow water | Normal condition. |  |
| All week | PH2 SCCH | Velocity <1.5 fps | Improper installation | No action taken until winter maintenance. |  |
| 4/10 | PH 2 gatwells | 11C, 15B, 16A debris cover >50% | Debris load in the river | Riggers cleaned first thing on 4/11. |  |
| 4/11-15 | Cascades Island | FG6-12 OOS | Bent shaft | FG6-11 to compensate see 6.1.2 |  |
| 4/13 | Cascades Island | FV5-9, UMT closed. | Fish rescue activities | All fishways back to normal by 1600. |  |
| 4/15-16 | DSM2 | 16C-N orifice light not working | Unknown | Operators notified |  |

Table 2.  FPP Out of Criteria items.

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island as well as at the junction of the UMT and the main ladder at Washington shore.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 4/10 | 47.6 | 47.6 |
| 4/11 | 47.9 | 48.0 |
| 4/12 | 48.1 | 48.2 |
| 4/13 | 48.4 | 48.5 |
| 4/14 | 48.7 | 48.8 |
| 4/15 | 48.9 | 49.1 |
| 4/16 | 49.0 | 49.5 |

Table 3.  Bonneville Fishway temperatures.

  - Sensor Calibration.  Calibration was checked on 16 April.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - PH 1 Gantry Crane Delivery and Erection.  Crane testing performed this reporting week.

  - B2 VBS modifications.  Gatewell modifications are scheduled to begin in Winter 05/06.

  - PH1 STS/VBS removal and TIES removal.  Six TIES and 23 PH1 STSs/VBSs will be removed in August.

- FISHWAY STATUS

  - Adult Fishways.

    - Bradford Island and Cascades Island fishway PLCs are in manual.

    - Cascades Island diffuser FG6-12 has a bent shaft and will remain out of service until winter 2006/07.  FG6-11 is used to compensate.

    - A-branch has been reprogrammed.  FG3-4 will remain out of service until winter 2005/06.

    - B2CC opened at 1500 on 15 April.  See 8.2.

  - Juvenile Fishways.

    - SMF booster pump is on order.

- HAZMAT, SPILLS AND CLEANUP – Nothing to report this week.

- TELETYPES AND OTHER NOTEWORTHY ITEMS.

  - Marine mammals have been observed in the tailrace, forebay, and fishways.   Fish Field Unit and Univ. of Idaho are conducting hazing activities and looking into hydrophone installation as well as exclusion devices to keep marine mammals from entering the fishways.

  - A new teletype was issued instructing Bonneville to increase spill to 50 kcfs at 1800 on 15 April, open B2CC 1500-1800 on 15 April, and maintain a minimum tailwater of 11.5’ until further notice.

JAMES R. MAHAR, P.E.

Operations Manager

//S/