### CENWP-OP-B                                                                                                                                  24 April 2005

MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 17 of 2005, which covers the period from 17 through 23 April 2005.

- OPERATION SUMMARY

  - Daily average river flows ranged from 126.0 to 175.6 kcfs.  Daily average project tailwater elevation ranged from 11.9’ to 14.8’ msl.  Daily average powerhouse forebay elevation ranged from 74.9’ to 76.0’ msl.  Daily average river temperature ranged from 49 to 52F.  Daily turbidity measured 4.5 to 6.5 on the secchi disk.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are open 0.5’ during daylight hours.  Daily average spill ranged from 50.3 to 76.6 kcfs.

  - Unit operation.

    - Unit 0 remains out of service due to governor trouble.

    - Unit 1 was operated outside the FPP priority from 2008 on 18 April until 1752 on 19 April due to unit testing.

    - Unit 2 remains out of service for turbine rehab.

    - Unit 9 went out of service at 0732 on 18 April until 0838 on 19 April to facilitate breaker work for Unit 2.  Unit 9 remains out of service due to gantry crane three testing.

    - Unit 10 remains out of service for gantry crane three construction.

    - Unit 12 was out of service from 1130 until 1545 on 20 April due to the PSS not working correctly.  Unit 12 was also operated out of FPP priority from 1553 until 1748 on 20 April for unit testing.

    - Unit 13 was out of service from 0730 until 0936 on 20 April for trashrack transducer replacement.

    - Unit 15 was out of service from 0928 until 1350 on 20 April due to governor trouble.  Unit 15 was also operated outside FPP priority from 1553 until 1748 on 20 April for Unit 12 testing.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

  - Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

- ACTIVITIES

  - AWS Closures.  No AWS valves were closed this reporting week.

  - STS/VBS Inspections.   No STS/VBSs were inspected this reporting week.

  - Dewatering and Fish Salvage.  No dewatering or salvage activities occurred this reporting week.

- RESEARCH

  - Battelle FPE and FGE.  Hydroacoustic installation complete.  Data collection began 15 April.

  - CRITFC.  Sampling continues twice a week through the summer.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - Fisheries Field Unit.  Monitoring and hazing of marine mammals entering the fishways.  See. 8.1

  - NOAA barge transportation study.  Barge arrival dates are tentatively scheduled for 30 April, 4, 14, and 20 May between 1930 and 0200 hours.  Tagged fish releases will occur 48 hours later.

  - ODFW Pikeminnow Tagging.  Electrofishing in Bonneville’s forebay and tailrace.

  - PSMFC.   Five adult steelhead were observed crossing the separator bars.

  - Univ of Idaho.  UI researchers are assisting with the marine mammal hazing.

  - USDA Wildlife Services.   Hazing activities for gulls continues.

  - USGS FPE and Survival.  Installation of radio telemetry equipment complete.

  - WDFW Fish Counting.  16-hr visual counting at Washington Shore and Bradford Island.

- FISHWAYS

  - Project biologists inspected 17 through 23 April.   FPC inspected on 20 April and NOAA Fish on 22 April.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 N PCC | Velocity <1.5 fps | Slow water | Normal condition. |  |
| All week | PH2 SCCH | Velocity <1.5 fps | Improper installation | No action taken until winter maintenance. |  |
| 4/18-19 | PH1 Unit 1 | First on/ last off. Out of priority | Unit testing starting at 1800 on 18 April. | Testing over at 1200 on 19 April. |  |
| 4/20 | PH2 | Units 12 and 15 out of FPP priority | Unit 12 testing | Back to normal priority by 1800 |  |

Table 2.  FPP Out of Criteria items.

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island as well as at the junction of the UMT and the main ladder at Washington shore.

| Date | Wash. Shore | Bradford Is. |
| --- | --- | --- |
| 4/17 | 49.4 | 49.2 |
| 4/18 | 49.7 | 49.7 |
| 4/19 | 50.1 | 50.2 |
| 4/20 | 50.5 | 50.5 |
| 4/21 | 50.9 | 51.1 |
| 4/22 | 51.5 | 51.4 |
| 4/23 | 51.6 | 51.8 |

Table 3.  Bonneville Fishway temperatures.

  - Sensor Calibration.  Calibration was checked on 21 April.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - PH 1 Gantry Crane Delivery and Erection.  Crane testing performed this reporting week.

  - B2 VBS modifications.  Gatewell modifications are scheduled to begin in Winter 05/06.

  - PH1 STS/VBS removal and TIES removal.  Six TIES and 23 PH1 STSs/VBSs will be removed in August.

- FISHWAY STATUS

  - Adult Fishways.

    - Bradford Island and Cascades Island fishway PLCs are in manual.

    - Cascades Island diffuser FG6-12 has a bent shaft and will remain out of service until winter 2006/07.  FG6-11 is used to compensate.

    - A-branch has been reprogrammed.  FG3-4 will remain out of service until winter 2005/06.

  - Juvenile Fishways.

    - SMF booster pump is still scheduled for installation.  The contractor is ordering materials.

- HAZMAT, SPILLS AND CLEANUP – Nothing to report this week.

- TELETYPES AND OTHER NOTEWORTHY ITEMS.

  - Marine mammals have been observed in the tailrace, forebay, and fishways.   Fish Field Unit and Univ. of Idaho are conducting hazing activities and looking into hydrophone installation as well as exclusion devices to keep marine mammals from entering the fishways.

  - A teletype issued on 19 April requests an increase in spill to 75 KCFS during the day and up to the 120% gas cap at night.  This request is effective from 1800 on 19 April until 31 August.

  - The 19 April teletype also requests a tailwater elevation minimum of 11.5’ msl and a soft constraint of 11.9’ be maintained until further notice.  The tailwater elevations are to protect emerging chum down stream of the dam.

JAMES R. MAHAR, P.E.

Operations Manager

//S/