### CENWP-OP-B                                                                                                                                  05 June 2005

MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 23 of 2005, which covers the period from 29 May through 04 June 2005.

- OPERATION SUMMARY

  - Daily average river flows ranged from 193.0 to 244.3 kcfs.  Daily average project tailwater elevation ranged from 16.5’ to 19.5’ msl.  Daily average powerhouse forebay elevation ranged from 73.6’ to 75.6’ msl.  Daily average river temperature ranged from 59 to 60F.  Secchi disk measurements ranged from 4.0 to 6.0 feet.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are included in the spill pattern.  Daily average spill ranged from 89.2 to 96.1 kcfs.

  - Unit operation.

    - Unit 0 remains out of service due to governor trouble.

    - Unit 2 remains out of service for turbine rehab.

    - Unit 9 was out of service from 0743 on 31 May until 1710 on 01 June for semi-annual maintenance.

    - Unit 10 remains out of service for gantry crane three construction.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

    - F1 was out of service from 0930 until 1422 on 31 May due to a gland water leak.

  - Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

- ACTIVITIES

  - AWS Closures.  The following AWS valves were closed this reporting week.

    - FV1-1 closed for cleaning from 1530 until 1540 on 02 June.

  - STS/VBS Inspections.   No STSs/VBSs were inspected this reporting week.

  - Dewatering and Fish Salvage.  No dewatering or salvage activities occurred this reporting week.

- RESEARCH

  - Battelle FPE and FGE.  Monitoring continues.

  - CRITFC.  Sampling continues twice a week through the summer.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - Fisheries Field Unit (FFU).  Monitoring of marine mammals concluded.  See. 8.1

  - NOAA Fish Lamprey.  The Bradford Island LPS in operation.

  - PSMFC.   Observed fallbacks: Eight steelhead, one salmon.  Fish for research included: 330 clipped yearling chinook and 30 coho for NMFS, 390 clipped yearling chinook and 605 clipped steelhead for USGS

  - Univ of Idaho.  Trapping in the AFF.

  - USDA Wildlife Services.   Hazing activities for gulls and sea lions continue.

  - USGS FPE and Survival.  Juvenile tracking continues.

  - USGS.  Marine mammal monitoring and observing concluded.

  - WDFW Fish Counting.  16-hr visual counting at Washington Shore and Bradford Island.

- FISHWAYS

  - Project biologists inspected 29 May through 04 June.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 N PCC | Velocity <1.5 fps | Slow water | Normal condition. |  |
| All week | PH2 SCCH | Velocity <1.5 fps | Improper install. | Repairs scheduled for 06/07 |  |
| All week | Cascades Is. | FG6-12 OOS | Bent shaft | See 6.1.2 |  |
| 5/30 | PH2 PCC | Diffuser 6 not in correct position. | Diffuser keeps closing. | Operators opened disconnect. |  |
| 5/30 | Cascades Is. | Entrance gate incorrect for TW | System in manual | Operators adjusted gates. |  |
| 5/30, 6/3-4 | Weir 37 | Reading >1.3’ | Shad stuck on the drain. | Operators cleaned screen. Riggers cleaned on 31 May. |  |
| 5/31 | B-branch | Entrance diff. > 2.0 | In manual | Operators adjusted |  |
| 5/31, 6/2 | B-Branch | Diffuser positions incorrect for TW. | In manual | Operators adjusted |  |
| 5/31 | Monoliths | Differential <1.5’ | F1 OOS | See 1.4.1 |  |
| 6/1, 6/3-4 | SA-24 Board | SUE bad indication | Unknown | Electricians to look at. |  |
| 6/2 | ITS | Chaingate 3 closed | DIDSON work | Back in criteria by 1055 |  |

Table 2.  FPP Out of Criteria items.

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island as well as at the junction of the UMT and the main ladder at Washington shore.   The AFF is measured inside the lab, at the fish return.

| Date | Wash. Shore Average | Bradford Is. Average | ##### AFF exit max |
| --- | --- | --- | --- |
| 5/29 | 59.5 | 59.4 | 59.4 |
| 5/30 | 59.6 | 59.6 | 59.7 |
| 5/31 | 59.9 | 59.8 | 59.8 |
| 6/1 | 60.1 | 60.0 | 59.9 |
| 6/2 | 60.1 | 60.0 | 59.9 |
| 6/3 | 60.1 | 60.1 | 60.0 |
| 6/4 | 60.2 | 60.2 | 60.1 |

Table 3.  Bonneville Fishway temperatures.

  - Sensor Calibration.  Calibration was checked on 02 June.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - B2CC PIT Tag Detector.  Installation is planned for winter 05/06.

  - B2 Fish Unit Trash rake.  Delivery and installation scheduled for late summer 2005.

  - B2 VBS modifications.  Gatewell modifications are scheduled to begin in winter 05/06.

  - Bradford Island Adult PIT Tag Detectors.  Installation scheduled for winter 05/06.

  - PH 1 Gantry Crane Delivery and Erection.  Crane testing continues this reporting week.

  - PH1 STS/VBS removal and TIES removal.  Six TIES and 23 PH1 STSs/VBSs will be removed in August.

  - SMF Full flow PIT Tag Detectors.  CH2M Hill has been contracted to write the letter report.

- FISHWAY STATUS

  - Adult Fishways.

    - Bradford Island and Cascades Island fishway PLCs are in manual.

    - Cascades Island diffuser FG6-12 has a bent shaft and will remain out of service until winter 2006/07.  FG6-11 is used to compensate.

    - A-branch has been reprogrammed.  FG3-4 will remain out of service until winter 2005/06.

    - PH1 NCCH sensor still OOS.  Channel elevation is now controlled from CCCH.

    - SLEDS were removed from all four entrances at 1500 on 02 June.

  - Juvenile Fishways.

    - SMF remains in the high outfall.

    - PH1 ITS chain gate 3 closed from 0825 until 1055 on 02 June to facilitate DIDSON camera work.

- HAZMAT, SPILLS AND CLEANUP – Nothing to report this week.

- TELETYPES AND OTHER NOTEWORTHY ITEMS.

  - Marine mammals appear to have moved out of the Bonneville tailrace this reporting week.   FFU, UI, and USGS concluded monitoring.  Hydrophones are in the Washington Shore fishway entrance and sea lion exclusion devices are built for the fishway entrances.  Testing will begin once permits are in place.

  - A teletype issued on 30 May revises the 120% gas cap to 130 kcfs.

JAMES R. MAHAR, P.E.

Operations Manager

//S/