### CENWP-OP-B                                                                                                                                  19 June 2005

MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 25 of 2005, which covers the period from 12 through 18 June 2005.

- OPERATION SUMMARY

  - Daily average river flows ranged from 175.9 to 191.0 kcfs.  Daily average project tailwater elevation ranged from 15.1’ to 16.3’ msl.  Daily average powerhouse forebay elevation ranged from 73.7’ to 74.8’ msl.  Daily average river temperature ranged from 61 to 62F.  Secchi disk measurements ranged from 5.0 to 7.0 feet.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are included in the spill pattern.  Daily average spill ranged from 90.5 to 95.6 kcfs.

  - Unit operation.

    - Unit 0 remains out of service due to governor trouble.

    - Unit 2 remains out of service for turbine rehab.

    - Unit 10 remains out of service for gantry crane three construction.

    - Unit 7 was out of service from 0408 on 13 June until 1456 on 16 June for semi annual maintenance.

    - Unit 8 was out of service from 0408 on 13 June until 1630 on 15 June for semi annual maintenance.

    - There was a 1% violation at 1400 on 15 June to provide emergency assistance to BPA.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

  - Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

- ACTIVITIES

  - AWS Closures.  The following AWS valves were closed this reporting week.

    - FV1-1 closed for cleaning from 1509 until 1520 on 16 June.

    - FV3-9 closed for cleaning from 1536 until 1537 on 16 June.

    - FV5-9 closed for cleaning from 1500 until 1505 on 16 June.

    - Weir 37 closed for cleaning from 1030 until 110 on 16 June.

  - STS/VBS Inspections.   No STSs/VBSs were inspected this reporting week.

  - Dewatering and Fish Salvage.  No dewatering or salvage activities occurred this reporting week.

- RESEARCH

  - Battelle FPE and FGE.  Monitoring continues.

  - CRITFC.  Sampling continues twice a week through the summer.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - NOAA Fish Lamprey.   The NDE ramp and trap is schedule for installation on 22 June.

  - PSMFC.   Observed fallbacks:  One steelhead.  Research fish: 335 subyearling chinook for USGS.  262 subyearling chinook for NMFS.

  - Univ of Idaho.  Trapping in the AFF.  See 6.1.5 for SLED install/removal.

  - USDA Wildlife Services.   Hazing activities for gulls continue.

  - USGS FPE and Survival.  Juvenile tracking continues.

  - WDFW Fish Counting.  16-hr visual counting at Washington Shore and Bradford Island.

- FISHWAYS

  - Project biologists inspected 12 June through 18 June.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 N PCC | Velocity <1.5 fps | Slow water | Normal condition. |  |
| All week | PH2 SCCH | Velocity <1.5 fps | Improper install. | Repairs scheduled for 06/07 |  |
| All week | Cascades Is. | FG6-12 OOS | Bent shaft | See 6.1.2 |  |
| 6/12, 6/15-16 | Weir 37 | Reading >1.3’ | Shad stuck on drain. | Operators notified/riggers cleaned screen |  |

Table 2.  FPP Out of Criteria items.

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island as well as at the junction of the UMT and the main ladder at Washington shore.   The AFF is measured inside the lab, at the fish return.

| ##### Date | Wash. Shore Average | Bradford Is. Average | ##### AFF exit max |
| --- | --- | --- | --- |
| 6/12 | 61.1 | 61.3 | 61.1 |
| 6/13 | 61.4 | 61.3 | 61.3 |
| 6/14 | 61.5 | 61.5 | 61.5 |
| 6/15 | 61.9 | 61.9 | 61.9 |
| 6/16 | 62.4 | 62.4 | 62.4 |
| 6/17 | 62.4 | 62.5 | 62.5 |
| 6/18 | 62.8 | 62.8 | 62.9 |

Table 3.  Bonneville Fishway temperatures.

  - Sensor Calibration.  Calibration was checked on 16 June.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - B2CC PIT Tag Detector.  Installation is planned for winter 05/06.

  - B2 Fish Unit Trash rake.  Delivery and installation scheduled for late summer 2005.

  - B2 VBS modifications.  Gatewell modifications are scheduled to begin in winter 05/06.

  - Bradford Island Adult PIT Tag Detectors.  Installation scheduled for winter 05/06.

  - PH 1 Gantry Crane Delivery and Erection.  Nothing new to report.

  - PH1 STS/VBS removal and TIES removal.  Six TIES and 23 PH1 STSs/VBSs will be removed in August.

  - SMF Full flow PIT Tag Detectors.  CH2M Hill has been contracted to write the letter report.

- FISHWAY STATUS

  - Adult Fishways.

    - Bradford Island and Cascades Island fishway PLCs are in manual.  Parts on order.

    - Cascades Island diffuser FG6-12 has a bent shaft and will remain out of service until winter 2006/07.  FG6-11 is used to compensate.

    - A-branch has been reprogrammed.  FG3-4 will remain out of service until winter 2005/06.

    - PH1 NCCH sensor still OOS.  Channel elevation is now controlled from CCCH.

    - SLEDS were installed at all four PH2 entrances at 1950 on 12 June.  SLEDS were removed from all entrances at 1953 on 16 June.

  - Juvenile Fishways.

- HAZMAT, SPILLS AND CLEANUP – Nothing to report this week.

- TELETYPES AND OTHER NOTEWORTHY ITEMS.

  - A teletype issued on 08 June requests a low forebay from 0800 until 1000 on 14 June.  Forebay elevation should hold between 72.5’ to 73.5’ msl, soft constraint, to allow environmental sampling at Hood River.

  - Ranger and Project Fisheries hosted a Bass Lake Restoration meeting at Bonneville on 15 June.  Interested parties were invited to tour the lake and discuss restoration goals.

JAMES R. MAHAR, P.E.

Operations Manager

//S/