### CENWP-OP-B                                                                                                                                  24 July 2005

MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 30 of 2005, which covers the period from 17 through 23 July 2005.

- OPERATION SUMMARY

  - Daily average river flows ranged from 157.3 to 190.8 kcfs.  Daily average project tailwater elevation ranged from 14.3’ to 16.3’ msl.  Daily average powerhouse forebay elevation ranged from 74.8’ to 75.4’ msl.  Daily average river temperature ranged from 69 to 70F.  Secchi disk measurements ranged from 5.0 to 7.0 feet.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are included in the spill pattern.  Daily average spill ranged from 83.8 to 96.0 kcfs.

  - Unit operation.

    - Unit 0 remains out of service due to governor trouble.

    - Unit 1 was out of service from 0405 on 18 July until 1731 on 21 July for slip ring turning.

    - Unit 2 remains out of service for turbine rehab.

    - Unit 3 was out of service from 0332 on 19 July until 1731 on 21 July due to gantry crane rail work.

    - Unit 4 was run out of priority from 0425 until 0735 on 18 July to flush fish from the draft tube.  Unit 4 went out of service at 0735 on 18 July for a five-year overhaul and turbine obstruction removal.

    - Units 5-8 were out of service from 0345 on 19 July until 1731 on 21 July due to gantry crane rail work.

    - Unit 9 was out of service from 0406 on 19 July until 1731 on 21 July due to gantry crane rail work.

    - Unit 10 remains out of service for gantry crane three construction.

    - Unit 13 went out of service at 0308 on 18 July for headgate HSS inspections.

    - Units 15-18 were out of service from 1615 until 1852 on 17 July due to a transformer outage.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

    - F2 was out of service from 1615 until 1849 on 17 July due to a transformer outage.

  - Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

- ACTIVITIES

  - AWS Closures.  The following AWS valves were closed this reporting week

    - FV1-1- closed for cleaning from 1244 until 1324 on 21 July.

    - FV3-9- closed for cleaning from 1610 until 1620 on 18 July and from 1244 until 1324 on 21 July.

  - STS/VBS Inspections.   No STSs/VBSs were inspected this reporting week.

  - Dewatering and Fish Salvage.  The following dewatering or salvage activities occurred this reporting week.

    - On 19 July, biologists removed two dead sturgeon (24” each), two live sturgeon (24-36” each), two adult lamprey, two clipped juvenile steelhead, one channel catfish, and one sculpin from Unit 4 draft tube.  The unit is OOS for five-year overhaul and turbine obstruction removal.

    - On 20 July, biologists moved one adult unclipped sea-run cutthroat from the AFF brail pool exit area to the AFF exit ladder.  Please see 6.1.5

- RESEARCH

  - Battelle FPE and FGE.  Monitoring continues.

  - CRITFC.  Sampling is on hold until water temperatures drop below 70.0F.

  - CTUIR.  Received 259 lamprey to date from NOAA Fisheries.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - NOAA Fish Lamprey.   The NDE ramp installation occurred on 21 July.

  - PSMFC.   No kelts or fallback salmon was observed crossing the separator bars this week.  A female merganser was found on the separator bars this week.  SMF personnel released the bird to the wetland ponds.  Subyearling chinook research fish collection: 230 for USGS.  Summer collections for USGS concluded on 17 July.  NOAA Fisheries tagging was cancelled this reporting week due to water temperatures reaching 70.0F.

  - Univ of Idaho.  Salmon trapping concluded until 01 August.  Lamprey trapping and testing continues.

  - USDA Wildlife Services.   Hazing activities for gulls continue.

  - USGS FPE and Survival.  Juvenile tracking continues.   Tagging concluded on 17 July.

  - Warm Springs Tribes.  Received 49 lamprey to date from NOAA Fisheries.

  - WDFW Fish Counting.  16-hr visual counting at Washington Shore and Bradford Island.

- FISHWAYS

  - Project biologists inspected 17 July through 23 July.  NOAA Fisheries inspected on 22 July.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 N PCC | Velocity <1.5 fps | Slow water | Normal condition. |  |
| All week | PH2 SCCH | Velocity <1.5 fps | Improper install. | Repairs scheduled for 06/07 |  |
| All week | B-Branch | North entrance gate closed. | Separated from shaft | South gate open to compensate. See 6.1.4 |  |
| All week | Cascades Is. | FG6-12 OOS | Bent shaft | See 6.1.2 |  |
| 7/17-18 | DSM2 13B-S | Inoperable | Bolt damage on shaft | Mechanics repaired by 7/18. |  |
| 7/17-18 | B-Branch | Diffuser positions | Problem within the PLC system | Electricians corrected on 7/18 |  |
| 7/20 | Weir 37 | Reading <1.3> | Unknown | No action taken |  |
| 7/20 | S. Monolith | Differential <1.0 | Unknown | Operators adjusted the Fish Units |  |
| 7/22 | Weir 38 | Reading <1.3> | Unknown | No action taken |  |

Table 2.  FPP Out of Criteria items.

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island as well as at the junction of the UMT and the main ladder at Washington shore.   The AFF is measured inside the lab at the fish return.

| Date | WA Shore Average | Bradford Is Average | ##### AFF exit max |
| --- | --- | --- | --- |
| 7/17 | 69.5 | 69.5 | 69.5 |
| 7/18 | 70.0 | 70.1 | 70.0 |
| 7/19 | 70.0 | 70.1 | 70.2 |
| 7/20 | 69.8 | 69.9 | 68.9 |
| 7/21 | 70.1 | 70.1 | 70.2 |
| 7/22 | 70.2 | 70.4 | 70.5 |
| 7/23 | 70.5 | 70.4 | 70.6 |

Table 3.  Bonneville Fishway temperatures.

  - Sensor Calibration.  Calibration was checked on 23 July.  All sensors are within criteria.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - B2CC PIT Tag Detector.  Installation is planned for winter 05/06.

  - B2 Fish Unit Trash rake.  Delivery and installation scheduled for late summer 2005.

  - B2 VBS modifications.  Gatewell modifications are scheduled to begin in winter 05/06.

  - Bradford Island Adult PIT Tag Detectors.  Installation scheduled for winter 05/06.

  - PH 1 Gantry Crane Delivery and Erection.  Nothing new to report.

  - PH1 STS/VBS removal and TIES removal.  Six TIES and 23 PH1 STSs/VBSs will be scrapped in August.

  - SMF Full flow PIT Tag Detectors.  No further updates at this time.

- FISHWAY STATUS

  - Adult Fishways.

    - B-Branch fishway PLC is in manual.  Cascades Island PLC remains in auto.

    - Cascades Island diffuser FG6-12 has a bent shaft and will remain out of service until winter 2006/07.  FG6-11 is used to compensate.

    - A-branch has been reprogrammed.  FG3-4 will remain out of service until winter 2005/06.

    - B-Branch south entrance gate remains open to compensate for the north entrance gate.  Mechanics are waiting for a tailwater of 13’ or less before making repairs.

    - The AFF exit was closed from 0700 until 1530 on 20 July.  Water remained in the outside section of the exit ladder.  The inside section was dewatered to facilitate water level sensor repairs.

    - FV1-1 returned to auto operation on 21 July.

  - Juvenile Fishways.

    - Gatewell 13C was dewatered on 18 July to accommodate headgate HSS inspections.

- HAZMAT, SPILLS AND CLEANUP – Nothing to report this week.

- TELETYPES AND OTHER NOTEWORTHY ITEMS.

  - A teletype issued on 22 July requests Bonneville pool be operated at a hard constraint of 75.0’ – 76.5’ and a soft constraint of 75.5’- 76.5’ for treaty gillnet fishing.  This forebay elevation is effective from 0600 on 25 Jul 2005 until 1800 on 29 Jul 2005.

JAMES R. MAHAR, P.E.

Operations Manager

//S/