### CENWP-OP-B                                                                                                                                  08 August 2005

MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 32 of 2005, which covers the period from 31July through 06 August 2005.

- OPERATION SUMMARY

  - Daily average river flows ranged from 138.3 to 183.2 kcfs.  Daily average project tailwater elevation ranged from 12.4’ to 15.6’ msl.  Daily average powerhouse forebay elevation ranged from 74.8’ to 75.9’ msl.  Daily average river temperature was 72F.  Secchi disk measurements ranged from 6.0 to 7.0 feet.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are included in the spill pattern.  Daily average spill ranged from 80.1 to 90.4 kcfs.

  - Unit operation.

    - Unit 0 remains out of service due to governor trouble.

    - Unit 2 remains out of service for turbine rehab.

    - Unit 4 remains out of service for a five-year overhaul and turbine obstruction removal.

    - Unit 10 remains out of service for gantry crane three construction.

    - Unit 14 went out of service at 0340 on 01 August for HSS inspections.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

    - F1 was out of service from 0730 until 1714 on 02 August for PH2 collection channel ROV inspection and slip ring maintenance.

    - F2 was out of service from 0730 until 1720 on 02 August for PH2 collection channel ROV inspection and slip ring maintenance.

  - Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

- ACTIVITIES

  - AWS Closures.  The following AWS valves were closed this reporting week

    - FV1-1 - closed from 1655 until 1845 on 02 August for PH1 PCC ROV inspection.

    - FV3-7 - closed from 1655 until 1900 on 02 August for PH1 PCC ROV inspection.

    - FV3-9 - closed from 1603 until 1900 on 02 August for B-branch ROV inspection.

    - FV4-3 - closed from 1603 until 1705 on 02 August for B-branch ROV inspection.

    - FV4-4 - closed from 1603 until 1705 on 02 August for B-branch ROV inspection.

    - FV5-3 - closed from 1444 until 1552 on 02 August for Cascades Is. ROV inspection.

    - FV5-4 - closed from 1444 until 1552 on 02 August for Cascades Is. ROV inspection.

    - FV5-9 - closed from 1444 until 1552 on 02 August for Cascades Is. ROV inspection.

    - FV6-9 - closed from 0845 until 1545 on 02 August for PH2 PCC ROV inspection.

  - STS/VBS Inspections.   No STSs/VBSs were inspected this reporting week.

  - Dewatering and Fish Salvage.  On 01 August, biologists removed 12 subyearling chinook from Unit 14 gatewells and released them in to DSM2.

- RESEARCH

  - Battelle FPE and FGE.  Completed data collection on 16 July.  Powerhouse 1 transducers scheduled for removal during the week of 08 August.

  - CRITFC.  Sampling for chinook is on hold until water temperatures drop below 70.0F.  See 6.1.5

  - CTUIR.  Received 259 lamprey to date from NOAA Fisheries.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - NOAA Fish Lamprey.   Trapping and monitoring of the LPS continues.

  - PSMFC.   One fallback steelhead was observed crossing the separator bars this week.

  - Univ of Idaho.  Trapping and tagging on hold until water temperatures drop.  See 6.1.5

  - USDA Wildlife Services.   Completed full-time hazing activities for gulls on 22 July.

  - USGS FPE and Survival.  Last tagged fish released on 18 July.  Completed data collection on 4 August.

  - Warm Springs Tribes.  Received 49 lamprey to date from NOAA Fisheries.

  - WDFW Fish Counting.  16-hr visual counting at Washington Shore and Bradford Island.

- FISHWAYS

  - Project biologists inspected 31 July through 06 August.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 N PCC | Velocity <1.5 fps | Slow water | Normal condition. |  |
| All week | PH2 SCCH | Velocity <1.5 fps | Improper install. | Repairs scheduled for 06/07 |  |
| All week | B-Branch entrance | North entrance gate closed. | Separated from shaft | South gate open to compensate. See 6.1.4 |  |
| 7/31-8/2 | Cascades Is. | FG6-12 OOS | Bent shaft | See 6.1.2 |  |
| 7/31-8/2 | Weir 38 | Reading <1.3> | Unknown | No action taken |  |
| 8/2 | Weir 37 | Reading <1.3> | ROV inspection | No action taken |  |
| 8/2 | PH 2 collection channel | F1 and F2 OOS | ROV inspection / slip ring maintenance | In criteria by 1720 |  |
| 8/2 | B-Branch | FG3-25 open | PLC in manual | Operators adjusted. |  |

Table 2.  FPP Out of Criteria items.

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island as well as at the junction of the UMT and the main ladder at Washington shore.   The AFF is measured inside the lab at the fish return.

| Date | WA Shore Average | Bradford Is Average | ##### AFF exit max |
| --- | --- | --- | --- |
| 7/31 | 72.3 | 72.4 | 72.3 |
| 8/1 | 72.0 | 72.1 | 72.1 |
| 8/2 | 71.9 | 71.9 | 72.0 |
| 8/3 | 71.9 | 71.8 | 71.9 |
| 8/4 | 72.1 | 72.1 | 72.2 |
| 8/5 | 72.5 | 72.4 | 72.5 |
| 8/6 | 72.8 | 72.7 | 72.8 |

Table 3.  Bonneville Fishway temperatures.

  - Sensor Calibration.  Calibration was checked on 05 August.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - B2CC PIT Tag Detector.  Installation is planned for winter 05/06.

  - B2 Fish Unit Trash rake.  Delivery and installation scheduled for late summer 2005.

  - B2 VBS modifications.  Gatewell modifications are scheduled to begin in winter 05/06.

  - Bradford Island Adult PIT Tag Detectors.  Installation scheduled for winter 05/06.

  - PH 1 Gantry Crane Delivery and Erection.  Nothing new to report.

  - PH1 STS/VBS removal and TIES removal.  Six TIES and 23 PH1 STSs/VBSs will be scrapped in August.

  - SMF Full flow PIT Tag Detectors.  No new updates.

- FISHWAY STATUS

  - Adult Fishways.

    - B-Branch fishway PLC is in manual.  Cascades Island PLC in manual.

    - Cascades Island diffuser FG6-12 has a bent shaft and will remain out of service until winter 2006/07.  FG6-11 is used to compensate.

    - A-branch has been reprogrammed.  FG3-4 will remain out of service until winter 2005/06.

    - B-Branch south entrance gate remains open to compensate for the north entrance gate.  Mechanics are waiting for a tailwater of 13’ or less before making repairs.

    - The AFF is closed to trapping until temperatures drop to 71.5F or until CRITFC receives the paperwork for mandatory steelhead sampling.

  - Juvenile Fishways.

    - DSM2 screen sweeps exercised at 1129 on 31 July.

    - 14C-N and 14C-S orifice valves were closed at 0720 on 01 August for gatewell dipping and HSS inspections.

    - 14A-N and 14A-S orifice valves were closed from 0855 until 1005 on 01 August for 14 A, B, C STS removal.

- HAZMAT, SPILLS AND CLEANUP – Nothing to report this week.

- TELETYPES AND OTHER NOTEWORTHY ITEMS.

  - A new teletype was issued on 02 August lowering the gas cap to 130 kcfs.

JAMES R. MAHAR, P.E.

Operations Manager

//S/