### CENWP-OP-B                                                                                                                                  18 September 2005

MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 38 of 2005, which covers the period from 11 through 17 September 2005.

- OPERATION SUMMARY

  - Daily average river flows ranged from 75.3 to 91.9 kcfs.  Daily average project tailwater elevation ranged from 7.3’ to 9.1’ msl.  Daily average powerhouse forebay elevation ranged from 76.0’ to 76.2’ msl.  Daily average river temperature ranged from 67 to 69F.  Secchi disk measurements averaged 7.0 feet.

  - Daily average spill ranged from 1.3 to 1.6 kcfs.   Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are open 6” during daylight hours.

  - Unit operation.

    - Unit 0 remains out of service due to governor trouble.

    - Unit 1 was out of service from 1120 until 1425 on 15 September for STS installation.

    - Unit 2 remains out of service for turbine rehab.

    - Unit 3 was out of service from 0720 until 1116 on 15 September for STS installation.

    - Unit 6 remains out of service for a five-year overhaul.

    - Unit 10 remains out of service for gantry crane three construction.

    - Unit 11 remains out of service due to a possible rotor failure.

    - Unit 15 remains out of service for main unit circuit breaker replacement and annual maintenance.

    - Unit 16 remains out of service for main unit circuit breaker replacement and annual maintenance.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

  - Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

- ACTIVITIES

  - AWS Closures.  No AWS valves were closed this reporting week

  - STS/VBS Inspections.  No STSs/VBSs were inspected this reporting week.

  - Dewatering and Fish Salvage.  No dewatering or salvage activities occurred this reporting week.

- RESEARCH

  - CRITFC.  Chinook sampling resumed.  Steelhead sampling continues.  One chinook mort on 15 September.  See 8.2.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - NOAA Fish Lamprey.  Washington Shore and BI ramp continue to operate.

  - NOAA Fish PIT tag.  Tagging started 13 September.

  - PSMFC.  One fallback salmon was observed crossing the separator bars this week.

  - Univ of Idaho.  No restrictions on trapping in the AFF.

  - WDFW Fish Counting.  16-hr visual counting at Washington Shore and Bradford Island.

  - WDFW and ODFW.  Tule counting at Washington Shore and Bradford Island continues.

- FISHWAYS

  - Project biologists inspected 11 through 17 September.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 N PCC | Velocity <1.5 fps | Slow water | Normal condition. |  |
| All week | PH2 SCCH | Velocity <1.5 fps | Improper install. | Will repair in 06/07 |  |
| All week | B-Branch | North gate closed | Separated from shaft | See 6.1.4 |  |
| 9/11 | B-branch | Differential high. | System in manual | Operators adjusted valves. |  |
| 9/11-9/12 | Cascades Is. | North gate closed | Unknown | Operators corrected |  |
| 9/11, 9/13 | A-Branch | FG3-3 not in correct position for TW | Diffuser in manual, won’t move. | Operators TR’d |  |
| 9/13-9/15 | Weir 37/ 38 | Reading <1.0’> | Unknown | No action taken. |  |

Table 2.  FPP Out of Criteria items.

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island as well as at the junction of the UMT and the main ladder at Washington shore.  The AFF is measured inside the lab at the fish return.

| Date | WA Shore Average | Bradford Is Average | ##### AFF exit max |
| --- | --- | --- | --- |
| 9/11 | 68.7 | 68.4 | 68.6 |
| 9/12 | 68.4 | 68.3 | 68.4 |
| 9/13 | 68.4 | 68.1 | 68.2 |
| 9/14 | 68.1 | 68.0 | 68.0 |
| 9/15 | 67.5 | 67.3 | 67.4 |
| 9/16 | 67.0 | 66.8 | 66.9 |
| 9/17 | 66.7 | 66.5 | 66.6 |

Table 3.  Bonneville Fishway temperatures.

  - Sensor Calibration.  Calibration was checked on 17 September.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - B2CC PIT Tag Detector.  Installation is planned for winter 05/06.

  - B2 Fish Unit Trash rake.  Delivery and installation scheduled for late summer 2005.

  - B2 VBS modifications.  Gatewell modifications to begin in Unit 13 starting early November.

  - Bradford Island Adult PIT Tag Detectors.  Installation scheduled for winter 05/06.

  - Main Unit Circuit Breaker.  Mods started on 15 August with Units 15/16.

  - PH1 STS/VBS removal and TIES removal.  Six TIES and 23 PH1 STSs/VBSs will be scrapped.

  - SMF Full flow PIT Tag Detectors.  No new updates.

- FISHWAY STATUS

  - Adult Fishways.

    - B-Branch fishway PLC is in manual.  Cascades Island PLC is in manual.

    - Cascades Island diffuser FG6-12 has a bent shaft and remains out of service until winter 2006/07.  FG6-11 is used to compensate.

    - A-branch has been reprogrammed.  FG3-4 remains out of service until winter 2005/06.

    - B-Branch north gate has separated from the shaft.  The south gate compensates when possible.

    - The AFF is open for salmonid trapping without restriction.

  - Juvenile Fishways.

    - DSM2 screen sweeps are exercised on Sunday evenings.

    - DSM1 began flowing south on 15 September.  STSs and VBSs are installed in Units 1 and 3.

  - Upcoming Events.

    - The JMF will go to bypass at 0700 on 31 October.

    - The UMT is scheduled to go OOS (to accommodate VBS modification work) on 31 October.

    - Cascades Island is scheduled to open on 31 October.

- HAZMAT, SPILLS AND CLEANUP – Nothing to report this week.

- TELETYPES AND OTHER NOTEWORTHY ITEMS.

  - A 16 September teletype requests Bonneville maintain a forebay of 75.0-76.5’ msl from 0600 on 19 September until 1800 on 23 September.  This is to limit wear and tear on tribal gillnet equipment.

  - CRITFC reported a sample mortality that occurred around 1500 on 15 September.  Project Biologists examined the fish and took photos.  The fish appeared to be healthy with no discoloration of the gills or abnormal injuries.  A memo with pictures was sent to NOAA Fisheries, CRITFC, Corps, and FPC.

JAMES R. MAHAR, P.E.

Operations Manager

//S/