### CENWP-OP-B                                                                                                                                  08 November 2005

MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 45 of 2005, which covers the period from 30 October through 05 November 2005.

- OPERATION SUMMARY

  - Daily average river flows ranged from 91.8 to 124.8 kcfs.  Daily average project tailwater elevation ranged from 9.2’ to 12.7’ msl.  Daily average powerhouse forebay elevation ranged from 72.6’ to 75.3’ msl.  Daily average river temperature ranged from 57 to 60F.  Secchi disk measurements averaged 5.2 to 7.0 feet.

  - Daily average spill ranged from 1.0 to 1.1 kcfs.   Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are open 6” during daylight hours.

  - Unit operation.

    - Unit 0 remains out of service due to governor trouble.

    - Unit 2 remains out of service for turbine rehab.

    - Unit 4 has a 1% violation for testing.

    - Unit 6 remains out of service for a five-year overhaul.

    - Unit 10 remains out of service for gantry crane three construction.

    - Unit 11 remains out of service due to a rotor failure.

    - Unit 12 returned to service at 0839 on 04 November.

    - Unit 13 went out of service at 0717 on 01 November for VBS mods and a four-year overhaul.

    - Unit 14 went out of service at 0454 on 03 November for T-11 bus link installation and MUCB replacement.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

  - Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

- ACTIVITIES

  - AWS Closures. The following AWS valves were closed this reporting week

    - FV5-9- closed from 1720 on 01 November until 0915 on 02 November.  This closure was to reduce flow in the UMT prior to installation of the south bulkhead and dewatering.

  - STS/VBS Inspections.  The following STSs/VBSs were inspected this reporting week.

  - Dewatering and Fish Salvage.  The following dewatering or salvage activities occurred this reporting week.

    - 31 October- Unit 6 tail logs.  Biologists removed three sturgeon, three bluegill and 10 sculpin.

    - 02 November- UMT.  Biologists, CRITFC, BPA, PSMFC, and FFU removed one juvenile steelhead, one adult steelhead, one rainbow trout, on chinook jack, one carp, two northern pikeminnow, two peamouth and 50-75 juvenile shad.  All fish were released into the Washington Shore fishway.

    - 02 November- Unit 13 draft tube.  Biologists and PSMFC removed nine catfish and 12 sturgeon.  All fish were released at the Hamilton Island boat launch.

- RESEARCH

  - Battelle/Normandeau.  Sensor fish to be released at Unit 16 later this month.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - PSMFC.  One steelhead was observed crossing the separator bars this week.  Smolt monitoring concluded at 0700 on 31 October.

  - WDFW Fish Counting.  16-hr visual counting at Washington Shore, Bradford Island until 31 October.  16-hr video counting started on 31 October at Cascades Island and 01 November for Washington Shore and Bradford Island.

- FISHWAYS

  - Project biologists inspected 30 October through 02 November and 04-05 November.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH1 N PCC | Velocity <1.5 fps | Slow water | Normal condition. |  |
| All week | PH2 SCCH | Velocity <1.5 fps | Improper install. | Will repair in 06/07 |  |
| All week | B-Branch | North gate closed | Separated from shaft | See 6.1.4 |  |
| 10/30 | Weir 37 | Reading <1.0> | Unknown | Operators blow off valve. |  |
| 10/30 | Cascades Is. | Differential <1.0-2.0’> | System in manual | Operators adjusted FV5-3 and FV5-4 |  |
| 10/31 | B-Branch | FG3-27 position | System in manual | Operators adjusted. |  |
| 11/03 | All Fishways | No inspection by Project Fisheries. | Scheduling conflicts. | All other days were inspected. The operators inspected on 11/03 as well. |  |
| 10/04 | Calibration | Not checked | Scheduling conflicts. |  |  |

Table 2.  FPP Out of Criteria items.

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island as well as at the junction of the UMT and the main ladder at Washington shore.

| ##### Date | WA Shore Average | Bradford Is Average |
| --- | --- | --- |
| 10/30 | 59.9 | 59.7 |
| 10/31 | 59.4 | 59.3 |
| 11/01 | 59.3 | 59.1 |
| 11/02 | 58.7 | 58.7 |
| 11/03 | 58.3 | 58.3 |
| 11/04 | 57.8 | 57.8 |
| 11/05 | 57.6 | 57.1 |

Table 3.  Bonneville Fishway temperatures.

  - Sensor Calibration.  Calibration was not checked this reporting week due to scheduling conflicts.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - B2CC PIT Tag Detector.  Construction activities have begun.

  - B2 Fish Unit Trash rake.  Delivered on 31 October.

  - B2 VBS modifications.  Gatewell VBS slot modifications began in Unit 13 on 04 November.

  - Bradford Island Adult PIT Tag Detectors.  Installation scheduled for winter 05/06.

  - Main Unit Circuit Breaker.  Mods continue on Units 13/14.

  - PH1 STS/VBS removal and TIES removal.  Six TIES and 23 PH1 STSs/VBSs will be scrapped.

  - SMF Full flow PIT Tag Detectors.  Team meeting scheduled for 07 November.

- FISHWAY STATUS

  - Adult Fishways.

    - B-Branch fishway PLC is in manual.  Cascades Island PLC is in manual.

    - Cascades Island diffuser FG6-12 has a bent shaft and remains out of service until winter 2006/07.  FG6-11 is used to compensate.

    - A-branch has been reprogrammed.  FG3-4 remains out of service until winter 2005/06.

    - B-Branch north gate has separated from the shaft.  The south gate compensates when possible.

    - UMT went out of service at 0925 on 31 October and was dewatered on 02 November.

    - Cascades Island fishway exit was opened at 0925 on 31 October.

  - Juvenile Fishways.

    - DSM2 screen sweeps are exercised on Sunday evenings.

    - The JMF went to bypass at 0820 on 31 October.

  - Upcoming Events.

    - AFEP review at Walla Walla from 14-17 November.

    - Bradford Island dewatering on 05 December.

- HAZMAT, SPILLS AND CLEANUP.  Nothing to report

- TELETYPES AND OTHER NOTEWORTHY ITEMS.

  - A teletype on 03 November requests a forebay between 73.0 to 74.0’ msl from 0800 until 1500 on 07 November.  This request was made by Skamania Co. Noxious Weed Board to facilitate inspection of Drano Lake for future milfoil control treatment.

  - A teletype on 04 November requests the following tailwater adjustments to occur on 09 November.  This request is so researchers can test the hydrophones to be used in the chum spawning studies later this month.

0800 HRS. HOLD TW AT 11.5 FEET

1000 HRS. INCREASE TW FROM 11.5 FT TO 13.5 FEET

1200 HRS. INCREASE TW FROM 13.5 FT TO 15.5 FEET

1400 HRS. DECREASE TW FROM 15.5 FT TO 11.5 FEET

JAMES R. MAHAR, P.E.

Operations Manager

//S/