### CENWP-OP-B                                                                                                                                  03 January 2006

MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 53 of 2005, which covers the period from 25 through 31 December 2005.

- OPERATION SUMMARY

  - Daily average river flows ranged from 109.4 to 190.7 kcfs.  Daily average project tailwater elevation ranged from 11.9’ to 19.7’ msl.  Daily average powerhouse forebay elevation ranged from 73.1’ to 75.9’ msl.  Daily average river temperature ranged from 40 to 41F.  Secchi disk measurements averaged 3.0 to 7.0 feet.

  - Daily average spill averaged 0.5 to 0.6 kcfs.   Spill Bay 1, which supplies attraction flow for Cascades Island, is open 6” during daylight hours.

  - Unit operation.

    - Unit 0 remains out of service due to governor trouble.

    - Unit 1 remains out of service for sump painting.

    - Unit 2 remains out of service for turbine rehab.

    - Unit 10 remains out of service for turbine rehab.

    - Unit 11 remains out of service due to a rotor failure.

    - Unit 13 remains out of service for VBS mods and a four-year overhaul.

    - Unit 14 was out of service from 0835 until 1150 on 28 December for exciter work.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

- ACTIVITIES

  - AWS Closures.  The following AWS valves were closed this reporting week.

    - FV5-9- closed from 0840 until 0905 on 27 December for cleaning.

  - STS/VBS Inspections.  STSs/VBSs have been removed for the winter.

  - Dewatering and Fish Salvage.  No dewatering or salvage activities occurred this reporting week.

- RESEARCH

  - Battelle/Normandeau.  Sensor fish postponed until further notice.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - WDFW Fish Counting.  16-hr video counting at Cascades Island and Washington Shore.

- FISHWAYS

  - Project biologists inspected 27 through 29 December.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH2 SCCH | Velocity <1.5 fps | Improper install. | Will repair in 06/07 |  |
| 12/27 | Weir 37 | Reading <1.0> | Clogged trashrack | Operators adjusted. |  |

Table 2.  FPP Out of Criteria items.

  - Fish Ladder Temperatures.  Temperatures are not recorded during the winter maintenance period.

  - Sensor Calibration.  PH1 collection channel and B-Branch are out of service.  Cascades Island is out of calibration until sensors can be replaced and recalibrated.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - B2CC PIT Tag Detector.  Demolition and construction continue.

  - B2 VBS modifications.  Gatewell VBS slot modifications continue on Unit 13.

  - Bradford Island Adult PIT Tag Detectors.  Installation continues.

  - Main Unit Circuit Breaker.  Mods continue on Units 13.

  - PH1 STS/VBS removal and TIES removal.  Six TIES and 23 PH1 STSs/VBSs will be scrapped.

  - SMF Full flow PIT Tag Detectors.  Pipe survey occurred 20-22 December.

- FISHWAY STATUS

  - Adult Fishways.

    - Cascades Island PLC is in manual.

    - Cascades Island diffuser FG6-12 has a bent shaft and remains out of service until winter 2006/07.

    - Bradford Island fishway remains out of service until late February.

  - Juvenile Fishways.

    - DSM1 and will remain out of service until 15 September.

    - DSM2 will remain out of service until late February.

  - Upcoming Events.

    - PH2 PCC stab plates installed starting 05 January.

    - B-Branch going below tailwater the first or second week of January.

    - AFF entrance ladder dewatering on 09 January.

    - F2 going OOS on 09 January.

- HAZMAT, SPILLS AND CLEANUP.  Nothing to report

- TELETYPES AND OTHER NOTEWORTHY ITEMS.

  - A teletype issued on 31 December authorizes a minimum tailwater of 11.5’ for all hours, starting at 0000 on 01 January.  This operation is to provide chum incubation flows below Bonneville.

JAMES R. MAHAR, P.E.

Operations Manager

//S/