### CENWP-OP-B                                                                                                                                  09 January 2006

MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 01 of 2006, which covers the period from 01 through 07 January 2006.

- OPERATION SUMMARY

  - Daily average river flows ranged from 152.0 to 207.3 kcfs.  Daily average project tailwater elevation ranged from 21.1’ to 16.0’ msl.  Daily average powerhouse forebay elevation ranged from 73.2’ to 74.6’ msl.  Daily average river temperature ranged from 41 to 42F.  Secchi disk measurements averaged 1.0 to 2.0 feet.

  - Daily average spill averaged 0.5 to 0.6 kcfs.   Spill Bay 1, which supplies attraction flow for Cascades Island, is open 6” during daylight hours.

  - Unit operation.

    - Unit 0 remains out of service due to governor trouble.

    - Unit 1 remains out of service for sump painting.

    - Unit 2 remains out of service for turbine rehab.

    - Unit 10 remains out of service for turbine rehab.

    - Unit 11 remains out of service due to a rotor failure.

    - Unit 13 remains out of service for VBS mods and a four-year overhaul.

    - Unit 16 was out of service from 0743 until 0838 on 05 January for exciter diode repair work.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

- ACTIVITIES

  - AWS Closures.  The following AWS valves were closed this reporting week.

    - FV5-9- closed from 1557 until 1628 on 04 January for cleaning.

  - STS/VBS Inspections.  STSs/VBSs have been removed for the winter.

  - Dewatering and Fish Salvage.  No dewatering or salvage activities occurred this reporting week.

- RESEARCH

  - Battelle/Normandeau.  Sensor fish postponed until further notice.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - WDFW Fish Counting.  16-hr video counting at Cascades Island and Washington Shore.

- FISHWAYS

  - Project biologists inspected 03 through 05 January.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | PH2 SCCH | Velocity <1.5 fps | Improper install. | Will repair in 06/07 |  |
| 1/3 | Weir 37 | Reading <1.0> | Clogged trashrack | Operators adjusted. |  |
| 1/3 | CI fishway | South gate open | System in manual. | Operators closed gate. |  |
| 1/4 | CI fishway | FG6-10, FG6-12 not in proper position. | System in manual. FG6-12 OOS | Operators closed 6-10. |  |
| 1/5 | WS monoliths | Entrance head low | F2 OOS | Stab plates installed. Operators adjusted entrance gates. |  |

Table 2.  FPP Out of Criteria items.

  - Fish Ladder Temperatures.  Temperatures are not recorded during the winter maintenance period.

  - Sensor Calibration.  PH1 collection channel and B-Branch are out of service.  Cascades Island is out of calibration until sensors can be replaced and recalibrated.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - B1 ITS Wall Removal.  A letter report is expected in February 2006.

  - B2CC PIT Tag Detector.  Shield delivery expected the week of 16 January.

  - B2 VBS modifications.  Gatewell VBS slot modifications continue on Unit 13.  Moving to Unit 12 the week of 23 January.

  - Bradford Island Adult PIT Tag Detectors.  Installation continues.

  - Main Unit Circuit Breaker.  Mods continue on Units 13.

  - PH1 STS/VBS removal and TIES removal.  Six TIES and 23 PH1 STSs/VBSs will be scrapped.

  - SMF Full flow PIT Tag Detectors.  Nothing new to report.

- FISHWAY STATUS

  - Adult Fishways.

    - Cascades Island PLC is in manual.

    - Cascades Island diffuser FG6-12 has a bent shaft and remains out of service until winter 2006/07.

    - Bradford Island fishway remains out of service until late February.

    - As of 06 January, FG3-3 and FG3-4 are able to return to service when BI returns.

  - Juvenile Fishways.

    - DSM1 will remain out of service until 15 September.

    - DSM2 will remain out of service until late February.

  - Upcoming Events.

    - PH2 PCC stab plates installed starting 05 January.

    - B-Branch going below tailwater the first or second week of January.

    - AFF entrance ladder dewatering on 09 January.

    - F2 going OOS on 09 January.

- HAZMAT, SPILLS AND CLEANUP.  Nothing to report

- TELETYPES AND OTHER NOTEWORTHY ITEMS.

  - A teletype issued on 31 December authorizes a minimum tailwater of 11.5’ for all hours, starting at 0000 on 01 January.  This operation is to provide chum incubation flows below Bonneville.

JAMES R. MAHAR, P.E.

Operations Manager

//S/