### CENWP-OP-B                                                                                                                                  11 June 2006

MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 23 of 2006, which covers the period from 04 through 10 June 2006.

- OPERATION SUMMARY

  - Daily average river flows ranged from 335.0 to 372.6 kcfs.  Daily average project tailwater elevation ranged from 24.3’ to 26.2’ msl.  Daily average powerhouse forebay elevation ranged from 74.7’ to 75.8 msl.  Daily average river temperature ranged from 58 to 61F.  Secchi disk measurements averaged 1.4 to 3.0 feet.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are included in the spill pattern.  Daily average spill ranged from 109.5 to 149.5 kcfs.

  - Unit Operation.

    - Unit 0 remains out of service due to governor trouble.

    - Unit 1 was out of service from 0945 on 05 June until 1526 on 06 June for annual maintenance.

    - Unit 2 remains out of service for turbine rehab.

    - Unit 5 was out of service from 0440 on 07 June until 1330 on 08 June for annual maintenance.

    - Unit 10 remains out of service for turbine rehab.

    - Unit 11 remains out of service for bearing alignment.

    - Unit 12 was out of service from 0616 until 0938 on 05 June for STS repair.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays; VBS drawdowns are measured daily on modified units.

- ACTIVITIES

  - AWS Closures.  No AWS valves were closed this reporting week.

  - STS/VBS Inspections.   No STSs/VBSs were inspected this reporting week.

  - Dewatering and Fish Salvage.  No dewatering activities occurred this reporting week.

- RESEARCH

  - CRITFC.  Sampling adult fish in the AFF.

  - CTUIR.  Collecting adult lamprey for continuation of Umatilla Basin lamprey re-introductions.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - NOAA Fisheries.  Salt water rearing subyearling chinook at the SMF to begin 13 June.

  - PSMFC.  Four steelhead and one salmon were observed crossing the separator bars.

  - PSMFC PIT tag.  Monthly testing of the B2CC detectors starting 06 June through October.

  - U of I.  South Fork Salmon R. and shad sampling continues through late July.   Finished collecting approximately 100 shad for John McConnaughey on 06 June.

  - U of I tissue sampling.  On 06 June, 10 fish were collected and transported from the Little White Salmon hatchery to the AFF.  Lipid level samples and measurements were taken.   All remains were disposed of by U of I.  Please contact Chris Anderson with questions.

  - WDFW Fish Counting.  Visual fish counting continues.

- FISHWAYS

  - Project biologists inspected 04 through 10 June 2006.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| 6/5-7, 6/9-10 | Weir 37 | Reading >1.3’ | Blocked drain | Riggers cleaned. See 6.1.2 |  |
| 6/10 | DSM2 | Airburst screens 1 and 4 not bursting. | Unknown | Submitted TR | 06-40863 |

Table 2.  FPP Out of Criteria items.

    - PH1 electronic velocity meter is in an eddy.  Due to this placement, the velocity meter reads less than 1.5 fps.  Readings to meet FPP criteria are taken from the south end mechanical meter.

    - PH2 electronic velocity meter was incorrectly installed in the winter of 04/05.

    - Cascades Island FG6-12 is closed due to a bent shaft.  Repairs can not be made until winter 06/07.

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Diffuser failure occurred during water up.  Repairs are scheduled for winter 07/08.  For now, the collection channel is operating within FPP criteria.

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

| Date | WS avg. | BI avg. | AFF max | SMF max |
| --- | --- | --- | --- | --- |
| 6/04 | 58.6 | 58.7 | 58.5 | 5.8 |
| 6/05 | 59.6 | 59.6 | 59.4 | 58.7 |
| 6/06 | 60.1 | 60.1 | 59.9 | 59.3 |
| 6/07 | 60.5 | 60.5 | 60.4 | 59.7 |
| 6/08 | 60.7 | 60.6 | 60.7 | 59.8 |
| 6/09 | 60.6 | 60.5 | 60.5 | 59.7 |
| 6/10 | 61.2 | 61.1 | 61.0 | 60.2 |

Table 3.  Bonneville Fishway temperatures

  - Sensor Calibration.  Calibration was checked on 05 June.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - B1 ITS Wall Removal.  A letter report is expected in June 2006.

  - PH1 STS/VBS removal and TIES removal.  Six TIES and 23 PH1 STSs/VBSs will be scrapped.

  - SMF Full flow PIT Tag Detectors.  Electrical work will be performed on 12 June.  This will require powering down the PIT tag detectors and cutting power to the upper switchgate.  Care will be taken to isolate the switchgate to prevent it from going to bypass.  PSMFC has been notified of the PIT tag detector outage and will be available to reset the system when the power comes back.  This work is expected to take about a half day.

- FISHWAY STATUS

  - Adult Fishways.

    - Cascades Island PLC is in manual.

    - Due to the numbers of shad blocking the weir 37 drain, Fisheries has requested AFF users to leave the southern most picket lead up at all times.

    - SLEDS remain installed.

  - Juvenile Fishways.

    - DSM1 remains out of service until 15 September.

    - TIES remain installed.

- HAZMAT, SPILLS AND CLEANUP - Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS.

  - A teletype sent on 09 June requests flows not exceed 360 Kcfs from 1500 on 11 June until 0300 on 12 June.  This request is to control water levels in Portland Harbor to allow Rose Festival ships to pass under the steel bridge on the Willamette River.

  - Tim Darland’s last day with Bonneville Fisheries was 09 June.  He has taken a position with the Bonneville Park Rangers.  His phone number and email will remain the same and he is still the ANS coordinator for the Project and District.

JAMES R. MAHAR, P.E.

Operations Manager