### CENWP-OP-B                                                                                                                                  03 July 2006

MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 26 of 2006, which covers the period from 25 June through 01 July 2006.

- OPERATION SUMMARY

  - Daily average river flows ranged from 196.3 to 267.8 kcfs.  Daily average project tailwater elevation ranged from 16.7’ to 20.4’ msl.  Daily average powerhouse forebay elevation ranged from 74.7’ to 76.2 msl.  Daily average river temperature ranged between 64 and 66F.  Secchi disk measurements averaged 4.0 to 5.0 feet.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are included in the spill pattern.  Daily average spill ranged from 63.8 to 89.2 kcfs.

  - Unit Operation.

    - Unit 0 remains out of service due to governor trouble.

    - Unit 1 was out of service from 0440 to 1518 on 26 June for Kaplan packing.

    - Unit 2 remains out of service for turbine rehab.

    - Unit 4 was out of service from 0430 on 27 June to 1154 on 28 June for annual maintenance.

    - Unit 10 remains out of service for turbine rehab.

    - Unit 14 had a 1% violation on 27 June due to a STS inspection.

    - Unit 15 had a 1% violation on 27 June due to a STS inspection.

    - Unit 15 was out of service from 0719 to 0840 on 27 June for breaker work.

    - Unit 16 had a 1% violation on 27 June due to a STS inspection.

    - Unit 18 had a 1% violation on 27 June due to a STS inspection.

    - Unit 18 was out of service from 0644 to 0950 on 29 June for STS repairs.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays; VBS drawdowns are measured daily on modified units.

- ACTIVITIES

  - AWS Closures.  The following AWS valves were closed this reporting week.

    - FV3-9 was closed for cleaning from 0755 until 0830 on 26 June.

    - FV6-9 was closed for cleaning from 1313 until 1318 on 28 June.

  - STS/VBS Inspections.  The following STSs/VBSs were inspected this reporting week.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | 6/29 | 237 |  |
| 12 | 6/29 | 662 |  |
| 13 | 6/29 | 664 |  |
| 14 | 6/29 | 667 |  |
| 15 | 6/29 | 665 |  |
| 16 | 6/29 | 480 |  |
| 17 | 6/29 | 666 |  |
| 18 | 6/29 | 662 | Repaired 48” x 20” rip in A slot screen |

  - Dewatering and Fish Salvage.  The following dewatering activities occurred this reporting week.

    - On 26 June, one adult chinook was removed from the DSM2 add-in.  See 6.2.4

- RESEARCH

  - CRITFC.  Sampling adult fish in the AFF.

  - CTUIR.  Collecting adult lamprey for continuation of Umatilla Basin lamprey re-introductions.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - NOAA Fisheries.  Salt water rearing subyearling chinook at the SMF continues.

  - NOAA Fisheries lamprey.  Fish collection and monitoring continues.

  - PSMFC smolt monitoring.  One salmon was observed crossing the separator bars.   357 subyearling chinook were collected for NOAA researchers.

  - PSMFC PIT tag.  Monthly testing of the B2CC detectors June through October.

  - U of I adults.  South Fork Salmon R. and shad sampling continues through late July.

  - U of I lamprey.  Collection and testing continue.

  - WDFW Fish Counting.  Visual fish counting continues.

- FISHWAYS

  - Project biologists inspected 25 through 29 June 2006.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| All week | DSM2 airburst | Screens 1, 4-5, 11, 12 not working properly. | Electrical | See 6.2.3 | 06-40863 |
|  | Weir 37 | Reading >1.3’ | Blocked drain | See 6.1.2 |  |
| 6/26 | DSM2 | Orifice lights off | Contractor clearance | See 6.2.5 |  |
| 6/25-26 | DSM2 add-in | Fish in the add in | Unknown | See 6.2.4 |  |

Table 2.  FPP Out of Criteria items.

    - PH1 electronic velocity meter is in an eddy.  Due to this placement, the velocity meter reads less than 1.5 fps.  Readings to meet FPP criteria are taken from the south end mechanical meter.

    - PH2 electronic velocity meter was incorrectly installed in the winter of 04/05.

    - Cascades Island FG6-12 is closed due to a bent shaft.  Repairs can not be made until winter 06/07.

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Diffuser failure occurred during water up.  Repairs are scheduled for winter 07/08.  For now, the collection channel is operating within FPP criteria.

  - Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

| Date | WS avg. | BI avg. | AFF max | SMF max |
| --- | --- | --- | --- | --- |
| 6/25 | 64.2 | 64.2 | 64.3 | - |
| 6/26 | 65.0 | 64.9 | 64.9 | - |
| 6/27 | 65.5 | 65.5 | 65.5 | - |
| 6/28 | 65.6 | 65.5 | 65.5 | - |
| 6/29 | 65.5 | 65.5 | 65.5 | - |
| 6/30 | 66.1 | 66.0 | 66.1 | - |
| 7/01 | 66.7 | 66.6 | 66.6 | - |

Table 3.  Bonneville Fishway temperatures

- The temperature probe was found outside the holding tank at the SMF this week.  The data has been excluded due to it being inaccurate.

  - Sensor Calibration.  Calibration was not checked this week.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

- CONSTRUCTION

  - B1 ITS Wall Removal.  A letter report is expected in June 2006.

  - PH1 STS/VBS removal and TIES removal.  Six TIES and 23 PH1 STSs/VBSs will be scrapped.

  - SMF Full flow PIT Tag Detectors.  Fiber optic testing occurred on 23 June.

- FISHWAY STATUS

  - Adult Fishways.

    - Cascades Island PLC is in manual.

    - Due to the numbers of shad blocking the weir 37 drain, the southern most picket lead will be up from 1330 until 1530.  UI and CRITFC will sample from 0830 until 1330 and from 1530 until 2030.

    - The B-branch north entrance gate closed at 1300 on 1 July and cannot be reopened.  The south gate is being used until the repair can be made.

  - Juvenile Fishways.

    - DSM1 remains out of service until 15 September.

    - TIES remain installed.

    - DSM2 airburst screens 1, 4, 5, 11 and 12 do not operate consistently.  The Project is waiting on contractors before repairs may be completed.

    - There are adult fish in the DSM2 add-in water.  DSM2 Add-in and Unit 11 orifices were closed from 1423 until 1501 on 26 June to facilitate the removal of these trapped adult salmonids.  One unclipped adult Chinook was removed and released.

    - On 26 June, a clearance for the B2 HVAC Contractor cut power to Units 17 and 18 DSM2 orifice lights and the lamprey PIT tag detectors.  This outage lasted from 1300 on 26 June until 0130 on 27 June.

    - The orifice valves of Unit 18 A slot were closed from 0648 until 0950 for STS repairs.

- HAZMAT, SPILLS AND CLEANUP - Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS.

  - A teletype sent on 26 June requests Bonneville to spill 100 KCFS or to the gas cap, whichever is less, 24 hours a day for juvenile fish passage.

  - A teletype sent on 28 June requests that Bonneville start summer spill of 75 KCFS during daytime hours and spill to the 120% gas cap at night until further notice.

  - A teletype sent on 29 June revised the projects’ voluntary and involuntary spill priority list and provided spill caps for various TDG levels by project.

  - A teletype sent on 30 June requests a soft constraint of 75’ to 76.5’ on the Bonneville pool from 0600, 3 July to 0600, 5 July.  A hard constraint of 75’ to 76.5’ and a soft constraint of 75.5’ to 76.5’ was requested for 0600, 5 July to 1800, 7 July.   These requests were to limit wear and tear on gill nets due to pool fluctuations.

JAMES R. MAHAR, P.E.

Operations Manager