### CENWP-OP-B                                                                                                                          15 October, 2006

MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 41 of 2006, which covers the period from 08 through 14 October 2006.

- OPERATION SUMMARY

  - Daily average river flows ranged from 101.8 to 117.9 kcfs.  Daily average project tailwater elevation ranged from 9.6’ to 10.7’ msl.  Daily average powerhouse forebay elevation ranged from 75.8’ to 76.2’ msl.  Daily average river temperature ranged between 62 and 63F.  Secchi disk measurements averaged 7.0 feet.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are included in the spill pattern.  Daily average spill ranged from 1.2 to 1.3 kcfs.

  - Unit Operation.

    - Unit 0 remains out of service due to governor trouble.

    - Unit 2 remains out of service for turbine rehab.

    - Unit 7 was out of service from 0600 until 1635 on 10 October for semi-annual maintenance.

    - Unit 8 remains out of service for turbine rehab.

    - Unit 9 was out of service from 0410 until 1615 on 11 October for semi-annual maintenance.

    - Unit 10 remains out of service for turbine rehab.

    - Unit 13 remains out of service for a four year overhaul.

    - Unit 16 remains out of service for VBS modifications.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays; VBS drawdowns are measured daily on modified units.

- ACTIVITIES

  - AWS Closures.   No AWS valves were closed.

  - STS/VBS Inspections.  No STSs/VBSs were inspected this reporting week.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11-18 | NA | NA | NA |

Table 1.  STS and VBS inspections.

  - Dewatering and Fish Salvage.

    - Biologists removed one adult unclipped coho, three adult shad and 50 juvenile shad from the UMT on 11 October.

- RESEARCH

  - CRITFC.  Sampling adult fish in the AFF until late October.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - NOAA Fisheries- Gilbreath.  Fish continue to be held.

  - PSMFC smolt monitoring.  Two fall back salmon were observed crossing the separator bars.  45 juvenile shad collected for UI.

  - University of Idaho.  Collecting juvenile shad from the SMF to understand their migration biology.  45 juvenile shad received from PSMFC.

  - USGS lamprey.  Lamprey held in the FERL for tagging evaluation.

  - WDFW Fish Counting.  Visual fish counting continues at WS and BI.  Video counting at CI.

- FISHWAYS

  - Project biologists inspected 08 through 14 October 2006.  NOAA Fisheries inspected on 13 October.

  - Fish Passage Plan Observations.

| DATE | ##### LOCATion | ##### FPP Deviation | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| 8-9 | B-Branch | Weir reading <1.0 | Unknown | No action taken |  |
| 8 | B-Branch | Differential <1.0’ | PLC in manual | Operators adjusted |  |
| 10 | A-Branch | FG3-3 stuck | Unknown | Electricians and mechanics looking at it. | 07-2660 |
| 10-11 | Weir 37 | Reading <1.0’ | AFF trapping | No action taken |  |
| 10 | Weir 38 | Reading <1.0’ | AFF trapping | No action taken |  |
| All week | Weir 67 | Reading >1.0’ | UMT closed | See 4.5.6 |  |

Table 2.  FPP Out of Criteria items.

    - PH1 electronic velocity meter is in an eddy.  Due to this placement, the velocity meter reads less than 1.5 fps.  Readings to meet FPP criteria are taken from the south end mechanical meter.

    - PH2 electronic velocity meter was incorrectly installed.  Will be corrected this winter.

    - Cascades Island FG6-12 is closed due to a bent shaft.  This is OOC when TW exceeds 13’.  Repairs will be made this winter.

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Diffuser failure occurred during water up.  Repairs are scheduled for winter 07/08.

    - DSM2 airburst screens 1, 4, 5, 11 and 12 do not operate consistently.  A portion of the PLC has been reprogrammed.  TR#06-40863.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

| Date | WS average | BI average |
| --- | --- | --- |
| 10/08 | 63.1 | 62.8 |
| 10/09 | 62.7 | 62.5 |
| 10/10 | 62.5 | 62.4 |
| 10/11 | 62.5 | 62.4 |
| 10/12 | 62.5 | 62.4 |
| 10/13 | 62.8 | 62.7 |
| 10/14 | 63.2 | 63.0 |

Table 3.  Bonneville Fishway temperatures.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration is checked on the weekends.  Cascades Island entrance is significantly out of calibration.

    - Cascades Island and B-Branch PLCs are in manual.

    - Stab plates remain installed in the PH2 FOGs.

    - Cascades Island exit remains open.

    - The UMT remains OOS.

    - Due to the UMT closure, Weir 67 is reading greater than 1.0’ +/- .1’.  With FV6-9 completely closed, the only other way to lower the elevation is it to throttle the exit, which Project Fisheries is disinclined to do.  As such, Weir 67 may read up to 1.2’ until 01 December depending on forebay elevations.

  - Juvenile Fishways.

    - DSM1 will continue to remain OOS until removal in winter 07/08.

- CONSTRUCTION

  - B1 ITS Wall Removal.  Plans are to remove the wall, slope the floor, and install three automated gates.

  - PH1 STS/VBS removal and TIES removal.  Six TIES and 32 PH1 STSs/VBSs will be scrapped.

  - PH2 VBS mods.  Unit 16 is OOS.  Units 11 and 15 will follow later this Autumn/Winter.

- HAZMAT, SPILLS AND CLEANUP – nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS.

A teletype sent on 12 October requests Bonneville Pool be operated with a hard constraint of 71.5’ – 72.5’ msl. and a soft constraint of 71.5’ – 72.0’ msl. From 0800 until 1600 on 17 October.  This operation request is to facilitate the salvage activities at Nichols Boatworks.

  - Bonneville’s winter maintenance schedule is available.  Please contact Fisheries for a copy.

JAMES R. MAHAR, P.E.

Operations Manager