### CENWP-OP-B                                                                                                                          12 March 2007

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 10 of 2007, which covers the period from 04 through 10 March 2007.

- OPERATION SUMMARY

  - Daily average river flows ranged from 138.4 to 190.2 kcfs.  Daily average project tailwater elevation ranged from 13.3’ to 16.4’ msl.  Daily average powerhouse forebay elevation ranged from 74.1’ to 75.7’ msl.  Daily average river temperature ranged between 41F and 42F.  Secchi disk measurements averaged 6.0’ to 7.0’.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are included in the spill pattern.  Daily average spill averaged 0.7 to 1.3 kcfs.

    - Spill Bay 1 reopened for daylight hours on 09 March when Cascades Is. ladder returned to service.

    - Spill Bay 18 is open during daylight hours.

  - Unit Operation. Powerhouse 2 priority.

    - Unit 1 was out of service from 0648 until 0756 on 05 March for CT/PT inspection.

    - Unit 2 remains out of service for turbine rehab but has been running periodically for testing.

    - Unit 8 remains out of service for turbine rehab.

    - Unit 10 remains out of service for turbine rehab.

    - Unit 13 remains out of service for a four year overhaul.

    - Unit 15 remains out of service, as of 08 February, to facilitate VBS/gatewell modifications.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays; VBS drawdowns are measured daily on modified units.

- ACTIVITIES

  - AWS Closures.  The following AWS valves are closed.

    - FV1-1 was closed from 1058 until 1132 on 08 March for cleaning.

  - STS/VBS Inspections. No STSs/VBSs were inspected this reporting week.

  - Dewatering and Fish Salvage.  Unit 2 tail logs were pulled and 27 sturgeon were removed and released alive and in good condition downstream of Bonneville Dam on 06 March.

- RESEARCH

  - Battelle.  Monitoring with hydroacoustics near B2CC entrance to study kelt passage.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - Fish Field Unit.  Sea lion observations continue.

  - NOAA Lamprey.  Construction of the lamprey ramp in the WA shore AWS began in mid January.

  - PSMFC Smolt Monitoring.  Index and condition sampling began on 01 March.

  - USGS.  Lamprey held in the FERL for tagging evaluation.

  - USDA Wildlife Services.  Hazing sea lions from the powerhouses began 01 March.

  - WDFW Fish Counting.  Video counting continues at Bradford Island.  Video counting at WA shore began 21 February.

  - WDFW/ODFW Sea lion hazing.  Hazing from boats began 01 March.

- FISHWAYS

  - Project biologists inspected  04 – 10 March 2007.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE |  | ##### RESPONSE | ##### TR# |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND AND BRANCHES |  |  |  |  |  |  |  |
| 3/3-3/6 | PH1 Ice and Trash sluiceway | Chain gates 1C, 3C, 6C not open |  | 1B, 2B, 3B, and 5A are open at 71.5 msl. |  |  | Riggers notified |
| 3/8 | B-Branch | Differential < 1.0 | Unknown | Operators adjusted |  |  |  |
| 3/9 | A-branch | Staff gauge < 1.0 | Unknown | Operators adjusted |  |  |  |
| 3/9 | B-branch | Staff gauge < 1.0 | Unknown | Operators adjusted |  |  |  |
| 3/9 | Brad. Is. weir | Staff gauge < 1.0 | Unknown | Operators adjusted |  |  |  |
| 3/9 | FG3-24 | Incorrect position for tailwater | In manual | Operator adjusted |  |  |  |

    - PH1 electronic velocity meter is in an eddy.  Due to this placement, the velocity meter reads less than 1.5 fps.  Readings to meet FPP criteria are taken from the south end mechanical meter.

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 07/08.

    - FG3-3 stuck partially open.  Repairs are scheduled for winter 07/08.  TR#07-2660

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

Table 3.  Bonneville Fishway temperatures.

| Date | WS average | BI average |
| --- | --- | --- |
| 3/4 | 41.8 | 41.9 |
| 3/5 | 42.0 | 42.0 |
| 3/6 | 42.2 | 42.3 |
| 3/7 | 42.4 | 42.4 |
| 3/8 | 42.5 | 42.5 |
| 3/9 | 42.6 | 42.5 |
| 3/10 | 42.5 | 42.6 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration is checked on the weekends.

    - B-Branch PLC is partially in auto and manual.

    - Cascades Island and the UMT returned to service at 1620 on 09 March.

    - SLEDS installed at Cascades Island entrance on 09 March.

  - Juvenile Fishways.

    - DSM1 continues to remain OOS.

    - B2 Corner Collector was opened for the season at 1328 on 01 March.

    - DSM2 air burst system and brush screen cleaners are in auto.

    - Hydrocannon returned to service 07 March.

    - DSM2 LED orifice lights were replaced at 2100 on 09 March with incandescent bulbs.

  - Lamprey Fishways.  Remain dewatered.

- CONSTRUCTION

  - B1 ITS Wall Removal.  Plans are to remove the wall, slope the floor, and install three automated gates.

  - PH1 STS/VBS removal and TIES removal.  Six TIES and 24 PH1 STSs/VBSs will be scrapped.

  - PH2 VBS mods.  Units 11 and 16 are complete.  Unit 15 is currently OOS.

- HAZMAT, SPILLS AND CLEANUP – nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 22 December details tailwater operations for chum incubation.   Effective as soon as possible, a hard tailwater constraint to operate project tailwater no lower than 13.0 feet during all hours.  One pulse per day with a minimum duration of 20 minutes and minimum elevation of 13.5 feet.

  - A teletype sent on 09 March requests that Powerhouse 1 main unit 2 become the priority unit to be operated first on, last off effective 0600 hours 1 March until 1800 hours 13 March 2007 then return to unit priority as determined in the 2007 Fish Passage Plan (3, 1, 4, 6, 2, 5, 7, 10, 9, 8).  This is a modification from the Fish Passage Plan and is necessary due to testing work on Unit 2 prior to returning to service.

JAMES R. MAHAR, P.E.

Operations Manager