### CENWP-OP-B                                                                                                                          02 April 2007

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 13 of 2007, which covers the period from 25 through 31 March 2007.

- OPERATION SUMMARY

  - Daily average river flows ranged from 248.7 to 293.1 kcfs.  Daily average project tailwater elevation ranged from 20.0’ to 22.2’ msl.  Daily average powerhouse forebay elevation ranged from 72.4’ to 75.6’ msl.  Daily average river temperature ranged between 46F and 47F.  Secchi disk measurements averaged 3.0’ to 4.0’.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are open 0.5’ during daylight hours and are included in the spill pattern.  Daily average spill averaged 31.3 to 58.4 kcfs.  Spillbays 2-17 were on sill from 0700 until 1500 on 30 March to allow researcher to retrieve equipment.

  - Unit Operation. Powerhouse 2 priority.

    - Unit 5 was out of service from 1135 until 1210 on 29 March for cooling water strainer work.

    - Unit 6 was out of service from 1135 until 1210 on 29 March for cooling water strainer work.

    - Unit 8 remains out of service for turbine rehab.

    - Unit 10 remains out of service for turbine rehab.

    - Unit 11 was out of service from 0654 until 0905 on 28 March for STS repair.

    - Unit 12 was out of service from 1037 until 1130 and from 1345 until 1400 on 29 March to facilitate the VBS barge arrival and departure.

    - Unit 13 remains out of service for a four year overhaul.

    - Unit 14 was out of service from: 1128 until 1421 on 26 March for STS replacement; 1037 until 1130 on 29 March for VBS barge arrival; 1233 until 1616 on 29 March for STS failure.

    - Unit 15 remains out of service, as of 08 February, to facilitate VBS/gatewell modifications.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays; VBS drawdowns are measured daily on modified units.

- ACTIVITIES

  - AWS Closures.  The following AWS valves were closed for cleaning.

    - FV1-1 from 0732 until 0900 on 26 March and from 1517 until 1539 on 28 March.

  - STS/VBS Inspections. The following STSs/VBSs were inspected this reporting week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | 3/26 | 161 | A-slot replaced with a spare, rip in C-slot |
| 12 | 3/27 | 803 |  |
| 13 | 3/28 | 0 | Installed 28 March |
| 14 | 3/26 | 779 |  |
| 15 |  |  | OOS |
| 16 | 3/26 | 826 |  |
| 17 | 3/26 | 788 |  |
| 18 | 3/26 | 820 |  |

  - Dewatering and Fish Salvage.  No dewatering or fish salvage activities this reporting week.

- RESEARCH

  - Battelle.  Monitoring with hydroacoustics near B2CC entrance to study kelt passage.

  - ERDC TDG.  Sensors remain installed in the tailrace.

  - Fish Field Unit.  Sea lion observations continue.

  - NOAA Lamprey.  Construction of the lamprey ramp in the WA shore AWS began in mid January.

  - PSMFC Smolt Monitoring.  One adult steelhead crossed the separator bars.

  - USGS.  Lamprey held in the FERL for tagging evaluation.

  - USDA Wildlife Services.  Hazing sea lions from the powerhouses continues.

  - WDFW Fish Counting.  Video counting continues at Bradford Island and WA shore.

  - WDFW/ODFW/CRITFC Sea lion hazing.  Hazing from boats continues.

- FISHWAYS

  - Project biologists inspected on 25 through 31 March 2007.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |  |
| --- | --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND AND BRANCHES |  |  |  |  |  |  |
| 3/26 | PH1 PCC | Sensors OOS | Unknown | Electricians working on repairs. |  |  |
| 3/27-29 | A-Branch weir | Reading <1.0’ | FV3-9 not controlling | FV3-9 TR’d |  |  |
| 3/27-28 | B-Branch weir | Reading <1.0’ | See above | See above |  |  |
| 3/31 | B-Branch weir | Reading >1.0’ | FV3-9 not controlling | FV3-9 TR’d |  |  |
| 3/27-29 | BI weir | Reading <1.0’ | See above | See above |  |  |
| WASHINGTON SHORE |  |  |  |  |  |  |
| 3/25, 3/31 | Weir 37 | Reading >1.0’ | In manual | Operators adjusted. |  |  |
| 3/29, 3/31 | Weir 38 | Reading <1.0’ | Unknown |  |  |  |

    - PH1 electronic velocity meter is in an eddy.  Due to this placement, the velocity meter reads less than 1.5 fps.  Readings to meet FPP criteria are taken from the south end mechanical meter.

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 07/08.

    - FG3-3 stuck partially open.  Repairs are scheduled for winter 07/08.  TR#07-2660

    - FG3-5 has a damaged shaft and the valve won’t operate.  Repairs are scheduled for winter 07/08.

    - FG3-6 stuck partially open.  Repairs are scheduled for winter 07/08.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore. Temperatures are in Fahrenheit.

Table 3.  Bonneville Fishway temperatures.

| Date | WS average | BI average |
| --- | --- | --- |
| 3/25 | 46.9 | 46.9 |
| 3/26 | 46.6 | 46.6 |
| 3/27 | 46.5 | 46.5 |
| 3/28 | 46.7 | 46.6 |
| 3/29 | 46.8 | 46.8 |
| 3/30 | 46.8 | 46.9 |
| 3/31 | 46.8 | 46.8 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration is checked on the weekends.

    - B-Branch and Cascades Island PLCs are in automatic.

    - SLEDS installed at all main fish entrances.

    - Stab plates remain installed in the FOGs due to the sea lion barriers not fitting properly.

    - The AFF was watered up at 1546 on 26 March.

  - Juvenile Fishways.

    - DSM1 continues to remain OOS.

    - DSM2 orifice lights continue to be lighted with the halogen bulbs.

    - The B2CC was closed from 1027 until 1418 on 29 March to facilitate the arrival and departure of the VBS delivery barge.

    - Spillbays 2-17 were on sill from 0700 until 1500 on 30 March to allow researcher to retrieve equipment.

  - Lamprey Fishways.

    - The Bradford Island lamprey ramp remains in service as of 23 March.

- CONSTRUCTION

  - B1 ITS Wall Removal.  Plans are to remove the wall, slope the floor, and install three automated gates.

  - PH1 STS/VBS removal and TIES removal.  Six TIES and 23PH1 STSs/VBSs will be scrapped.

  - PH2 VBS mods.  Units 11 and 16 are complete.  Unit 15 is currently OOS.

- HAZMAT, SPILLS AND CLEANUP – nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 22 December details tailwater operations for chum incubation.   Effective as soon as possible, a hard tailwater constraint to operate project tailwater no lower than 13.0 feet during all hours.  One pulse per day with a minimum duration of 20 minutes and minimum elevation of 13.5 feet.

  - A teletype sent on 27 March requests the B2CC be closed from 0930 until 1300 on 29 March to facilitate the delivery and off loading of the new VBSs for PH2.

  - A teletype sent on 27 March requests spillbays 2-17 closed, and tagged out, from 0700 until 1500 on 30 March.  This is to allow researcher access to the forebay pier noses to remove research equipment.

JAMES R. MAHAR, P.E.

Operations Manager