### CENWP-OP-B                                                                                                                          10 April 2007

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 14 of 2007, which covers the period from 01 through 07 April 2007.

- OPERATION SUMMARY

  - Daily average river flows ranged from 223.7 to 279.7 kcfs.  Daily average project tailwater elevation ranged from 18.6’ to 21.7’ msl.  Daily average powerhouse forebay elevation ranged from 74.1’ to 75.0’ msl.  Daily average river temperature ranged between 46F and 47F.  Secchi disk measurements averaged 3.2’ to 4.5’.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are open 0.5’ during daylight hours and are included in the spill pattern.  Daily average spill averaged 1.6 to 78.5 kcfs.

  - Unit Operation. Powerhouse 2 priority.

    - Unit 8 remains out of service for turbine rehab.

    - Unit 10 remains out of service for turbine rehab.

    - Unit 12 was out of service from 1256 until 1437 on 05 April to repair a governor oil leak.

    - Unit 13 returned to service at 1637 on 02 April.

    - Unit 15 remains out of service, as of 08 February, to facilitate VBS/gatewell modifications.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays; VBS drawdowns are measured daily on modified units.

- ACTIVITIES

  - AWS Closures.  No AWS valves were closed for cleaning.

  - STS/VBS Inspections. No STSs/VBSs were inspected this reporting week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11-18 | NA | NA | NA |

  - Dewatering and Fish Salvage.  No dewatering or fish salvage activities this reporting week.

- RESEARCH

  - Battelle.  Monitoring with hydroacoustics near B2CC entrance to study kelt passage.

  - Battelle.  Installed equipment on spillway piernoses for a spillway survival evaluation.

  - Fish Field Unit.  Sea lion observations continue.

  - NOAA Lamprey.  Construction of the lamprey ramp in the WA shore AWS began in mid January.

  - PSMFC Smolt Monitoring.  One adult steelhead crossed the separator bars.

  - USACE TDG.  New TDG monitors were installed in the spillway tailrace.

  - USGS.  Lamprey held in the FERL for tagging evaluation.

  - USDA Wildlife Services.  Hazing sea lions from the powerhouses continues.

  - WDFW Fish Counting.  Visual counting began on 01 April at Bradford Island and Washington Shore

  - WDFW/ODFW/CRITFC Sea lion hazing.  Hazing from boats continues.

- FISHWAYS

  - Project biologists inspected on 01 through 07 April 2007.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |  |
| --- | --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND AND BRANCHES |  |  |  |  |  |  |
| 4/5-6 | A-Branch weir | Reading <1.0’> | FV3-9 not controlling | FV3-9 TR’d |  |  |
| 4/5 | B-Branch weir | Reading <1.0’> | See above | See above |  |  |
| 4/5 | BI weir | Reading <1.0’> | See above | See above |  |  |
| Once | BI and CI | Calibration not done | Personnel shortage |  |  |  |
| CASCADES ISLAND |  |  |  |  |  |  |
| 4/1-7 | FG6-10 | Open ½ way | Unknown | FY08 winter maint. |  |  |
| 4/2 | UMT weir | Reading <1.0’> | FV5-9 not controlling | Operators notified. |  |  |
| WASHINGTON SHORE |  |  |  |  |  |  |
| 4/2-3 | Weir 37 | Reading <1.0’> | In manual | Operators adjusted FV6-9 |  |  |
| 4/2-3 | Weir 38 | Reading <1.0’> | Unknown | Operators adjusted FV6-9 |  |  |
| 4/2 | Weir 67 | Reading <1.0’> | Unknown | Operators adjusted FV6-9 |  |  |
| 4/3-7 | Velocity meter | Reading <1.5 fps | Unknown | Meter TR’d |  |  |

    - PH1 electronic velocity meter is in an eddy.  Due to this placement, the velocity meter reads less than 1.5 fps.  Readings to meet FPP criteria are taken from the south end mechanical meter.

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 07/08.

    - FG3-3 stuck partially open.  Repairs are scheduled for winter 07/08.  TR#07-2660

    - FG3-5 has a damaged shaft and the valve won’t operate.  Repairs are scheduled for winter 07/08.

    - FG3-6 stuck partially open.  Repairs are scheduled for winter 07/08.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore. Temperatures are in Fahrenheit.

Table 3.  Bonneville Fishway Temperatures.

| Date | WS average | BI average |
| --- | --- | --- |
| 4/1 | 46.4 | 46.4 |
| 4/2 | 46.2 | 46.2 |
| 4/3 | 46.3 | 46.2 |
| 4/4 | 46.7 | 46.7 |
| 4/5 | 47.2 | 47.2 |
| 4/6 | 47.5 | 47.6 |
| 4/7 | 47.8 | 47.8 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration is checked on the weekends.

    - B-Branch and Cascades Island PLCs are in automatic.

    - SLEDS installed at all main fish entrances.

    - Stab plates remain installed in the FOGs due to the sea lion barriers not fitting properly.

  - Juvenile Fishways.

    - DSM1 continues to remain OOS.

    - DSM2 orifice lights continue to be lighted with the halogen bulbs.

    - DSM2 screen cleaner #1 had a broken chain from 2300 on 01 April until 1222 on 03 April.

  - Lamprey Fishways.

    - The Bradford Island lamprey ramp remains in service as of 23 March.

- CONSTRUCTION

  - B1 ITS Wall Removal.  Plans are to remove the wall, slope the floor, and install three automated gates.

  - PH1 STS/VBS removal and TIES removal.  Six TIES and 23PH1 STSs/VBSs will be scrapped.

  - PH2 VBS mods.  Units 11 and 16 are complete.  Unit 15 is currently OOS.

- HAZMAT, SPILLS AND CLEANUP – nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 22 December details tailwater operations for chum incubation.   Effective as soon as possible, a hard tailwater constraint to operate project tailwater no lower than 13.0 feet during all hours.  One pulse per day with a minimum duration of 20 minutes and minimum elevation of 13.5 feet.

  - A teletype sent on 04 April requests spillbays 2 through 17 be closed from 0800 until 1600 on 06 April.  This is to allow Battelle access to the piernoses for research equipment installation.

  - A teletype sent on 06 April requests juvenile spill start at 0001 on 10 April.  Bonneville will spill 100K 24 hours a day.  The spill table may be found in the 1007 FPP page Bon-51, Table Bon-15.  At no time will the project voluntarily spill above the 120% gas cap.

JAMES R. MAHAR, P.E.

Operations Manager